SUPPLEMENTARY FIGURE

CHEMICAL STRUCTURES

- Retinol (C20H30O)

- Retinyl Acetate (C22H32O2)

- γ-Tocotrienol (C28H42O2)

- β-Tocopherol (C28H48O2)

- γ-Tocopherol (C28H48O2)

- α-Tocopherol (C29H50O2)

- δ-Tocopherol (C27H46O2)

- α-Tocotrienol (C29H44O2)

*Source : PubChem (https://pubchem.ncbi.nlm.nih.gov/compound/)

SUPPLEMENTARY TABLE

| Vitamin Class | Standard (vitamer) | Concentration (mM) |
| --- | --- | --- |
| A | Retinol | 0.1 mM |
|  | Retinyl Acetate |  |
| E | α-Tocopherol | 10 mM |
|  | β-Tocopherol |  |
|  | γ-Tocopherol |  |
|  | δ-Tocopherol |  |
|  | α-Tocotrienol |  |
|  | γ-Tocotrienol |  |

Table S1. Concentration of stock standard solutions

| Calibrator | Vitamin A (RET, RET.ACE) (µM) | Vitamin E (BTF, GTF, DTF, ATT) (µM) | Vitamin E (GTT) (µM) | Vitamin E (ATF) (µM) | ISTD (mM) |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.25 | 5.0 | 2.5 | 10.0 | 0.1 |
| 2 | 0.5 | 10.0 | 5.0 | 20.0 | 0.1 |
| 3 | 1.0 | 20.0 | 10.0 | 40.0 | 0.1 |
| 4 | 2.0 | 40.0 | 20.0 | 60.0 | 0.1 |
| 5 | 5.0 | 80.0 | 50.0 | 80.0 | 0.1 |
| 6 | 10.0 | 100.0 | 100.0 | 100.0 | 0.1 |

Table S2. Concentrations of standards in calibrator solutions

| Standard | Standard concentrations in QC |  |  |
| --- | --- | --- | --- |
|  | Low QC (µM) | Medium QC (µM) | High QC (µM) |
| Retinol (RET) | 0.75 | 5.13 | 7.56 |
| Retinyl Acetate (RET. ACE) | 0.75 | 5.13 | 7.56 |
| γ-Tocotrienol (GTT) | 7.50 | 51.25 | 75.63 |
| α-Tocotrienol (ATT) | 15.00 | 52.50 | 76.25 |
| δ-Tocopherol (DTF) | 15.00 | 52.50 | 76.25 |
| β-Tocopherol (BTF) | 15.00 | 52.50 | 76.25 |
| γ-Tocopherol (GTF) | 15.00 | 52.50 | 76.25 |
| α-Tocopherol (ATF) | 30.00 | 55.00 | 77.50 |

Table S3. Concentration of standards in spiked quality control (QC) samples

| Analytes | Elution Time (min) | Detection Wavelength (nm) |
| --- | --- | --- |
| Retinol (RET) | ~4.3 | 325 |
| Retinyl Acetate (RET.ACE) | ~5.6 | 325 |
| γ-Tocotrienol (GTT) | ~11.0 | 298 |
| α-Tocotrienol (ATT) | ~12.0 | 298 |
| δ-Tocopherol (DTF) | ~18.0 | 298 |
| β-Tocopherol (BTF) | ~22.0 | 298 |
| γ-Tocopherol (GTF) | ~23.0 | 298 |
| α-Tocopherol (ATF) | ~27.0 | 298 |
| Internal Standard (ISTD) | ~4.6 | 255 |

Table S4. Retention time and detection wavelengths of analysed vitamers and internal standards using the developed UHPLC method. Retention time variability (~) accounts for minor differences due to experimental conditions.