Table S1. Definition of geometric features from CT annotations.

| Name | Definition |
| --- | --- |
| Mesh Volume | A 3D structure of aorta was generated from annotation on CT images. The structure was consisted of triangle meshes generated by a algorithm of PyRadiomics. The detail of algorithm is referred to PyRadiomics official documents.* The mesh volume is calculated from the triangle mesh of the structure. |
| Voxel Volume | The voxel volume of the structure is approximated by mulplying the number of voxels in the structure by the volume of a single voxel. |
| Surface Area | The surface area of each triangle in the mesh is calculated. And the total surface area of the structure. is then obtained by taking the sum of all calculated areas. |
| Surface Volume Ratio | A lower value of the surface area to mesh volume ratio indicates a more compact (sphere-like) shape. |
| Sphericity | Ity is a measure of the roundness of the shape of the structure relative to a sphere. If a value of the sphericity is 1, it indicates a perfect sphere. |
| Maximum 2D Diameter (Column) | It is defined as the largest pairwise Euclidean distance between surface mesh vertices of the structure in the row-slice (usually the coronal plane. |
| Maximum 2D Diameter (Row) | It is defined as the largest pairwise Euclidean distance between surface mesh vertices of the structure in the column-slice (usually the sagittal) plane. |
| Maximum 2D Diameter (Slice) | It is defined as the largest pairwise Euclidean distance between surface mesh vertices of the structure in the row-column (generally the axial) plane. |
| Maximum 3D Diameter | It is defined as the largest pairwise Euclidean distance between surface mesh vertices of the structure. |
| Major Axis Length | It is defined as the largest axis length of the structure-enclosing ellipsoid and is calculated using the largest principal component λmajor . |
| Minor Axis Length | It is defined as the second-largest axis length of the structure -enclosing ellipsoid and is calculated using the largest principal component λminor . |
| Least Axis Length | It is defined as the smallest axis length of the structure -enclosing ellipsoid and is calculated using the largest principal component λleast . |
| Elongation | It is defined as the relationship between the two largest principal components in the structure shape. If a value of the elongationis 1, the cross section through the first and second largest principal components is circle-like (non-elongated). |
| Flatness | It is defined as the relationship between the largest and smallest principal components in the structure shape. If a value of the flatness 1, the cross section through the largest and smallest principal components is sphere-like. |

* The contents of this table was a summary PyRadiomics official docmument

For more detail, please refer to PyRadiomics official documentation:

https://pyradiomics.readthedocs.io/en/latest/features.html#module-radiomics.shape