# Supplementary Material

# Supplementary material to Teran, Yee-Rendon, Sosa-Arámbula, De La Herrán-Arita & Woods (submitted) “Home Lighting, Blue-Light Filtering, and Their Effects on Melatonin Suppression”

## Calculation of the figures of merit used in the calculation of circadian illuminance (CIL) and the melatonin suppression value (MSV)

The MSV was calculated using the luminous efficacy of radiation (LER) and circadian efficiency of radiation (CER) which determine the relative photopic and circadian efficiency, respectively, of the source plus and filter.  We used two types of parameters to evaluate the effect of radiation on the visual and circadian systems.  The circadian illuminance (CIL) help us to quantify the effects of light on the circadian clock, this parameter weights the effect of the wavelengths on the circadian clock such as the photopic (visual) illuminance (PIL) weights the effect of visible light.  The circadian action factor (CAF) refers to the biological action per unit of visual response.  Different CAF values are advisable depending on the time of day: high CAF during daytime and low CAF during nighttime.

The circadian illuminance (CILi,j) of lamp i and filter j was calculated as the product of the circadian action factor (CAFi,j) [37] and the photopic illuminance (PILi,j) [38] :

CILi,j=CAFi,j×PILi,j, 	[blm/m2]			(S1),

where blm denotes a bio-lumen.  CAF and PIL are calculated using the measured spectral irradiances of the lamps as described above.  PIL can be obtained through the relationship:

PILi,j=683380780Ic,iλ Tjλ Vλ dλ.	[lm/m2]				(S2).

Here, Ic,iλ=Φc,iλAp, Φcλ is the flux or power at the cornea of lamp i, Ap the pupil area, and Tj() the transmittance of filter j, and V() is the human photopic luminous efficiency function.  For calculations of lamps alone (no filter),Tj()=1. The constant 683 came from the definition of a lumen. A lumen is equal to 1/683 Watts at 540 THz (approximately a wavelength of 555 nm, the peak of the luminous efficiency function).

CAF is defined by the ratio:

CAFi,j=CERi,j/LERi,j,	[blm/lm]				(S3),

where the luminous efficacy of radiation (LER) and the circadian efficiency of radiation (CER) can be calculated by means of the following expressions:

LERi,j=683380780Ic,iλ Tjλ Vλ dλ/380780Ic,iλ Tjλ dλ, 	[lm/W]			(S4),

and

CERi,j=683380780Ic,iλ Tjλ Cλ dλ/380780Ic,iλ Tjλ dλ,	[blm/W]		(S5),

where the circadian luminous efficiency function is denoted by  Cλ [37]. These parameters allow the  evaluation of the effect of the incident light on the circadian response of the eye. However, these parameters alone are not sufficient to evaluate the response of the body to the incoming light.  Equation (2) in the manuscript describes the relationship CIL and MSV (melanopic suppression value) obtained by fitting previously reported data from human studies of melatonin concentrations in the blood.

## Calculation of the error in the Melatonin Suppression Value

Since the CIL is defined by the product of CAF and PIL, it can be shown that the propagated error to MSV can be calculated as

δMSVMSV=δCAFCAF2+δPILPIL2.			(S6).

## Normalisation of the Melatonin Suppression Value

The normalized photopic illuminance was defined as

PILi, j(N)=683380780Ic,i(N)λ Tjλ Vλ dλ,        [lm/m2]                                (S7),

and, the normalized circadian illuminance was

CILi, j(N)=CAFi,j×PILi, j(N).      [blm/m2]                                     (S8).

We should note that the circadian action factor is not affected by the normalization, CAF(N)=CAF.

The factor γ can be calculated in terms of an arbitrary source.  We chose a source of a 75 W incandescent lamp at 2 m, with an illuminance of PIL75W=40 lx [28]. Thus, from equation (A7), the photopic illuminance can be written as

PIL75W=γIinc(vis)683380780Ic,iλ Tjλ Vλ dλ,        [lm/m2]                       (S9),

where Ic,iλ represented the spectral irradiance of the light source, here an incandescent lamp need to define Iinc(vis).  Then, solving for the factor γ, we obtain

γ=PIL75WPILIncIInc(vis).       Wm2                                         (S10),

where PILInc represents the photopic illuminance of one of the incandescent lamps.

The normalized photopic illuminance of the lamps, see equation (S7), can written in terms of the reference photopic illuminance using equations (S2) and (S10), as:

PILx(n)=PIL(75w)PIL(x)PIL(Inc)IInc(vis)Ix(vis)        [lx]            (S11),

where  x= cool-LED, cool-CFL, warm-LED, etc. For instance, the normalized photopic illuminance of the cool-LED lamps can be calculated from equation (S11) and the median value of the PIL for cool-LED lamps from Table S6 to obtain PILcool-LED(n)=4054.7/23.9×0.1/0.2=56.8 lx.

| Table S1. All the (fixed correlated colour temperature: CCT) lamps measured in this study grouped by technology and CCT category (warm or cool white light) for the compact fluorescent (CFL) and Light-emitting diode (LED) lamps. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturer | LED |  | CFL |  | Incandescent |
|  | Warm | Cool | Warm | Cool |  |
| Argos | - | 9403014 (12W) | - | 8900010 (11W); 8900030 (15W);8900050 (25W), | 8900810 (40W); 8900810 (40W); 8900810 (40W); 8900810 (25W) |
| Aski | - | - | - | Aski | - |
| Etko | - | - | - | Etko | - |
| Filsh | - | - | - | - | filsh100w |
| Fulgore | - | - | Fulgore-25W | Fulgore-15W | - |
| General Electric | - | - | - | - | general.electric-40w |
| Great value | GVT-LED-12W-30 | GVT-LED-12W-65 | GVASP-C-23W | GVASP-23W | - |
| Line starlux | - | - | - | - | Lw 100 |
| Osram | A60 9W/830 | A60 9w/865 | Dulux value El Twist 15W/827 | Dulux value El Twist 15W/865 | P45 40 125 E26 Clear decorative incandescent lamp; osram-60w |
| Philips | Ess ledbulb 8-60W E27 3000K 1PF; Ess ledbulb 12-100W E27 3000K 1PF; 13A19/LED/830/FR/P/ND 1BC MX; ledbulb 4W E27 3000K 100-240VG16.5 MX | Ess ledbulb 8-60W E27 6500K 1PF/6 MX; 13A19/LED/865/FR/P/ND 1BC/6 MX;ledbulb 4W E27 6500K 100-240VG16.5 MX; philiphs-4.5W | - | Mini Twister 13W | P45 Claro (40W) |
| Red dot | - | - | - | ETWFL 54; ETWFL 59; ETWFL 60; | - |
| Saglite | A19 9W 30K | A21 E26 65K; A19 E26 65K | - | - | - |
| Tecnolite | NUNKI A19-LED/007/30;HIDRUS I A19-LED/010/30;LUMIA I A19-LED/012/30; TITANIUM V A19-LED/005/30 | NUKI A19-LED/007/65; HIDRUS I A19-LED/010/65; LUMIA I A19-LED/012/65;TITANIUM V A19-LED/005/65 | - | Hel-15W/27-T2 | - |

Figure S1. Correlated color temperature (CCT) of the lamps claimed versus measured. Dotted line represents equality.

| ##### Table S2. Wattage, equivalent wattage (as reported by the manufacturer) and luminous flux claimed for the lamps. Values are the median with the range in parentheses. |  |  |  |
| --- | --- | --- | --- |
| Lamp category | Wattage(W) | Equivalent wattage(W) | Lumen(lx) |
| Cool-LED | 11(4/15) | 75(40/120) | 820(350/1,310) |
| Warm-LED | 10(4/15) | 60(10/100) | 900(350/1,310) |
| Cool-CFL | 15(11/25) | 59.5(54/120) | 845.5(365/1,571) |
| Warm-CFL | 15(5/25) | 60(40/100) | 800(365/1,675) |
| Incandescent | 40(40/100) | 40(40/100) | 365(365/2,200) |

Figure S2. Wattage(A), equivalent wattage (B) and lumens(C) claimed by the lamps: cool-LED (olive-green), warm-LED (pink), cool-CFL (orange), warm-CFL (blue), and Incandescent (green). Please see Table S2 for related values.

| Table S3. Normalized figures of merit and melatonin suppression values of the median lamp of each category. Range shown in parentheses. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lamp | LER (lm/W) | CER (lm/w) | CAF (-) | PIL(n) (lx) | CIL(n) (blx) | MSV(n) (%) |
| cool-LED | 299.3 ( 292.3/ 303.2) | 204.9 ( 201.3/ 208.5) | 0.7 ( 0.7/ 0.7) | 57.8 ( 27.2/ 146.2) | 39.6 ( 18.2/ 104.2) | 15.3 ( 4.3/ 50.3) |
| warm-LED | 297.0 ( 290.6/ 299.2) | 95.9 ( 80.9/ 103.3) | 0.3 ( 0.3/ 0.4) | 58.1 ( 27.4/ 151.9) | 18.6 ( 9.4/ 48.6) | 4.5 ( 1.4/ 20.6) |
| cool-CFL | 297.0 ( 253.6/ 316.2) | 204.2 ( 146.2/ 215.8) | 0.7 ( 0.5/ 0.8) | 57.5 ( 27.8/ 149.2) | 39.4 ( 18.0/ 108.8) | 15.2 ( 4.3/ 52.2) |
| warm-CFL | 328.1 ( 307.7/ 336.3) | 78.0 ( 72.4/ 82.4) | 0.2 ( 0.2/ 0.3) | 67.6 ( 31.5/ 175.7) | 15.8 ( 7.4/ 40.7) | 3.4 ( 0.9/ 15.9) |
| Incandescent | 110.9 ( 102.6/ 123.4) | 31.7 ( 23.9/ 41.4) | 0.3 ( 0.2/ 0.3) | 40.2 ( 17.9/ 107.4) | 10.9 ( 4.6/ 36.0) | 1.8 ( 0.4/ 13.2) |
| LER= Luminous efficiency of radiation; CER=circadian luminous efficiency of radiation; CAF=circadian action factor; PIL(n)=normalized phototopic illuminance; CIL(n)=normalized circadian illuminance; MSV(n)=normalized melatonin suppression values. |  |  |  |  |  |  |

| Table S4. Absolute (without normalization) figures of merit and melatonin suppression values of the median lamp of each category. Range shown in parentheses. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Lamp | ##### LER (lm/W) | ##### CER (lm/w) | ##### CAF (-) | ##### PIL (lx) | ##### CIL (blx) | ##### MSV (%) | ##### I(vis) [W/m2] |
| cool-LED | ## 299.3 ( 292.3/ 303.2) | ## 204.9 ( 201.3/ 208.5) | ## 0.7 ( 0.7/ 0.7) | ## 57.5 ( 27.2/ 102.2) | ## 39.6 ( 19.0/ 70.8) | ## 15.3 ( 4.7/ 33.7) | ## 0.2 ( 0.1/ 0.3) |
| warm-LED | ## 297.0 ( 290.6/ 299.2) | ## 95.9 ( 80.9/ 103.3) | ## 0.3 ( 0.3/ 0.4) | ## 59.2 ( 25.6/ 111.8) | ## 18.6 ( 6.9/ 37.0) | ## 4.5 ( 0.8/ 13.8) | ## 0.2 ( 0.1/ 0.4) |
| cool-CFL | 297.0 ( 253.6/ 316.2) | ## 204.2 ( 146.2/ 215.8) | ## 0.7 ( 0.5/ 0.8) | ## 33.6 ( 15.5/ 46.6) | ## 22.9 ( 10.0/ 32.0) | ## 6.4 ( 1.5/ 11.0) | ## 0.1 ( 0.0/ 0.1) |
| warm-CFL | ## 328.1 ( 253.6/ 316.2) | ## 78.0 ( 80.9/ 215.8) | 0.2 ( 0.3/ 0.8) | ## 34.5 ( 15.5/ 111.8) | ## 8.4 ( 6.9/ 63.9) | ## 1.1 ( 0.8/ 50.0) | ## 0.1 ( 0.0/ 0.4) |
| Incandescent | ## 110.9 ( 102.6/ 123.4) | ## 31.7 ( 23.9/ 41.4) | ## 0.3 ( 0.2/ 0.3) | ## 23.9 ( 10.6/ 63.8) | ## 6.5 ( 2.8/ 21.4) | ## 0.7 ( 0.2/ 5.7) | ## 0.1 ( 0.1/ 0.3) |
| LER= Luminous efficiency of radiation; CER=circadian luminous efficiency of radiation; CAF=circadian action factor; PIL=phototopic illuminance; CIL=circadian illuminance; MSV=melatonin suppression value; I(vis)= area under the curve in the visible range of the lamp’s irradiance. |  |  |  |  |  |  |  |

| ##### Table S5. Figures of Merit and melatonin suppression values (MSV) of the circadian (tunable) lamps as a function of the correlated color temperature (CCT); See Figure 1F. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Circadian lamp | ##### LER (lm/W) | ##### CER (lm/w) | ##### CAF (-) | ##### PIL (lx) | ##### CIL (blx) | ##### MSV (%) | ##### CCT (K) |
| Estevez | 298.53 | 80.84 | 0.27 | 46.97 | 12.72 | 2.33 | 2972 |
|  | 297.77 | 97.61 | 0.33 | 46.68 | 15.30 | 3.21 | 3201 |
|  | 297.39 | 112.00 | 0.38 | 47.26 | 17.80 | 4.16 | 3423 |
|  | 297.08 | 122.42 | 0.41 | 47.43 | 19.54 | 4.88 | 3597 |
|  | 296.76 | 138.76 | 0.47 | 47.46 | 22.19 | 6.04 | 3895 |
|  | 296.51 | 147.12 | 0.50 | 47.13 | 23.38 | 6.59 | 4058 |
|  | 296.30 | 163.02 | 0.55 | 46.72 | 25.71 | 7.71 | 4393 |
|  | 296.13 | 174.59 | 0.59 | 45.91 | 27.07 | 8.39 | 4656 |
|  | 296.11 | 190.97 | 0.64 | 44.52 | 28.71 | 9.24 | 5054 |
|  | 296.20 | 206.33 | 0.70 | 42.86 | 29.85 | 9.84 | 5456 |
|  | 296.38 | 216.77 | 0.73 | 41.87 | 30.62 | 10.25 | 5745 |
| Philips hue-color | 283.26 | 50.03 | 0.18 | 16.48 | 2.91 | 0.17 | 2161 |
|  | 282.02 | 71.29 | 0.25 | 27.74 | 7.01 | 0.82 | 2559 |
|  | 282.02 | 71.29 | 0.25 | 27.74 | 7.01 | 0.82 | 2828 |
|  | 284.58 | 80.18 | 0.28 | 30.69 | 8.65 | 1.18 | 3035 |
|  | 294.38 | 99.14 | 0.34 | 34.27 | 11.54 | 1.97 | 3366 |
|  | 303.36 | 117.77 | 0.39 | 37.44 | 14.53 | 2.94 | 3674 |
|  | 307.10 | 139.04 | 0.45 | 37.48 | 16.97 | 3.83 | 4043 |
|  | 299.73 | 167.48 | 0.56 | 31.36 | 17.53 | 4.05 | 4647 |
|  | 292.73 | 190.37 | 0.65 | 26.95 | 17.53 | 4.05 | 5214 |
|  | 288.50 | 202.63 | 0.70 | 24.79 | 17.41 | 4.01 | 5561 |
| Philips: Scene-switching | 251.89 | 47.89 | 0.19 | 2.65 | 0.50 | 0.01 | 2200 |
|  | 290.08 | 78.39 | 0.27 | 15.66 | 4.23 | 0.33 | 2700 |
|  | 305.42 | 172.64 | 0.57 | 14.90 | 8.42 | 1.13 | 5000 |
| Wiz | 312.30 | 100.80 | 0.32 | 44.40 | 14.49 | 2.68 | 2800 |
|  | 311.70 | 107.80 | 0.34 | 44.14 | 15.37 | 3.07 | 3000 |
|  | 312.97 | 83.23 | 0.27 | 43.83 | 11.65 | 3.46 | 3200 |
|  | 309.76 | 123.21 | 0.40 | 43.66 | 17.37 | 3.99 | 3400 |
|  | 309.31 | 127.94 | 0.41 | 43.53 | 18.01 | 4.24 | 3600 |
|  | 308.86 | 133.75 | 0.43 | 42.99 | 18.62 | 4.49 | 3800 |
|  | 308.04 | 144.39 | 0.47 | 42.51 | 19.93 | 5.04 | 4000 |
|  | 307.64 | 149.27 | 0.49 | 42.41 | 20.58 | 5.32 | 4200 |
|  | 306.99 | 157.11 | 0.51 | 42.40 | 21.70 | 5.82 | 4400 |
|  | 306.54 | 163.17 | 0.53 | 41.96 | 22.34 | 6.11 | 4600 |
|  | 305.93 | 171.34 | 0.56 | 41.84 | 23.43 | 6.62 | 4800 |
|  | 305.45 | 177.61 | 0.58 | 41.79 | 24.30 | 7.03 | 5000 |
| LER= Luminous efficiency of radiation; CER=circadian luminous efficiency of radiation; CAF=circadian action factor; PIL=phototopic illuminance; CIL=circadian illuminance; MSV=melatonin suppression value; CCT=correlated color temperature. |  |  |  |  |  |  |  |

| Table S6. Figures of Merit, and normalized melatonin suppression values (MSV) of the lamp categories weighted by the transmittance of the blue-light-filter (BLF) lenses, with no lens and clear lens provided as control conditions. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Lamp | Lens | LER (lm/W) | ##### CER (lm/w) | ##### CAF (-) | ##### PIL(n) (lx) | ##### CIL(n) (blx) | MSV(n) (%) |
| ##### cool-LED | ##### No lens | ##### 299.85 | ##### 204.57 | ##### 0.68 | ##### 58.05 | ##### 39.61 | ##### 15.29 |
|  | ##### clear | ##### 300.85 | ##### 201.51 | ##### 0.67 | 53.19 | ##### 35.63 | ##### 13.01 |
|  | ##### Seto | ##### 300.11 | ##### 203.46 | ##### 0.68 | ##### 55.63 | ##### 37.71 | ##### 14.20 |
|  | ##### Vaccutek | ##### 306.15 | ##### 196.32 | ##### 0.64 | 56.42 | ##### 36.18 | ##### 13.32 |
|  | ##### Essilor | 306.83 | ##### 193.82 | ##### 0.63 | ##### 57.82 | ##### 36.53 | ##### 13.52 |
|  | ##### Zeiss | ##### 306.98 | ##### 195.02 | ##### 0.64 | ##### 57.58 | ##### 36.58 | ##### 13.55 |
|  | ##### CGID | 310.69 | ##### 184.84 | ##### 0.59 | ##### 53.78 | ##### 31.99 | ##### 10.99 |
|  | ##### Spektrum | ##### 318.80 | ##### 171.95 | ##### 0.54 | ##### 53.57 | ##### 28.90 | ##### 9.33 |
|  | ##### Swanwick | ##### 315.77 | ##### 10.75 | ##### 0.03 | ##### 49.52 | ##### 1.69 | ##### 0.07 |
|  | ##### THL Sleep | ##### 320.84 | ##### 19.27 | ##### 0.06 | ##### 45.89 | 2.76 | ##### 0.16 |
| ##### warm-LED | ##### No lens | ##### 296.81 | ##### 94.07 | ##### 0.32 | ##### 58.80 | 18.64 | ##### 4.47 |
|  | ##### clear | ##### 295.45 | 95.51 | ##### 0.32 | ##### 53.60 | ##### 17.33 | ##### 3.97 |
|  | ##### Seto | ##### 292.98 | ##### 96.20 | ##### 0.33 | ##### 55.81 | ##### 18.33 | ##### 4.37 |
|  | ##### Vaccutek | ##### 295.68 | ##### 93.13 | ##### 0.31 | ##### 56.01 | ##### 17.64 | ##### 4.10 |
|  | ##### Essilor | ##### 297.78 | ##### 91.93 | ##### 0.31 | ##### 57.56 | ##### 17.77 | 4.15 |
|  | ##### Zeiss | ##### 298.45 | ##### 92.61 | ##### 0.31 | ##### 57.44 | ##### 17.82 | ##### 4.17 |
|  | ##### CGID | ##### 297.30 | ##### 86.50 | ##### 0.29 | ##### 52.82 | ##### 15.37 | 3.23 |
|  | ##### Spektrum | ##### 300.53 | ##### 80.57 | 0.27 | ##### 51.78 | ##### 13.88 | ##### 2.71 |
|  | ##### Swanwick | ##### 267.04 | ##### 5.49 | ##### 0.02 | ##### 42.16 | ##### 0.87 | ##### 0.02 |
|  | ##### THL Sleep | ##### 269.36 | ##### 9.30 | ##### 0.03 | ##### 38.88 | ##### 1.34 | ##### 0.04 |
| ##### cool-CFL | ##### No lens | ##### 297.61 | ##### 205.26 | ##### 0.69 | ##### 58.66 | ##### 40.46 | ##### 15.79 |
|  | clear | ##### 301.24 | ##### 202.58 | ##### 0.67 | ##### 54.00 | ##### 36.31 | ##### 13.39 |
|  | ##### Seto | ##### 311.69 | ##### 205.27 | ##### 0.66 | ##### 58.46 | ##### 38.50 | ##### 14.65 |
|  | Vaccutek | ##### 319.85 | ##### 196.74 | ##### 0.62 | ##### 59.70 | ##### 36.72 | ##### 13.63 |
|  | ##### Essilor | ##### 310.44 | ##### 195.46 | ##### 0.63 | 59.27 | ##### 37.32 | ##### 13.97 |
|  | ##### Zeiss | ##### 310.65 | ##### 196.80 | ##### 0.63 | ##### 58.96 | ##### 37.35 | ##### 13.99 |
|  | ##### CGID | ##### 322.58 | ##### 187.26 | ##### 0.58 | 56.52 | ##### 32.81 | ##### 11.43 |
|  | ##### Spektrum | 330.60 | ##### 174.80 | ##### 0.53 | ##### 56.28 | ##### 29.76 | ##### 9.78 |
|  | ##### Swanwick | ##### 319.59 | ##### 14.37 | ##### 0.04 | ##### 51.72 | ##### 2.32 | ##### 0.12 |
|  | ##### THL Sleep | ##### 330.58 | ##### 23.62 | ##### 0.07 | ##### 48.70 | ##### 3.48 | ##### 0.24 |
| ##### warm-CFL | ##### No lens | ##### 326.85 | ##### 76.06 | ##### 0.23 | ##### 67.08 | ##### 15.61 | ##### 3.32 |
|  | clear | ##### 328.60 | ##### 74.71 | ##### 0.23 | ##### 61.43 | ##### 13.97 | ##### 2.74 |
|  | ##### Seto | ##### 335.24 | ##### 73.05 | ##### 0.22 | ##### 65.66 | ##### 14.31 | ##### 2.86 |
|  | ##### Vaccutek | ##### 338.78 | ##### 69.48 | ##### 0.21 | ##### 66.01 | ##### 13.54 | 2.60 |
|  | ##### Essilor | ##### 333.31 | ##### 71.72 | ##### 0.22 | ##### 66.25 | ##### 14.26 | ##### 2.84 |
|  | ##### Zeiss | ##### 334.05 | ##### 72.18 | ##### 0.22 | ##### 66.12 | ##### 14.29 | ##### 2.85 |
|  | ##### CGID | ##### 338.21 | ##### 66.90 | 0.20 | ##### 61.78 | ##### 12.22 | ##### 2.17 |
|  | ##### Spektrum | ##### 340.64 | ##### 62.71 | ##### 0.18 | ##### 60.40 | ##### 11.12 | ##### 1.84 |
|  | ##### Swanwick | ##### 293.69 | ##### 7.50 | ##### 0.03 | ##### 48.67 | ##### 1.24 | ##### 0.04 |
|  | THL Sleep | ##### 299.70 | ##### 11.54 | ##### 0.04 | ##### 45.37 | ##### 1.75 | ##### 0.07 |
| ##### Incandescent | ##### No lens | ##### 109.67 | ##### 30.21 | ##### 0.28 | ##### 40.00 | ##### 11.02 | ##### 1.81 |
|  | ##### clear | ##### 109.18 | ##### 29.54 | ##### 0.27 | ##### 36.37 | ##### 9.84 | ##### 1.49 |
|  | ##### Seto | ##### 108.70 | ##### 29.24 | ##### 0.27 | ##### 37.78 | ##### 10.16 | ##### 1.57 |
|  | ##### Vaccutek | ##### 109.12 | ##### 28.32 | ##### 0.26 | ##### 37.86 | ##### 9.83 | ##### 1.48 |
|  | ##### Essilor | ##### 110.99 | ##### 28.87 | ##### 0.26 | ##### 38.96 | ##### 10.13 | 1.57 |
|  | ##### Zeiss | ##### 110.81 | ##### 28.96 | 0.26 | ##### 39.11 | ##### 10.22 | ##### 1.59 |
|  | ##### CGID | ##### 108.75 | ##### 26.34 | ##### 0.24 | ##### 35.56 | 8.61 | ##### 1.18 |
|  | ##### Prospek | ##### 109.56 | ##### 24.90 | ##### 0.23 | ##### 34.69 | ##### 7.89 | ##### 1.01 |
|  | ##### Swanwick | ##### 80.27 | ##### 1.53 | 0.02 | ##### 25.31 | ##### 0.48 | ##### 0.01 |
|  | ##### THL Sleep | ##### 80.18 | ##### 2.60 | ##### 0.03 | ##### 23.36 | ##### 0.76 | ##### 0.02 |
| LER= Luminous efficiency of radiation; CER=circadian luminous efficiency of radiation; CAF=circadian action factor; PIL(n)=normalized phototopic illuminance; CIL(n)=normalized circadian illuminance; MSV(n)=normalized melatonin suppression value |  |  |  |  |  |  |  |

| #### Table S7. Equivalent -optic illuminance of the lamp categories weighted by the transmittance of the blue-light-filter (BLF) lenses, with no lens and clear lens provided as control conditions. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #### erythropic | #### chloropic | #### rhodopic | #### melanopic | cyanopic | #### photopic | #### m_lux |
| LED cool | #### No lens | #### 11.20 | #### 10.80 | #### 9.56 | #### 8.65 | #### 7.66 | 11.70 | #### 46.73 |
|  | #### clear | #### 10.38 | #### 9.99 | #### 8.81 | #### 7.95 | #### 6.94 | #### 10.84 | #### 42.93 |
|  | #### Seto | #### 10.07 | #### 9.72 | #### 8.64 | #### 7.85 | #### 6.77 | #### 10.51 | #### 42.39 |
|  | #### Vaccutek | #### 10.61 | #### 10.19 | 8.90 | #### 7.96 | #### 6.44 | #### 11.11 | #### 42.98 |
|  | #### Essilor | #### 10.69 | #### 10.24 | #### 8.89 | #### 7.90 | #### 6.44 | 11.20 | #### 42.66 |
|  | #### Zeiss | #### 10.61 | #### 10.21 | #### 8.91 | #### 7.96 | #### 6.60 | #### 11.12 | #### 42.99 |
|  | CGID | #### 9.78 | #### 9.25 | #### 7.87 | #### 6.89 | #### 5.39 | #### 10.24 | #### 37.22 |
|  | #### Spektrum | #### 9.46 | #### 8.87 | #### 7.34 | 6.28 | #### 4.45 | #### 9.95 | #### 33.91 |
|  | #### Swanwick | #### 5.55 | #### 3.53 | #### 1.18 | #### 0.42 | #### 0.02 | #### 5.41 | 2.27 |
|  | #### THL Sleep | #### 5.23 | #### 3.53 | #### 1.43 | #### 0.64 | 0.02 | #### 5.19 | #### 3.44 |
| #### LED warm | #### No lens | #### 12.00 | #### 9.48 | #### 6.38 | #### 5.02 | #### 3.00 | 12.08 | #### 27.12 |
|  | #### clear | #### 11.14 | #### 8.78 | #### 5.89 | #### 4.62 | #### 2.73 | #### 11.21 | #### 24.97 |
|  | Seto | #### 10.82 | #### 8.52 | #### 5.76 | #### 4.56 | #### 2.67 | #### 10.87 | #### 24.61 |
|  | #### Vaccutek | #### 11.43 | #### 8.98 | #### 5.99 | 4.67 | #### 2.57 | #### 11.50 | #### 25.20 |
|  | #### Essilor | #### 11.51 | #### 9.04 | #### 6.00 | #### 4.64 | #### 2.55 | #### 11.59 | 25.07 |
|  | #### Zeiss | #### 11.41 | #### 8.99 | #### 6.00 | #### 4.66 | #### 2.61 | #### 11.49 | #### 25.18 |
|  | #### CGID | 10.64 | #### 8.25 | #### 5.35 | #### 4.08 | #### 2.15 | #### 10.69 | #### 22.02 |
|  | #### Spektrum | #### 10.36 | #### 7.98 | #### 5.06 | 3.77 | #### 1.79 | #### 10.42 | #### 20.35 |
|  | #### Swanwick | #### 7.58 | #### 4.25 | #### 1.16 | #### 0.36 | #### 0.01 | #### 7.08 | #### 1.97 |
|  | #### THL Sleep | #### 6.96 | #### 4.05 | #### 1.30 | #### 0.50 | 0.01 | #### 6.57 | #### 2.71 |
| #### CFL-cool | #### No lens | #### 6.15 | #### 5.94 | #### 5.30 | #### 4.62 | #### 4.49 | #### 6.50 | 24.95 |
|  | #### clear | #### 5.70 | #### 5.49 | #### 4.89 | #### 4.24 | #### 4.06 | #### 6.03 | #### 22.91 |
|  | #### Seto | #### 5.53 | #### 5.32 | #### 4.75 | #### 4.14 | #### 3.71 | #### 5.84 | #### 22.38 |
|  | #### Vaccutek | #### 5.83 | #### 5.59 | #### 4.90 | 4.19 | #### 3.43 | #### 6.18 | #### 22.62 |
|  | #### Essilor | #### 5.88 | #### 5.63 | #### 4.93 | #### 4.21 | #### 3.70 | #### 6.23 | 22.72 |
|  | #### Zeiss | #### 5.84 | #### 5.62 | #### 4.95 | #### 4.24 | #### 3.81 | #### 6.19 | #### 22.92 |
|  | #### CGID | 5.38 | #### 5.09 | #### 4.36 | #### 3.66 | #### 2.94 | #### 5.70 | #### 19.74 |
|  | #### Spektrum | #### 5.19 | #### 4.86 | #### 4.07 | 3.33 | #### 2.43 | #### 5.52 | #### 17.99 |
|  | #### Swanwick | #### 2.91 | #### 1.75 | #### 0.63 | #### 0.25 | #### 0.01 | #### 2.78 | #### 1.38 |
|  | #### THL Sleep | #### 2.79 | #### 1.85 | #### 0.80 | #### 0.38 | 0.01 | #### 2.75 | #### 2.03 |
| #### CFL-warm | #### No lens | #### 5.97 | #### 4.55 | #### 2.84 | #### 1.86 | #### 1.16 | #### 6.09 | #### 10.04 |
|  | #### clear | #### 5.54 | #### 4.22 | #### 2.63 | #### 1.71 | #### 1.07 | #### 5.65 | #### 9.25 |
|  | #### Seto | #### 5.37 | #### 4.08 | #### 2.54 | #### 1.66 | #### 0.96 | #### 5.47 | #### 8.95 |
|  | #### Vaccutek | #### 5.69 | #### 4.31 | #### 2.66 | 1.71 | #### 0.88 | #### 5.81 | #### 9.22 |
|  | #### Essilor | #### 5.73 | #### 4.35 | #### 2.69 | #### 1.73 | #### 0.97 | #### 5.85 | 9.34 |
|  | #### Zeiss | #### 5.68 | #### 4.33 | #### 2.69 | #### 1.74 | #### 1.00 | #### 5.80 | #### 9.37 |
|  | #### CGID | 5.31 | #### 3.99 | #### 2.42 | #### 1.52 | #### 0.76 | #### 5.41 | #### 8.22 |
|  | #### Spektrum | #### 5.16 | #### 3.86 | #### 2.30 | 1.42 | #### 0.62 | #### 5.26 | #### 7.69 |
|  | #### Swanwick | #### 3.75 | #### 1.97 | #### 0.55 | #### 0.20 | #### 0.00 | #### 3.43 | #### 1.08 |
|  | #### THL Sleep | #### 3.49 | #### 1.95 | #### 0.65 | #### 0.27 | 0.00 | #### 3.25 | #### 1.45 |
| #### Incandescent | No lens | #### 5.89 | #### 4.45 | #### 3.03 | #### 2.35 | #### 1.09 | #### 5.85 | #### 12.71 |
|  | #### clear | #### 5.47 | #### 4.12 | #### 2.80 | #### 2.17 | #### 0.99 | #### 5.43 | 11.71 |
|  | #### Seto | #### 5.32 | #### 3.99 | #### 2.73 | #### 2.13 | #### 0.90 | #### 5.27 | #### 11.49 |
|  | #### Vaccutek | 5.62 | #### 4.21 | #### 2.85 | #### 2.19 | #### 0.86 | #### 5.57 | #### 11.81 |
|  | #### Essilor | #### 5.65 | #### 4.25 | #### 2.86 | 2.19 | #### 0.92 | #### 5.61 | #### 11.81 |
|  | #### Zeiss | #### 5.60 | #### 4.21 | #### 2.86 | #### 2.20 | #### 0.94 | #### 5.56 | #### 11.86 |
|  | #### CGID | #### 5.23 | #### 3.87 | #### 2.54 | #### 1.92 | #### 0.73 | #### 5.18 | #### 10.36 |
|  | #### Spektrum | #### 5.09 | 3.74 | #### 2.42 | #### 1.79 | #### 0.62 | #### 5.05 | #### 9.66 |
|  | #### Swanwick | #### 3.77 | #### 1.93 | #### 0.53 | #### 0.17 | 0.00 | #### 3.44 | #### 0.93 |
|  | #### THL Sleep | #### 3.46 | #### 1.86 | 0.60 | #### 0.24 | #### 0.01 | #### 3.20 | #### 1.31 |

Figure S3. Equivalent -opic illuminance: melanopic (A) and photopic (B) illuminance by lamp group.

Figure S4. Normalized melatonin suppression value (MSV(n)) as function of the power (A) and the circadian action factor (B) of the standard and tunable lamps.

Figure S5. Melanopic to photopic illuminance (M:P) ratio as a function of the melanopic illuminance.

| Table S8. Blue-light hazard efficiency of radiation ηB and the blue light weighted radiance illuminance LB of the conventional lamps. |  |  |
| --- | --- | --- |
| Lamp | ηB | LB (W m-2 sr-1 nm-1) |
| CFL-cool | 0.19869219 | 734.209299 |
| CFL-warm | 0.07039614 | 233.91332 |
| LED cool | 0.17129749 | 1283.49176 |
| LED warm | 0.06595435 | 517.33289 |
| Incandescent | 0.03796294 | 189.156387 |

| Table S9. Blue-light hazard efficiency of radiation ηB and the blue light weighted radiance illuminance LB of the (tunable) lamps. |  |  |  |
| --- | --- | --- | --- |
| Lamp | CCT | ηB | LB (W m-2 sr-1 nm-1) |
| Estevez | 2700 | 0.04504263 | 256390.292 |
|  | 3600 | 0.0739821 | 426971.808 |
|  | 4600 | 0.09979858 | 581831.548 |
|  | 5500 | 0.12360984 | 712956.981 |
|  | 6500 | 0.17686172 | 920799.057 |
| Philips:hue-color | 2000 | 0.01945771 | 40986.4964 |
|  | 3100 | 0.04141497 | 145493.781 |
|  | 4200 | 0.06612056 | 276992.328 |
|  | 5300 | 0.10289442 | 457673.245 |
|  | 6500 | 0.17196379 | 542101.156 |
| Wiz | 2000 | 0.01945771 | 40986.4964 |
|  | 3100 | 0.04141497 | 145493.781 |
|  | 4200 | 0.06612056 | 276992.328 |
|  | 5300 | 0.10289442 | 457673.245 |
|  | 6500 | 0.17196379 | 542101.156 |
| Philips:scene-switching | 2200 | 0.13410687 | 240374.159 |
|  | 2700 | 0.0303534 | 11125.6469 |
|  | 5000 | 0.04565782 | 88365.869 |

| Table S10. Blue-light hazard efficiency of radiation ηB and the blue light weighted radiance illuminance LB of the conventional lamps weighted by the transmittance of the blue-light-filter (BLF) lenses, with no lens and clear lens provided as control conditions. |  |  |  |
| --- | --- | --- | --- |
| Lamp | BLF | ηB | LB (W m-2 sr-1 nm-1) |
| CFL-cool | CGID | 0.1565707 | 482.9661174 |
|  | Healthlist | 0.00463204 | 7.357474154 |
|  | Prospek | 0.13829157 | 403.1574921 |
|  | Swanwick | 0.00345969 | 5.849832518 |
|  | Clear lenses | 0.19483222 | 664.2536711 |
|  | Crizal | 0.1565707 | 482.9661174 |
|  | Anti Blu-Ray | 0.18405114 | 597.6951464 |
|  | 1.6 UV Blue light | 0.16819144 | 563.9216165 |
|  | BlueProtect | 0.18325085 | 627.4735138 |
| CFL-warm | CGID | 0.04875676 | 142.037065 |
|  | Healthlist | 0.00200817 | 4.162883259 |
|  | Prospek | 0.04176716 | 117.2280953 |
|  | Swanwick | 0.00154604 | 3.493331511 |
|  | Clear lenses | 0.06844906 | 210.8800823 |
|  | Crizal | 0.04875676 | 142.037065 |
|  | Anti Blu-Ray | 0.05966368 | 176.3621388 |
|  | 1.6 UV Blue light | 0.05316914 | 164.7090745 |
|  | BlueProtect | 0.06276407 | 195.9401084 |
| LED-cool | CGID | 0.14410016 | 930.1272131 |
|  | Healthlist | 0.0040101 | 13.97049221 |
|  | Prospek | 0.12740675 | 779.1216824 |
|  | Swanwick | 0.002941 | 11.04153994 |
|  | Clear lenses | 0.16829894 | 1166.121335 |
|  | Crizal | 0.14410016 | 930.1272131 |
|  | Anti Blu-Ray | 0.16837636 | 1142.278815 |
|  | 1.6 UV Blue light | 0.15679058 | 1103.183195 |
|  | BlueProtect | 0.16031314 | 1122.607003 |
| LED-warm | CGID | 0.05429183 | 380.1622727 |
|  | Healthlist | 0.00181822 | 9.168927898 |
|  | Prospek | 0.04798208 | 323.8867304 |
|  | Swanwick | 0.0013047 | 7.213657885 |
|  | Clear lenses | 0.06465476 | 470.8109415 |
|  | Crizal | 0.05429183 | 380.1622727 |
|  | Anti Blu-Ray | 0.06439035 | 460.8125222 |
|  | 1.6 UV Blue light | 0.05990595 | 449.1374228 |
|  | BlueProtect | 0.06176221 | 456.8584041 |
| Incandescent | CGID | 0.02778417 | 124.432786 |
|  | Healthlist | 0.0010715 | 3.887493549 |
|  | Prospek | 0.02494468 | 108.3080139 |
|  | Swanwick | 0.00072665 | 2.878694136 |
|  | Clear lenses | 0.0370653 | 171.6036352 |
|  | Crizal | 0.02778417 | 124.432786 |
|  | Anti Blu-Ray | 0.03291503 | 150.173814 |
|  | 1.6 UV Blue light | 0.03008213 | 143.9209675 |
|  | BlueProtect | 0.03506523 | 164.0622968 |

| Table S11. Blue-light hazard efficiency of radiation ηB and the blue light weighted radiance illuminance LB of the (tunable) lamps weighted by the transmittance of the blue-light-filter (BLF) lenses, with no lens and clear lens provided as control conditions. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Lamp | CCT | Filter | ηB | LB (W m-2sr-1nm-1) |
| Estevez | 2700 | CGID | 0.03776932 | 193285.907 |
|  |  | Healthlist | 0.00157467 | 6033.40221 |
|  |  | Prospek | 0.03374103 | 166958.447 |
|  |  | Swanwick | 0.00110409 | 4648.20679 |
|  |  | Clear lenses | 0.04421259 | 233821.397 |
|  |  | Crizal | 0.03776932 | 193285.907 |
|  |  | Anti Blu-Ray | 0.04448768 | 231522.315 |
|  |  | 1.6 UV Blue light | 0.0417338 | 227988.355 |
|  |  | BlueProtect | 0.04257737 | 229050.244 |
|  | 3600 | CGID | 0.06225864 | 320136.505 |
|  |  | Healthlist | 0.00199916 | 7202.64553 |
|  |  | Prospek | 0.05530359 | 273703.147 |
|  |  | Swanwick | 0.00140305 | 5537.76958 |
|  |  | Clear lenses | 0.07262319 | 389002.421 |
|  |  | Crizal | 0.06225864 | 320136.505 |
|  |  | Anti Blu-Ray | 0.07337075 | 386669.81 |
|  |  | 1.6 UV Blue light | 0.06871994 | 379169.12 |
|  |  | BlueProtect | 0.06975467 | 379616.225 |
|  | 4600 | CGID | 0.08418852 | 433734.578 |
|  |  | Healthlist | 0.00242462 | 8224.11035 |
|  |  | Prospek | 0.07466336 | 368694.339 |
|  |  | Swanwick | 0.00171222 | 6342.97402 |
|  |  | Clear lenses | 0.09799359 | 529746.539 |
|  |  | Crizal | 0.08418852 | 433734.578 |
|  |  | Anti Blu-Ray | 0.09899223 | 526301.504 |
|  |  | 1.6 UV Blue light | 0.09264032 | 514429.219 |
|  |  | BlueProtect | 0.09396179 | 515475.714 |
|  | 5500 | CGID | 0.10457065 | 528884.679 |
|  |  | Healthlist | 0.00287011 | 8990.49704 |
|  |  | Prospek | 0.09276091 | 447942.535 |
|  |  | Swanwick | 0.0020404 | 6958.02683 |
|  |  | Clear lenses | 0.12142363 | 648834.418 |
|  |  | Crizal | 0.10457065 | 528884.679 |
|  |  | Anti Blu-Ray | 0.1225495 | 643673.769 |
|  |  | 1.6 UV Blue light | 0.1146617 | 627602.103 |
|  |  | BlueProtect | 0.11631282 | 630016.94 |
|  | 6500 | CGID | 0.15096925 | 677055.468 |
|  |  | Healthlist | 0.00413846 | 9840.66688 |
|  |  | Prospek | 0.13440497 | 570439.356 |
|  |  | Swanwick | 0.0029949 | 7680.745 |
|  |  | Clear lenses | 0.17394805 | 837370.973 |
|  |  | Crizal | 0.15096925 | 677055.468 |
|  |  | Anti Blu-Ray | 0.17519978 | 827583.263 |
|  |  | 1.6 UV Blue light | 0.16408683 | 803584.772 |
|  |  | BlueProtect | 0.16648835 | 810275.507 |
| Philips:hue-color | 2000 | CGID | 0.01599115 | 30606.3068 |
|  |  | Healthlist | 0.00099772 | 1542.46226 |
|  |  | Prospek | 0.01435241 | 26643.2759 |
|  |  | Swanwick | 0.00065785 | 1120.70122 |
|  |  | Clear lenses | 0.0190799 | 37396.1744 |
|  |  | Crizal | 0.01599115 | 30606.3068 |
|  |  | Anti Blu-Ray | 0.01880089 | 36295.4213 |
|  |  | 1.6 UV Blue light | 0.0176106 | 35744.0159 |
|  |  | BlueProtect | 0.01830925 | 36551.964 |
|  | 3100 | CGID | 0.03414596 | 108077.255 |
|  |  | Healthlist | 0.00139611 | 3382.36425 |
|  |  | Prospek | 0.03012266 | 92240.4158 |
|  |  | Swanwick | 0.00099605 | 2649.78521 |
|  |  | Clear lenses | 0.04058434 | 132505.378 |
|  |  | Crizal | 0.03414596 | 108077.255 |
|  |  | Anti Blu-Ray | 0.04057813 | 130476.316 |
|  |  | 1.6 UV Blue light | 0.03779837 | 127584.902 |
|  |  | BlueProtect | 0.03881141 | 128827.085 |
|  | 4200 | CGID | 0.0549631 | 205659.53 |
|  |  | Healthlist | 0.00186158 | 4980.5923 |
|  |  | Prospek | 0.04838647 | 174496.623 |
|  |  | Swanwick | 0.00134758 | 3946.92116 |
|  |  | Clear lenses | 0.06483245 | 252142.878 |
|  |  | Crizal | 0.0549631 | 205659.53 |
|  |  | Anti Blu-Ray | 0.06523224 | 249562.456 |
|  |  | 1.6 UV Blue light | 0.06077121 | 243607.098 |
|  |  | BlueProtect | 0.06196578 | 244832.131 |
|  | 5300 | CGID | 0.08634934 | 338954.774 |
|  |  | Healthlist | 0.00255457 | 6497.3371 |
|  |  | Prospek | 0.07607896 | 286119.297 |
|  |  | Swanwick | 0.00187068 | 5186.18953 |
|  |  | Clear lenses | 0.10098345 | 416409.391 |
|  |  | Crizal | 0.08634934 | 338954.774 |
|  |  | Anti Blu-Ray | 0.10198369 | 413116.487 |
|  |  | 1.6 UV Blue light | 0.09510012 | 402399.315 |
|  |  | BlueProtect | 0.09646119 | 403634.475 |
|  | 6500 | CGID | 0.14597406 | 397844.166 |
|  |  | Healthlist | 0.00374823 | 5612.4205 |
|  |  | Prospek | 0.12865519 | 332069.499 |
|  |  | Swanwick | 0.00282057 | 4549.21658 |
|  |  | Clear lenses | 0.16896017 | 492684.803 |
|  |  | Crizal | 0.14597406 | 397844.166 |
|  |  | Anti Blu-Ray | 0.17065981 | 488952.195 |
|  |  | 1.6 UV Blue light | 0.15937613 | 473725.384 |
|  |  | BlueProtect | 0.16119765 | 475059.651 |
| Wiz | 2000 | CGID | 0.01599115 | 30606.3068 |
|  |  | Healthlist | 0.00099772 | 1542.46226 |
|  |  | Prospek | 0.01435241 | 26643.2759 |
|  |  | Swanwick | 0.00065785 | 1120.70122 |
|  |  | Clear lenses | 0.0190799 | 37396.1744 |
|  |  | Crizal | 0.01599115 | 30606.3068 |
|  |  | Anti Blu-Ray | 0.01880089 | 36295.4213 |
|  |  | 1.6 UV Blue light | 0.0176106 | 35744.0159 |
|  |  | BlueProtect | 0.01830925 | 36551.964 |
|  | 3100 | CGID | 0.03414596 | 108077.255 |
|  |  | Healthlist | 0.00139611 | 3382.36425 |
|  |  | Prospek | 0.03012266 | 92240.4158 |
|  |  | Swanwick | 0.00099605 | 2649.78521 |
|  |  | Clear lenses | 0.04058434 | 132505.378 |
|  |  | Crizal | 0.03414596 | 108077.255 |
|  |  | Anti Blu-Ray | 0.04057813 | 130476.316 |
|  |  | 1.6 UV Blue light | 0.03779837 | 127584.902 |
|  |  | BlueProtect | 0.03881141 | 128827.085 |
|  | 4200 | CGID | 0.0549631 | 205659.53 |
|  |  | Healthlist | 0.00186158 | 4980.5923 |
|  |  | Prospek | 0.04838647 | 174496.623 |
|  |  | Swanwick | 0.00134758 | 3946.92116 |
|  |  | Clear lenses | 0.06483245 | 252142.878 |
|  |  | Crizal | 0.0549631 | 205659.53 |
|  |  | Anti Blu-Ray | 0.06523224 | 249562.456 |
|  |  | 1.6 UV Blue light | 0.06077121 | 243607.098 |
|  |  | BlueProtect | 0.06196578 | 244832.131 |
|  | 5300 | CGID | 0.08634934 | 338954.774 |
|  |  | Healthlist | 0.00255457 | 6497.3371 |
|  |  | Prospek | 0.07607896 | 286119.297 |
|  |  | Swanwick | 0.00187068 | 5186.18953 |
|  |  | Clear lenses | 0.10098345 | 416409.391 |
|  |  | Crizal | 0.08634934 | 338954.774 |
|  |  | Anti Blu-Ray | 0.10198369 | 413116.487 |
|  |  | 1.6 UV Blue light | 0.09510012 | 402399.315 |
|  |  | BlueProtect | 0.09646119 | 403634.475 |
|  | 6500 | CGID | 0.14597406 | 397844.166 |
|  |  | Healthlist | 0.00374823 | 5612.4205 |
|  |  | Prospek | 0.12865519 | 332069.499 |
|  |  | Swanwick | 0.00282057 | 4549.21658 |
|  |  | Clear lenses | 0.16896017 | 492684.803 |
|  |  | Crizal | 0.14597406 | 397844.166 |
|  |  | Anti Blu-Ray | 0.17065981 | 488952.195 |
|  |  | 1.6 UV Blue light | 0.15937613 | 473725.384 |
|  |  | BlueProtect | 0.16119765 | 475059.651 |
| Philips:scene-switching | 2200 | CGID | 0.11325799 | 177325.147 |
|  |  | Healthlist | 0.00316569 | 2961.1252 |
|  |  | Prospek | 0.10013186 | 149425.998 |
|  |  | Swanwick | 0.00230425 | 2340.14219 |
|  |  | Clear lenses | 0.13172746 | 218643.595 |
|  |  | Crizal | 0.11325799 | 177325.147 |
|  |  | Anti Blu-Ray | 0.13290454 | 216576.905 |
|  |  | 1.6 UV Blue light | 0.12414208 | 210668.821 |
|  |  | BlueProtect | 0.1258738 | 211697.318 |
|  | 2700 | CGID | 0.02460833 | 8173.19602 |
|  |  | Healthlist | 0.0009943 | 269.28148 |
|  |  | Prospek | 0.02166608 | 6978.2101 |
|  |  | Swanwick | 0.00075422 | 224.978353 |
|  |  | Clear lenses | 0.02971066 | 10126.6894 |
|  |  | Crizal | 0.02460833 | 8173.19602 |
|  |  | Anti Blu-Ray | 0.02935844 | 9878.11887 |
|  |  | 1.6 UV Blue light | 0.02724439 | 9626.90178 |
|  |  | BlueProtect | 0.02838898 | 9825.03985 |
|  | 5000 | CGID | 0.03817018 | 66436.8269 |
|  |  | Healthlist | 0.00152615 | 2001.12977 |
|  |  | Prospek | 0.03398364 | 57194.7939 |
|  |  | Swanwick | 0.0010868 | 1565.34607 |
|  |  | Clear lenses | 0.0448002 | 80559.0161 |
|  |  | Crizal | 0.03817018 | 66436.8269 |
|  |  | Anti Blu-Ray | 0.04505297 | 79760.3671 |
|  |  | 1.6 UV Blue light | 0.0422142 | 78440.7574 |
|  |  | BlueProtect | 0.04309399 | 78791.4591 |