Supplementary materials

Table S1 Coefficient of determination (R2), root mean square error (RMSE), and percentage bias (BIAS) of model output vs. calibration (calibration phase: 2008-2014, validation phase: 2015-2019)

|  | Calibration(2008-2014) ¦ Validation(2015-2019) |  |  |
| --- | --- | --- | --- |
| Variable | R2 | RMSE | BIAS |
| Temperture(℃) | 0.98¦0.97 | 2.19¦2.25 | 0.16¦0.42 |
| DO(mg O2/L) | 0.79¦0.73 | 1.15¦1.37 | 0.10¦0.40 |
| Macrophytes Cover(%) | 0.64¦0.68 | 16.73¦19.58 | 5.74¦-0.82 |
| TP(mg P/L) | 0.51¦0.70 | 0.06¦0.04 | 0.03¦0.01 |
| TN(mg N/L) | 0.71¦0.62 | 0.88¦0.53 | 0.15¦-0.14 |
| PO4(mg P/L) | 0.48¦0.86 | 0.01¦0.01 | -0.01¦-0.01 |
| NO3(mg N/L) | 0.67¦0.84 | 0.45¦0.29 | 0.20¦0.24 |
| NH4(mg N/L) | 0.74¦0.77 | 0.28¦0.16 | 0.13¦0.05 |

Fig. S1 Model simulated values (black line) and calibration and validation observations (black solid and white hollow dots, respectively) Gray bands represent the uncertainty associated with the 15% error introduced　into the indications and observations. (a: TEMP; b: DO)

Fig. S2 Model simulated values (black line) and calibration and validation observations (black solid and white hollow dots, respectively) Gray bands represent the uncertainty associated with the 15% error introduced into the indications and observations. (a: PO4; b: NO3; c:NH4)

Fig. S3 Scenario-simulated pettitt mutation test for P control inflection point

Table S2 The WET model uses the original values and set values of the parameters.

| Paramet | Description | Unit | Parameter Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Default | Adjusted |
| Abiotic_Water Module |  |  |  |  |
| cThetaAer | temperature coefficient for reaeration | [−] | 1.024 | 1.001 |
| cThetaNitr | temperature coefficient for nitrification | [−] | 1.08 | 1.000 |
| cVSetPOM | maximum settling rate of POM | m·day−1 | −0.25 | −3.9 |
| cVSetIM | maximum settling rate of inorganic matter | m·day−1 | −1.0 | -0.39 |
| hNO3Denit | quadratic half saturation NO3 conc. for denitrification | mgN·L−1 | 2.0 | 0.31 |
| hO2BOD | half saturation oxygen conc. for BOD | mgO2·L−1 | 1.0 | 2.53 |
| hO2Nitr | half saturation oxygen conc. for nitrification | mgO2·L−1 | 2.0 | 3.0 |
| kNitrW | nitrification rate constant in water | day−1 | 0.1 | 0.04 |
| NO3PerC | denitrified NO3 per mol C mineralised | mol NO3 | 0.8 | 1.5 |
| O2PerNH4 | used O2 per mol NH4+ nitrified | mol O2 | 2.0 | 2.0 |
| cThetaMinPOMW | temperature coefficient for mineralization from POM to DOM | [−] | 1.07 | 1.05 |
| kDMinPOMW | decomposition constant for POM-DW to DOM-DW | day−1 | 0.01 | 0.0367 |
| kNMinPOMW | decomposition constant for POM-N to DOM-N | day−1 | 0.01 | 0.0238 |
| kPMinPOMW | decomposition constant for POM-P to DOM-P | day−1 | 0.01 | 0.0046 |
| kSiMinPOMW | decomposition constant for POM-Si to DOM-Si | day−1 | 0.01 | 0.01 |
| cThetaMinDOMW | temperature coefficient for DOM mineralization | [−] | 1.07 | 1.058 |
| kDMinDOMW | mineralization constant of dissolved organic DW | day−1 | 0.01 | 0.0658 |
| kNMinDOMW | mineralization constant of dissolved organic N | day−1 | 0.01 | 0.038 |
| kPMinDOMW | mineralization constant of dissolved organic P | day−1 | 0.01 | 0.015 |
| kSiMinDOMW | mineralization constant of dissolved organic Si | day−1 | 0.01 | 0.01 |
|  | Abiotic_Sediment Module |  |  |  |
| fRefrPOMS | refractory fraction of sediment POM | [−] | 0.15 | 0.13 |
| O2PerNH4 | O2 used per mol NH4+ nitrified | mol | 2.0 | 2.0 |
| kNitrS | nitrification rate constant | day−1 | 1.0 | 1.29 |
| cThetaNitr | temperature coefficient for nitrification | [−] | 1.08 | 1.00 |
| NO3PerC | NO3 denitrified per mol C mineralised | [−] | 0.8 | 1.5 |
| hNO3Denit | quadratic half-sat. NO3 conc. for denitrification | mgN·L−1 | 2.0 | 0.316 |
| kPSorp | P sorption rate constant not too high -> model speed | day−1 | 0.05 | 0.045 |
| cRelPAdsD | max. P adsorption per g DW | gP·gD−1 | 3 × 10−5 | 2.04× 10−5 |
| cRelPAdsFe | max. P adsorption per g Fe | gP·gFe−1 | 0.065 | 0.065 |
| fFeDIM | Fe content of inorganic. matter | gFe·Gd−1 | 0.01 | 0.041 |
| fRedMax | max. reduction factor of P adsorption affinity | [−] | 0.9 | 0.9 |
| cKPAdsOx | P adsorption affinity at oxidized conditions | m3·gP−1 | 0.6 | 0.6 |
| coPO4Max | max. SRP conc. in pore water | mgP·L−1 | 1.0 | 0.14 |
| cThetaDif | temperature coefficient for diffusion | [−] | 1.02 | 0.95 |
| kNDifNH4 | molecular NH4 diffusion constant | m2·day−1 | 0.000112 | 0.000154 |
| cTurbDifNut | bioturbation factor for diffusion | [−] | 5.0 | 25.81 |
| kO2Dif | molecular O2 diffusion constant | m2·day−1 | 2.6 × 10−5 | 0.00057 |
| cTurbDifO2 | bioturbation factor for diffusion | [−] | 5.0 | 4.68 |
| kDMinHum | maximum decomposition constant of humic material (1D−5) | day−1 | 1 × 10−5 | 2.0159 |
| cThetaMinPOMS | temperature coeff. for sediment mineralization of POM to DOM | [−] | 1.07 | 1.38 |
| kDMinPOMS | mineralization constant in sediment from POM-DW to DOM-DW | day−1 | 0.002 | 0.0152 |
| kNMinPOMS | mineralization constant in sediment from POM-N to DOM-N | day−1 | 0.002 | 0.00087 |
| kPMinPOMS | mineralization constant in sediment from POM-P to DOM-P | day−1 | 0.002 | 0.002 |
| cThetaMinDOMS | exp. temperature constant of sediment mineralization | [−] | 1.07 | 1.06 |
| kDMinDOMS | mineralization constant for sediment dissolved organic matter | day−1 | 0.002 | 0.0025 |
| kNMinDOMS | mineralization constant for sediment dissolved organic N | day−1 | 0.002 | 0.0108 |
| kPMinDOMS | mineralization constant for sediment dissolved organic P | day−1 | 0.002 | 0.0027 |
| kSiMinDOMS | mineralization constant for sediment dissolved organic Si | day−1 | 0.002 | 0.002 |
| kDDifDOM | molecular diffusion constant for dissolved organic matter | m2·day−1 | 0.000112 | 0.00076 |
| kNDifDOM | molecular diffusion constant for dissolved organic N | m2·day−1 | 0.000112 | 0.00051 |
| kPDifDOM | molecular diffusion constant for dissolved organic P | m2·day−1 | 0.000112 | 0.000227 |
| sPAIMS | sediment absorbed phosphate | g·m−2 | 2.0 | 0.288 |

| Auxiliary Module |  |  |  |  |
| --- | --- | --- | --- | --- |
| kVegResus | relative resuspension reduction per gram macrophytes | m2·g DW−1 | 0.01 | 0.05 |
| kTurbFish | relative resuspension by adult fish browsing | g·gfish−1·day−1 | 1.0 | 2.15 |
| cVSedPOM | maximum sedimentation velocity of POM | m·day−1 | 0.25 | 0.5 |
| cVSedDiat | sedimentation velocity of diatoms | m·day−1 | 0.5 | 0.68 |
| cVSedGren | sedimentation velocity of greens | m·day−1 | 0.2 | 0.54 |
| cVSedBlue | sedimentation velocity of blue-greens | m·day−1 | 0.06 | 0.01 |
| crt_shear | critical shear stress | N·m−2 | 0.005 | 0.014 |
|  | Macrophytes Module |  |  |  |
| cDVegIn | external macrophytes density | g D·m2 | 1.0 | 1.0 |
| kMigrVeg | macrophyte migration rate | day−1 | 0.001 | 0.0004 |
| cMuMaxVeg | maximum growth rate of macrophytes at 20 degrees | day−1 | 0.2 | 0.136 |
| cDCarrVeg | maximum macrophytes standing crop | g DW·m−2 | 400.0 | 250.0 |
| cDayWinVeg | day of the year for the end of growing season | day of the year | 259.0 | 259.0 |
| cTmInitVeg | temperature for onset of initial growth | - C | 9.0 | 10.88 |
| cCovSpVeg | specific cover | Gdw−1·m−2 | 0.5 | 0.2 |
| hLRefVeg | half-saturation for influence of light on macrophytes | W·m−2 PAR | 17.0 | 5.89 |
| fWinVeg | fraction surviving in winter ([−]), default = 0.3 | [−] | 0.3 | 0.295 |
| fSedUptVegMax | maximum sediment fraction of nutrient uptake | [−] | 0.998 | 0.5 |
| cHeightVeg | macrophytes height | m | 1.0 | 0.235 |
| cExtSpVeg | specific extinction of macrophytes | m2·g DW | 0.01 | 0.005 |
| cDVegMin | minimum dry weight of macrophytes in system | g DW·m−2 | 1 × 10−5 | 1 × 10−5 |
| cQ10ProdVeg | temperature quotient of production | [−] | 1.2 | 1.2 |
| cQ10RespVeg | temperature quotient of respiration | [−] | 2.0 | 2.0 |

Fig. S4 Research Technology Roadmap