Supplementary Information

Comparative Study of Conventional and Process Intensification by Reactive Distillation Designs for Glycerol Carbonate Production from Glycerol and Diethyl Carbonate

Bushra Chalermthai, Chayanin Sriharuethai, Bradley D. Olsen, Kanokwan Ngaosuwan, Apinan Soottitantawat, Suttichai Assabumrungrat, Pongtorn Charoensuppanimit*

*Corresponding author: pongtorn.ch@chula.ac.th

Binary interaction parameters

Supplementary Table S1. Binary interaction parameter used in GC simulation

| Component i | Component j | Bij | Bji | Cij |
| --- | --- | --- | --- | --- |
| Glycerol | Diethyl carbonate | 1051.4 | 788.549 | 0.3 |
| Glycerol | Glycerol carbonate | -128.13 | 136.22 | 0.3 |
| Glycerol | Ethanol | -189.059 | 626.78 | 0.3 |
| Diethyl carbonate | Glycerol carbonate | 324.759 | 427.781 | 0.3 |
| Diethyl carbonate | Ethanol | -157.84 | 155.95 | 0.3 |
| Glycerol carbonate | Ethanol | 138.681 | -685.73 | 0.3 |

Optimized operating condition results

Supplementary Table S2. Optimized operating condition results of Case I (CSTR).

| Unit | Equipment type | Operating conditions | Source |
| --- | --- | --- | --- |
| S-101 | SSplit | - Split fraction - MIXED stream: 0.99 - CISOLID stream: 0.01 | Designed based on the work of P. Thanahiranya et al. [35] |
| V-101 | Valve | - Outlet pressure: 0.1 bar |  |
| E-101 | Heater | - Outlet temperature: 95°C - Pressure drop: 0 bar |  |
| D-101 | Flash2 | - Pressure drop: 0 bar - Duty: 0 |  |
| P-101 | Pump | - Outlet pressure: 1 bar |  |
| S-102 | Sep | - MONG 99% separation |  |
| M-201 | Mixer | - Pressure drop: 0 bar |  |
| M-202 | Mixer | - Pressure drop: 0 bar |  |
| R-201 | RCSTR | - Pressure: 1 bar - Temperature: 80 OC - Residence time: 4 hours - Catalyst loading: 160 kg | Designed based on the reaction studied by J. Zhang et al. [26] |
| P-201 | Pump | - Outlet pressure: 1.01 bar |  |
| D-201 | RadFrac | - Number of stages: 26 - Feed stages: 8 - Pressure: 1.01 bar | Obtained from GA optimization |
|  |  | - Reflux ratio: 0.07 - D/F ratio: 0.81 | Obtained from Aspen Design Specifications feature (Target 99.9 % recovery of DEC in stream 16) (Target 99.9 % recovery of GL in stream 24) |
| P-202 | Pump | - Outlet pressure: 1.06 bar |  |
| V-201 | Valve | - Outlet pressure: 0.12 bar |  |
| D-202 | RadFrac | - Number of stages: 19 - Feed stages: 9 - Pressure: 1.06 bar | Obtained from GA optimization |
|  |  | - Reflux ratio: 0.78 - D/F ratio: 0.38 | Obtained from Aspen Design Specifications feature (Target 95 wt% EtOH in stream 20) (Target 99.9 wt% DEC in stream 21) |
| D-203 | RadFrac | - Number of stages: 24 - Feed stages: 12 - Pressure: 0.12 bar | Obtained from GA optimization |
|  |  | - Reflux ratio: 3.19 - D/F ratio: 0.24 | Obtained from Aspen Design Specifications feature (Target 97 wt% GC in stream 23) (Target 99 wt% GL in stream 24) |
| V-202 | Valve | - Outlet pressure: 1 bar |  |
| P-203 | Pump | - Outlet pressure: 1 bar |  |

Supplementary Table S3. Optimized operating condition results of Case II (RD).

| Unit | Equipment type | Operating conditions | Design comments |
| --- | --- | --- | --- |
| S-101 | SSplit | - Split fraction - MIXED stream: 0.99 - CISOLID stream: 0.01 | Designed based on the work of P. Thanahiranya et al. [35] |
| V-101 | Valve | - Outlet pressure: 0.1 bar |  |
| E-101 | Heater | - Outlet temperature: 95°C - Pressure drop: 0 bar |  |
| D-101 | Flash2 | - Pressure drop: 0 bar - Duty: 0 |  |
| P-101 | Pump | - Outlet pressure: 1 bar |  |
| S-102 | Sep | - MONG 99 % separation |  |
| V-201 | Valve | - Outlet pressure: 0.17 bar |  |
| M-201 | Mixer | - Pressure drop: 0 bar |  |
| V-202 | Valve | - Outlet pressure: 0.17 bar |  |
| RD-201 | RadFrac | - Number of stages: 21 - S11 Feed stage: 6 - S14 Feed stage: 19 - Pressure: 0.17 bar - Reflux ratio: 1.99 | Obtained from GA optimization |
|  |  | - D/F ratio: 0.48 | Obtained from Aspen Design Specifications feature (Target 95 wt% EtOH in stream 15) |
| P-201 | Pump | - Outlet pressure: 0.96 bar |  |
| D-201 | RadFrac | - Number of stages: 12 - Feed stages: 11 - Pressure: 0.96 bar - Reflux ratio: 0.31 | Obtained from GA optimization |
|  |  | - D/F ratio: 0.69 | Obtained from Aspen Design Specifications feature (Target 97 wt% GC in stream 18) |
| P-201 | Pump | - Outlet pressure: 1 bar |  |

Stream tables

Supplementary Table S4. Stream results of Case I (CSTR).

| Stream no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (OC) | 30.0 | 30.0 | 30.0 | 30.0 | 95.0 | 95.0 | 95.0 | 95.1 | 95.1 | 95.1 |
| Pressure (Bar) | 1.00 | 1.00 | 1.00 | 0.10 | 0.10 | 0.10 | 0.10 | 1.00 | 1.00 | 1.00 |
| Total mass flow (kg/h) | 716.4 | 40.7 | 675.7 | 675.7 | 675.7 | 68.2 | 607.5 | 607.5 | 7.4 | 600.1 |
| - GL (wt %) | 83.32 | 14.67 | 87.46 | 87.46 | 87.46 | 0.78 | 97.19 | 97.19 | 79.84 | 97.41 |
| - DEC (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - GC (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - EtOH (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - Water (wt %) | 11.24 | 1.98 | 11.79 | 11.79 | 11.79 | 94.77 | 2.47 | 2.47 | 2.03 | 2.48 |
| - MeOH (wt %) | 0.48 | 0.08 | 0.50 | 0.50 | 0.50 | 4.45 | 0.06 | 0.06 | 0.05 | 0.06 |
| - MONG (wt %) | 0.19 | 0.03 | 0.20 | 0.20 | 0.20 | 0.00 | 0.22 | 0.22 | 18.08 | 0.00 |
| - ASH (wt %) | 4.77 | 83.23 | 0.05 | 0.05 | 0.05 | 0.00 | 0.06 | 0.06 | 0.00 | 0.06 |

Supplementary Table S4 (continued). Stream results of Case I (CSTR).

| Stream no. | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (OC) | 112.0 | 30.0 | 106.4 | 80.0 | 80.0 | 97.5 | 287.5 | 97.5 | 234.7 | 79.5 |
| Pressure (Bar) | 1.00 | 1.00 | 1.00 | 1.00 | 1.01 | 1.01 | 1.01 | 1.06 | 0.12 | 1.06 |
| Total mass flow (kg/h) | 782.1 | 735.6 | 3,228.3 | 4,010.4 | 4,010.4 | 3,084.8 | 925.6 | 3,084.8 | 925.6 | 592.1 |
| - GL (wt %) | 97.78 | 0.00 | 0.00 | 5.05 | 5.05 | 0.00 | 21.88 | 0.00 | 21.88 | 0.00 |
| - DEC (wt %) | 0.03 | 99.90 | 99.90 | 62.44 | 62.44 | 81.17 | 0.03 | 81.17 | 0.03 | 2.31 |
| - GC (wt %) | 0.20 | 0.00 | 0.00 | 18.01 | 18.01 | 0.00 | 78.05 | 0.00 | 78.05 | 0.00 |
| - EtOH (wt %) | 0.00 | 0.00 | 0.00 | 14.03 | 14.03 | 18.24 | 0.00 | 18.24 | 0.00 | 95.00 |
| - Water (wt %) | 1.90 | 0.10 | 0.10 | 0.45 | 0.45 | 0.58 | 0.00 | 0.58 | 0.00 | 2.63 |
| - MeOH (wt %) | 0.04 | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 | 0.00 | 0.01 | 0.00 | 0.06 |
| - MONG (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - ASH (wt %) | 0.04 | 0.00 | 0.00 | 0.01 | 0.01 | 0.00 | 0.04 | 0.00 | 0.04 | 0.00 |

Supplementary Table S4 (continued). Stream results of Case I (CSTR).

| Stream no. | 21 | 22 | 23 | 24 | 25 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (OC) | 128.1 | 126.0 | 239.7 | 174.5 | 174.6 |  |  |  |  |  |
| Pressure (Bar) | 1.06 | 1.00 | 0.12 | 0.12 | 1.00 |  |  |  |  |  |
| Total mass flow (kg/h) | 2,492.7 | 2,492.7 | 743.6 | 182.1 | 182.1 |  |  |  |  |  |
| - GL (wt %) | 0.00 | 0.00 | 3.00 | 99.00 | 99.00 |  |  |  |  |  |
| - DEC (wt %) | 99.90 | 99.90 | 0.00 | 0.14 | 0.14 |  |  |  |  |  |
| - GC (wt %) | 0.00 | 0.00 | 96.95 | 0.86 | 0.86 |  |  |  |  |  |
| - EtOH (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| - Water (wt %) | 0.10 | 0.10 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| - MeOH (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| - MONG (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| - ASH (wt %) | 0.00 | 0.00 | 0.05 | 0.00 | 0.00 |  |  |  |  |  |

Supplementary Table S5. Stream results of Case II (RD).

| Stream no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (OC) | 30.0 | 30.0 | 30.0 | 30.0 | 95.0 | 95.0 | 95.0 | 95.1 | 95.1 | 95.1 |
| Pressure (Bar) | 1.00 | 1.00 | 1.00 | 0.10 | 0.10 | 0.10 | 0.10 | 1.00 | 1.00 | 1.00 |
| Total mass flow (kg/h) | 716.4 | 40.7 | 675.7 | 675.7 | 675.7 | 68.2 | 607.5 | 607.5 | 7.4 | 600.1 |
| - GL (wt %) | 83.32 | 14.67 | 87.46 | 87.46 | 87.46 | 0.78 | 97.19 | 97.19 | 79.84 | 97.41 |
| - DEC (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - GC (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - EtOH (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - Water (wt %) | 11.24 | 1.98 | 11.79 | 11.79 | 11.79 | 94.77 | 2.47 | 2.47 | 2.03 | 2.48 |
| - MeOH (wt %) | 0.48 | 0.08 | 0.50 | 0.50 | 0.50 | 4.45 | 0.06 | 0.06 | 0.05 | 0.06 |
| - MONG (wt %) | 0.19 | 0.03 | 0.20 | 0.20 | 0.20 | 0.00 | 0.22 | 0.22 | 18.08 | 0.00 |
| - ASH (wt %) | 4.77 | 83.23 | 0.05 | 0.05 | 0.05 | 0.00 | 0.06 | 0.06 | 0.00 | 0.06 |

Supplementary Table S5 (continued). Stream results of Case II (RD).

| Stream no. | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (OC) | 92.4 | 30.0 | 96.2 | 73.7 | 39.0 | 78.8 | 78.9 | 220.6 | 124.9 | 124.9 |
| Pressure (Bar) | 0.17 | 1.00 | 1.00 | 0.17 | 0.17 | 0.17 | 0.96 | 0.96 | 0.96 | 1.00 |
| Total mass flow (kg/h) | 600.1 | 788.5 | 2,536.3 | 2,536.3 | 615.6 | 2,520.8 | 2,520.8 | 773.0 | 1,747.8 | 1,747.8 |
| - GL (wt %) | 97.41 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - DEC (wt %) | 0.00 | 99.90 | 99.96 | 99.96 | 2.40 | 70.24 | 70.24 | 3.00 | 99.99 | 99.99 |
| - GC (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 29.73 | 29.73 | 96.96 | 0.00 | 0.00 |
| - EtOH (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 95.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - Water (wt %) | 2.48 | 0.10 | 0.04 | 0.04 | 2.54 | 0.01 | 0.01 | 0.00 | 0.01 | 0.01 |
| - MeOH (wt %) | 0.06 | 0.00 | 0.00 | 0.00 | 0.06 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - MONG (wt %) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| - ASH (wt %) | 0.06 | 0.00 | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.04 | 0.00 | 0.00 |