Appendix A: System Algorithms

Algorithm 1 On-Device TFLM Inference Logic

1: Initialize: tflm interpreter, input tensor, output tensor

2:

3: function RunInference()

4: // Populate the model’s input tensor

5: input tensor.data[0] ← current temperature

6: input tensor.data[1] ← current pH

7: input tensor.data[2] ← current TDS

8: input tensor.data[3] ← current turbidity

9: // Run the TFLM interpreter

10: tflm interpreter.Invoke()

11: // Get raw probability scores

12: output scores ← output tensor.data

13: // Find class with highest score

14: max score ← -1.0

15: predicted index ← -1

16: for i from 0 to NumberOfClasses - 1 do

17:	if output scores[i] > max score then

18:	max score ← output scores[i]

19:	predicted index ← i

### 20:	end if 21: end for

22: predicted label ← MapIndexToLabel(predicted index)

23: UpdateSystemState(predicted label)

### 24: end function

Algorithm 2 Continuous SD Card Logging

1: Initialize: Sensor pins, LCD, SD Card, Backend connection

### 2: loop

3:	sensorData ← ReadAllSensors(samples=100)

4:	processedData ← ComputeMedian(sensorData)

5:	DisplayOnLCD(processedData)

6:	SendToBackend(processedData)

7:	WriteToSDCard(timestamp, processedData)

8:	Wait(1 second)

### 9: end loop

Algorithm 3 Fluctuation-Based SD Card Logging

1: Initialize: lastLoggedData, thresholds

### 2: loop

3:	currentSensorData ← ReadAllSensors(samples=100)

4:	currentData ← ComputeMedian(currentSensorData)

5:	//Check for significant fluctuation

6:	hasFluctuation ← false

7:	for each parameter p in currentData do

8:	if abs(currentData[p] - lastLoggedData[p]) > thresholds[p] then

9:	hasFluctuation ← true

### 10:		break

### 11:	end if

### 12:end for

13:	if hasFluctuation is true then

14:	WriteToSDCard(timestamp, currentData)

15:	lastLoggedData ← currentData

### 16:	end if

17:	DisplayOnLCD(currentData)

18:	SendToBackend(currentData)

19:	Wait(1 second)

### 20: end loop