Supplementary Materials

Table S1. Technology-Specific Inventory Parameters (Source: USEEIO v2.0 (EPA, 2020); TEİAŞ, 2023; Akyüz et al., 2024; Broesicke et al., 2024)

| Technology | USEEIO Sector Code | Primary Energy Input | Amount (per pax) | Unit | Output (kWh-eq/pax) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Solar PV/BIPV | 221100 | Grid electricity (backup) | 0.50 | kWh | 2.50 (gen) – 1.80 (export) | Net export credit applied |
| On-site Wind | 221100 | Grid electricity (backup) | 0.35 | kWh | 3.00 (gen) – 2.20 (export) | Capacity factor = 30% |
| GSHP | 221100 | Electricity | 1.20 | kWh | 4.20 (thermal) | COP = 3.5 |
| Biomass / WtE | 321100, 562000 | Wood pellets | 0.80 | kg | 2.00 | Biogenic CO₂ excluded |
| AI/BMS | 518200 | ICT infrastructure | 0.05 | kWh | -0.40 (saving) | 15% energy reduction |
| SAF & e-GSE | 324110 | Sustainable aviation fuel | 0.20 | kg | 1.50 | 80% GHG reduction vs fossil |
| CHP / Tri-generation | 221100, 221200 | Natural gas | 1.20 | m³ | 2.0 (el), 1.5 (heat), 0.8 (cool) | Exergy-based allocation |
| Notes: USEEIO v2.0 data; Turkish grid mix (TEİAŞ 2023); COP/EER from literature. |  |  |  |  |  |  |

Table S2. Turkish Electricity Grid Mix (2023) (Source: TEİAŞ, 2023; EPA TRACI 2.1, 2020; IEA, 2022)

| Fuel Source | Share (%) | CO₂ (g/kWh) | SO₂ (mg/kWh) | NOₓ (mg/kWh) | PM₂.₅ (mg/kWh) |
| --- | --- | --- | --- | --- | --- |
| Natural Gas | 37.2 | 490 | 120 | 450 | 20 |
| Coal (lignite) | 36.8 | 980 | 2500 | 1800 | 350 |
| Hydroelectric | 15.4 | 24 | 0 | 0 | 0 |
| Wind | 6.8 | 11 | 0 | 0 | 0 |
| Solar PV | 2.1 | 48 | 0 | 0 | 0 |
| Geothermal | 1.7 | 38 | 50 | 20 | 0 |
| TOTAL | 100 | 548.9 | 1050 | 830 | 141 |

Table S3. Exergy-Based Allocation for CHP / Tri-generation (Source: IEA ETSAP CHP Technology Brief E04, 2015; Broesicke et al., 2024)

| Output | Energy Share (%) | Exergy Factor | Exergy-Weighted Value |
| --- | --- | --- | --- |
| Electricity | 40 | 1.00 | 0.400 |
| Heat (80 °C) | 45 | 0.25 | 0.113 |
| Cooling (7 °C) | 15 | 0.20 | 0.030 |