| Supplementary Table S1. Top 26 features for all participants based on weighted importance |  |  |
| --- | --- | --- |
| Features | HC (n = 43) | Early PDs (n = 63) |
| P1_MedLat_COP-COM (cm) | 28.05±4.43 | 31.31±2.90 |
| TP_COM_Speed (m/s) | 0.29±0.07 | 0.19±0.06 |
| P1_Left_BF_iEMG (µV·s) | 347.87±267.38 | 662.55±769.48 |
| P2_COM_Pspeed (m/s) | 0.80±0.16 | 0.63±0.16 |
| P1_COM_Sway_Area (cm2) | 36.52±27.88 | 25.05±20.90 |
| P2_Right_RF_iEMG (µV·s) | 606.38±358.90 | 503.77±219.47 |
| P1_Left_Ankle_ROM (°) | 4.52±9.51 | 9.79±13.95 |
| STS_P_Right_RF_iEMG (µV·s) | 545.59±457.63 | 905.67±800.01 |
| P2_Left_BF_PAmp (µV) | 23.65±13.02 | 37.55±23.67 |
| P1_Right_Hip_Mom (N·mm/kg) | 2.88±1.69 | 7.34±7.53 |
| P1_Right_Ankle_POW (W/kg) | 0.01±0.02 | 0.10±0.20 |
| P2_AP_COP-COM (cm) | 54.40±32.32 | 31.93±14.23 |
| P1_Left_GAS_RMS (µV) | 3.99±5.98 | 2.64±2.07 |
| P1_RMS_ASI_TA (%) | 46.97±20.15 | 40.29±19.42 |
| P1_Median_Freq (-) | 851.53±19.27 | 848.44±51.72 |
| P1_Right_RF_PAmp (µV) | 45.09±37.40 | 63.71±73.60 |
| STS_P_RMS_ASI_GAS (%) | 35.60±20.82 | 28.69±19.71 |
| P2_Left_Vetical_PGRF (BW) | 0.80±0.37 | 1.03±0.10 |
| P2_RMS_ASI_TA (%) | 41.80±16.16 | 44.89±23.58 |
| P2_T10_For_ROM (°) | 18.80±10.25 | 17.75±7.33 |
| P2_Right_RF_RMS (µV) | 9.06±5.49 | 6.93±3.24 |
| STS_P_RMS_ASI_TA (%) | 46.98±22.15 | 42.51±18.95 |
| P1_Left_Knee_ROM (°) | 15.66±9.77 | 24.20±22.68 |
| P2_Right_TCH (cm) | 7.94±1.45 | 7.22±1.05 |
| P1_AP_COP-COM (cm) | 32.83±14.31 | 23.15±8.00 |
| P2_Left_BF_iEMG (µV·s) | 520.80±288.94 | 891.15±921.84 |
| All data are presented as mean ± standard deviation. HC: Healthy control; PD: Parkinson’s disease; P1: Phase 1; MedLat: Mediolateral; COP: Center of press; COM: Center of mass; cm: centimeter; TP: Total phase; m/s: Meters per second; BF: Biceps femoris; iEMG: Integrated electromyography; µV·s: Microvolt-seconds; P2: Phase 2; Pspeed: Peak speed; cm²: Square centimeter; ROM: Range of motion; °: Degrees; STS_P: Sit-to-stand phase; PAmp: Peak amplitude; µV: Microvolt; Mom: Moment; N·mm/kg: Newton-millimeters per kilogram; POW: Power; W/kg: Watts per kilogram; AP: Anterior-Posterior; GAS: Gastrocnemius; RMS: Root mean square; ASI: Asymmetry Index; TA: Tibialis anterior; %: Percentage; Freq: Frequency; PGRF: Peak ground reaction force; BW: body weight; T10: The 10th thoracic vertebra; For: Forward; TCH: Toe clearance height. |  |  |

| Supplementary Table S2. Accuracies, precision, recall, and F1 scores of the seven classifiers from five-fold cross-validation |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | MLs | Early-stage PD vs. HC (200 Features) | Early-stage PD vs. HC (26 Features) | Early-stage PD vs. HC (3 Features) |
| Accuracy | LR | 78.6±5.5 | 88.2±4.6 | 76.2±6.2 |
|  | KNN | 75.3±8.8 | 83.4±7.6 | 83.3±3.5 |
|  | NB | 84.9±3.5 | 82.5±2.3 | 73.8±4.2 |
|  | LDA | 78.6±9.6 | 81.0±5.0 | 78.6±7.7 |
|  | QDA | 76.2±3.8 | 88.1±2.9 | 76.2±5.5 |
|  | SVM | 79.5±8.5 | 80.2±5.7 | 78.6±3.7 |
|  | RF | 87.3±3.2 | 92.1±2.8 | 84.9±5.3 |
| Precision | LR | 79.5±4.9 | 88.8±4.2 | 77.3±7.2 |
|  | KNN | 75.6±9.0 | 84.3±8.3 | 83.8±3.2 |
|  | NB | 86.5±3.9 | 83.4±2.4 | 77.3±5.5 |
|  | LDA | 79.7±8.5 | 81.4±5.1 | 79.7±8.5 |
|  | QDA | 84.0±1.6 | 88.6±3.0 | 78.7±5.8 |
|  | SVM | 80.5±8.4 | 81.9±5.2 | 79.0±8.4 |
|  | RF | 88.4±3.4 | 92.4±2.9 | 85.8±6.1 |
| Recall | LR | 78.6±5.5 | 88.2±4.6 | 76.2±6.2 |
|  | KNN | 75.3±8.8 | 83.4±7.6 | 83.3±3.5 |
|  | NB | 84.9±3.5 | 82.5±2.3 | 73.8±4.2 |
|  | LDA | 78.6±9.6 | 81.0±5.0 | 78.6±7.7 |
|  | QDA | 76.2±3.8 | 88.1±2.9 | 76.2±5.5 |
|  | SVM | 79.5±8.5 | 80.2±5.7 | 78.6±8.7 |
|  | RF | 87.3±3.2 | 92.1±2.8 | 84.9±5.3 |
| F1 score | LR | 78.4±5.8 | 88.1±4.7 | 76.1±6.0 |
|  | KNN | 75.3±8.8 | 83.3±7.6 | 83.3±3.0 |
|  | NB | 84.7±3.5 | 82.4±2.4 | 73.1±4.3 |
|  | LDA | 78.1±10.3 | 80.9±5.0 | 78.4±7.7 |
|  | QDA | 74.7±4.3 | 88.0±2.9 | 75.7±7.7 |
|  | SVM | 79.3±8.7 | 79.9±6.0 | 78.5±8.8 |
|  | RF | 87.2±3.2 | 92.0±2.8 | 84.8±5.2 |
| Values are expressed as mean (%) ± standard deviation (%) from five-fold cross-validation. Mean values presented in boldface denote the best performance (highest value) for each metric; MLs: Machine-learning techniques; PD: Parkinson’s disease; HC: Healthy control LR: Logistic regression; KNN: K-nearest neighbors; NB: Naїve Bayes; LDA: Linear discriminant analysis; QDA: Quadratic discriminant analysis; SVM: Support vector machine; RF: Random Forest. |  |  |  |  |

Supplementary Table S3. Result of Correlation Analysis for 200 Features

This table is provided as an Excel file (Supplementary Table S3.xlsx).

| Supplementary Table S4. Anatomical locations of 39 reflective markers based on the Plug-in Gait model |  |  |
| --- | --- | --- |
| Position | Definition | Maker |
| Left temple | Left front head | LFHD |
| Right temple | Right front head | RFHD |
| Left back of head (defines the transverse plane of the head, together with the frontal markers) | Left back head | LBHD |
| Right back of head (defines the transverse plane of the head, together with the frontal markers) | Right back head | RBHD |
| On the spinous process of the 7th cervical vertebra | 7th cervical vertebra | C7 |
| On the spinous process of the 10th thoracic vertebra | 10th thoracic vertebra | T10 |
| On the jugular notch where the clavicles meet the sternum | Clavicle | CLAV |
| On the xiphoid process of the sternum | Sternum | STRN |
| Anywhere over the right scapula | Right back | RBAK |
| On the acromio-clavicular joint | Left shoulder | LSHO |
| On the acromio-clavicular joint | Right shoulder | RSHO |
| On the lower lateral 1/3 surface of the left arm (Place asymmetrically with RUPA) | Left upper arm | LUPA |
| On the lower lateral 1/3 surface of the right arm (Place asymmetrically with LUPA) | Right upper arm | RUPA |
| On the lateral epicondyle approximating the elbow joint axis | Left elbow | LELB |
| On the lateral epicondyle approximating the elbow joint axis | Right elbow | RELB |
| On the lower lateral 1/3 surface of the left forearm (Place asymmetrically with RFRM) | Left forearm | LFRM |
| On the lower lateral 1/3 surface of the right forearm (Place asymmetrically with LFRM) | Right forearm | RFRM |
| At the thumb (radial) side of the left wrist, as close to the wrist joint center as possible | Left wrist marker A | LWRA |
| At the thumb (radial) side of the right wrist, as close to the wrist joint center as possible | Right wrist marker A | RWRA |
| At the little finger (ulna) side of the left wrist, as close to the wrist joint center as possible | Left wrist marker B | LWRB |
| At the little finger (ulna) side of the right wrist, as close to the wrist joint center as possible | Right wrist marker B | RWRB |
| Just proximal to the middle knuckle on the left hand | Left Finger | LFIN |
| Just proximal to the middle knuckle on the right hand | Right Finger | RFIN |
| Left anterior superior iliac spine | Left ASIS | LASI |
| Right anterior superior iliac spine | Right ASIS | RASI |
| Left posterior superior iliac spine (immediately below the sacro-iliac joints, at the point where the spine joins the pelvis) This marker is used with the RPSI marker as an alternative to the single SACR marker | Left PSIS | LPSI |
| Right posterior superior iliac spine (immediately below the sacro-iliac joints, at the point where the spine joins the pelvis) This marker is used with the LPSI marker as an alternative to the single SACR marker | Right PSIS | RPSI |
| Over the lower lateral 1/3 surface of the left thigh in the same plane as the hip and knee joint centers and knee flexion-extension axis | Left thigh | LTHI |
| Over the lower lateral 1/3 surface of the right thigh in the same plane as the hip and knee joint centers and knee flexion-extension axis | Right thigh | RTHI |
| On the flexion-extension axis of the left knee | Left knee | LKNE |
| On the flexion-extension axis of the right knee | Right knee | RKNE |
| Over the lower 1/3 surface of the left shank, in the same plane as the knee and ankle joint centres and ankle flexion/extension axis | Left tibia | LTIB |
| Over the lower 1/3 surface of the right shank, in the same plane as the knee and ankle joint centres and ankle flexion/extension axis | Right tibia | RTIB |
| On the calcaneus at the same height above the plantar surface of the foot as the toe marker | Left heel | LHEE |
| On the calcaneus at the same height above the plantar surface of the foot as the toe marker | Right heel | RHEE |
| On the lateral malleolus along an imaginary line that passes through the transmalleolar axis | Left ankle | LANK |
| On the lateral malleolus along an imaginary line that passes through the transmalleolar axis | Right ankle | RANK |
| Over the second metatarsal head, on the midfoot side of the equinus break between forefoot and mid-foot | Left toe | LTOE |
| Over the second metatarsal head, on the midfoot side of the equinus break between forefoot and mid-foot | Right toe | RTOE |

Supplementary Table S5. Description of 200 Features

This table is provided as an Excel file (Supplementary_Table_S5.xlsx).

Supplementary Table S6. Model evaluation metrics for RF and XGBoost

| Model | Accuracy | Precision | Recall | F1 Score | AUC |
| --- | --- | --- | --- | --- | --- |
| RF | 0.77±0.12 | 0.78±0.13 | 0.77±0.12 | 0.76±0.13 | 0.89±0.06 |
| XGBoost | 0.70±0.08 | 0.70±0.09 | 0.70±0.08 | 0.69±0.09 | 0.81±0.06 |

Mean (%) ± standard deviation (%) values are calculated from five-fold cross-validation. AUC: Area under the receiver operating characteristic curve; RF: Random Forest; XGBoost: Extreme Gradient Boosting.

| Supplementary Table S7. Model parameters of the seven classifiers estimated by grid search |  |  |  |
| --- | --- | --- | --- |
| MLs | Early-stage PD vs. HC (200 Features) | Early-stage PD vs. HC (26 Features) | Early-stage PD vs. HC (3 Features) |
| LR | C=0.01 | C=0.01 | C=10 |
| KNN | k=5 | k=9 | k=3 |
| NB | - | - | - |
| LDA | n_components=1 | n_components=1 | n_components=1 |
| QDA | reg_param=0.001 | reg_param=0.5 | reg_param=0.5 |
| SVM | C = 7.63, gamma = 0.0001, kernel = rbf | C= 443.67, gamma=0.00001, kernel=rbf | C= 6660.85, gamma=0.1, kernel=rbf |
| RF | max_depth=25, n_estimators=1500 | max_depth=15, n_estimators=1500 | max_depth=10, n_estimators=500 |
| ML: Machine learning; PD: Parkinson’s disease; HC: Healthy control; LR: Logistic regression (C is the inverse of the regularization strength); KNN: K-nearest neighbor (k is the number of neighbors); NB: Naïve Bayes; LDA: linear discriminant analysis (n_components is the number of components); QDA: quadratic discriminant analysis (reg_param is the regularization of the per-class covariance); SVM: support vector machine; RF: Random forest (C is the regularization parameter, and gamma is the kernel coefficient); “n_estimators” is the number of trees in the forest, and “max_depth” is the maximum depth of the tree. |  |  |  |

Supplementary Fig. S1. Definition of events and phases. Event 1 (E1) marks the onset of forward movement of the C7 marker. Event 2 (E2) is the time at which the anteroposterior ground reaction force peaks during seat-off, when the buttocks lift from the chair. Event 3 (E3) corresponds to heel-off of the first stepping foot. Event 4 (E4) is the heel-strike of the first stepping foot. Event 5 (E5) is the second heel-strike of the opposite limb, marking the completion of the second step. The analysis intervals are defined as: the sit-to-stand phase from E1 to E2 (STS_P), the first interval from E1 to E3 (P1), and the second interval from E3 to E5 (P2).