Bio-Inspired Chemometric Methods for Simultaneous UV Spectrophotometric Determination of Molnupiravir, Nirmatrelvir, and Favipiravir in Pharmaceutical Formulations and Environmental Samples

Ahmed M. Abdelzaher a,*, Omkulthom Al kamaly b, Mona A. Abdel Rahman c,

a Pharmaceutical Analytical Chemistry Department, Faculty of Pharmacy, Al-Azhar University, Nasr City 11751, Cairo, Egypt

b Department of Pharmaceutical Sciences, College of Pharmacy, Princess Nourah bint Abdulrahman University, P.O. Box 84428, Riyadh 11671, Saudi Arabia

c Analytical Chemistry Department, Faculty of Pharmacy, October 6 University, P.O. Box 12585, 6 October City, Giza, Egypt

*Corresponding author email address: (Ahmed M. Abdelzaher)  Ahmed.abdelzaher86@hotmail.com

Text S1: Operational Principles of GA-PLS and FFA-PLS Algorithms

The Genetic Algorithm-PLS (GA-PLS) method was implemented through evolutionary optimization of wavelength selection. Binary-encoded chromosomes represented wavelength subsets, with each bit indicating selection (1) or exclusion (0) of a specific wavelength. An initial population of chromosomes was randomly generated and evaluated using leave-one-out cross-validation to calculate fitness based on root mean square error of cross-validation (RMSECV). Superior chromosomes were selected for reproduction through genetic operators including crossover and mutation. The evolutionary process continued iteratively until convergence, yielding an optimized wavelength subset. To account for the stochastic nature of genetic algorithm optimization, the GA-PLS procedure was repeated 100 times using different random seeds. Wavelength selection frequency was recorded across all runs, and wavelengths selected in more than 70% of runs were retained as the final optimized subset for each analyte, ensuring that only consistently informative wavelengths contributed to the final calibration model. PLS calibration models were then constructed using the selected wavelengths, with the optimal number of latent variables determined through cross-validation.

The Firefly Algorithm-PLS (FFA-PLS) approach employed swarm intelligence principles for wavelength optimization. Fireflies representing wavelength selection vectors were initialized with continuous values subsequently converted to binary selections. Light intensity, inversely proportional to RMSECV, quantified solution quality. Fireflies moved toward brighter individuals according to attractiveness functions decreasing with distance, while incorporating random exploration components. This iterative process identified optimal wavelength combinations, followed by PLS model development using the selected variables. Similarly, the FFA-PLS optimization was repeated 100 times using different random seeds to verify the stability of the swarm intelligence-based wavelength selection. Wavelengths selected in more than 70% of runs were retained as the final optimized subset.

Table S1: Concentrations of the 25-mixture calibration set designed using Brereton partial factorial methodology.

| No. | Favipiravir (µg/mL) | Molnupiravir (µg/mL) | Nirmatrelvir (µg/mL) |
| --- | --- | --- | --- |
| 1 | 8 | 8 | 16 |
| 2 | 8 | 4 | 8 |
| 3 | 4 | 4 | 24 |
| 4 | 4 | 12 | 12 |
| 5 | 12 | 6 | 24 |
| 6 | 6 | 12 | 16 |
| 7 | 12 | 8 | 12 |
| 8 | 8 | 6 | 12 |
| 9 | 6 | 6 | 20 |
| 10 | 6 | 10 | 24 |
| 11 | 10 | 12 | 20 |
| 12 | 12 | 10 | 16 |
| 13 | 10 | 8 | 24 |
| 14 | 8 | 12 | 24 |
| 15 | 12 | 12 | 8 |
| 16 | 12 | 4 | 20 |
| 17 | 4 | 10 | 8 |
| 18 | 10 | 4 | 16 |
| 19 | 4 | 8 | 20 |
| 20 | 8 | 10 | 20 |
| 21 | 10 | 10 | 12 |
| 22 | 10 | 6 | 8 |
| 23 | 6 | 4 | 12 |
| 24 | 4 | 6 | 16 |
| 25 | 6 | 8 | 8 |

Table S2: Concentrations of the 20-mixture validation set designed using central composite methodology.

| No. | Favipiravir (µg/mL) | Molnupiravir (µg/mL) | Nirmatrelvir (µg/mL) |
| --- | --- | --- | --- |
| 1 | 11 | 11 | 22 |
| 2 | 5 | 5 | 22 |
| 3 | 11 | 5 | 10 |
| 4 | 8 | 8 | 16 |
| 5 | 5 | 5 | 10 |
| 6 | 11 | 11 | 10 |
| 7 | 11 | 5 | 22 |
| 8 | 8 | 8 | 16 |
| 9 | 8 | 8 | 16 |
| 10 | 5 | 11 | 22 |
| 11 | 5 | 11 | 10 |
| 12 | 8 | 8 | 16 |
| 13 | 8 | 8 | 16 |
| 14 | 8 | 8 | 7 |
| 15 | 8 | 8 | 25 |
| 16 | 8 | 12.5 | 16 |
| 17 | 8 | 3.5 | 16 |
| 18 | 8 | 8 | 16 |
| 19 | 3.5 | 8 | 16 |
| 20 | 12.5 | 8 | 16 |

Table S3: Optimized algorithm-specific parameters for GA-PLS models used in wavelength selection and multivariate calibration for simultaneous determination of favipiravir, molnupiravir, and nirmatrelvir.

| Parameter | Favipiravir | Molnupiravir | Nirmatrelvir |
| --- | --- | --- | --- |
| Population size | 44 | 48 | 56 |
| Maximum generations | 120 | 120 | 240 |
| Mutation rate | 0.004 | 0.003 | 0.006 |
| % Wavelength used at initiation | 15 | 20 | 20 |
| Window width | 1 | 1 | 1 |
| % Population same at convergence | 80 | 90 | 90 |
| Crossover type | Double | Double | Double |
| Maximum number of latent variables | 4 | 5 | 6 |
| Cross-validation | Random | Random | Random |
| Number of subsets (CV) | 25 | 25 | 25 |
| Number of iterations | 2 | 2 | 2 |

| Parameter | Favipiravir | Molnupiravir | Nirmatrelvir |
| --- | --- | --- | --- |
| Number of fireflies | 48 | 48 | 56 |
| Maximum generations | 250 | 250 | 300 |
| α | 0.2 | 0.2 | 0.5 |
| βο | 1.2 | 1.2 | 1.4 |
| γ | 1 | 1 | 1 |

Table S4: Optimized algorithm-specific parameters for FFA-PLS models used in wavelength selection and multivariate calibration for simultaneous determination of favipiravir, molnupiravir, and nirmatrelvir.

Table S5:  Accuracy and precision data at three concentration levels (low, medium, high) including mean recovery percentages with standard deviations, intra-day precision, and inter-day precision for assessment of method reliability according to ICH Q2(R2) guidelines.

| Concentration (µg/mL) |  |  | Accuracy (% R ± SD) |  |  |  | Precision (%RSD) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Favipiravir (µg/mL) | Molnupiravir (µg/mL) | Nirmatrelvir (µg/mL) | Favipiravir | Molnupiravir | Nirmatrelvir | Favipiravir |  |  |  |  |  | Molnupiravir |  |  | Nirmatrelvir |  |  |  |
|  |  |  |  |  |  | Intra-day |  |  |  | Inter-day |  | Intra-day | Inter-day |  | Intra-day |  | Inter-day |  |
| 6 | 6 | 12 | 99.47 ± 1.438 | 98.82 ± 1.347 | 96.74 ± 1.551 |  |  | 1.445 | 1.863 |  | 1.363 |  |  | 1.745 |  | 1.603 |  | 2.079 |  |
| 8 | 8 | 16 | 98.22 ± 1.414 | 101.46 ± 1.079 | 102.85 ± 1.495 |  |  | 1.439 | 1.815 |  | 1.064 |  |  | 1.299 |  | 1.453 |  | 1.887 |  |
| 10 | 10 | 20 | 101.48 ± 1.302 | 99.41 ± 1.156 | 99.74 ± 1.238 |  |  | 1.283 | 1.856 |  | 1.163 |  |  | 1.727 |  | 1.241 |  | 1.817 |  |

Table S6: Selectivity assessment against pharmaceutical excipients and environmental co-contaminants.

| Interferent | Favipiravir (8 µg/mL) | Molnupiravir (8 µg/mL) | Nirmatrelvir (16 µg/mL) |
| --- | --- | --- | --- |
|  | Recovery ± RSD (%) | Recovery ± RSD (%) | Recovery ± RSD (%) |
| None (Control) | 100.53 ± 1.12 | 101.58 ± 1.515 | 101.75 ± 1.337 |
| Pharmaceutical excipients |  |  |  |
| Lactose monohydrate | 101.33 ± 1.112 | 97.87 ± 1.215 | 97.83 ± 1.122 |
| Microcrystalline cellulose | 96.79 ± 1.600 | 101.8 ± 1.558 | 103.31 ± 1.319 |
| Magnesium stearate | 97.83 ± 1.366 | 99.79 ± 1.289 | 98.07 ± 1.475 |
| Povidone (PVP K-30) | 100.22 ± 1.856 | 98.86 ± 1.45 | 99.33 ± 1.288 |
| Croscarmellose sodium | 100.3 ± 1.477 | 100.93 ± 1.482 | 96.62 ± 1.303 |
| Starch | 99.32 ± 1.457 | 100.02 ± 1.664 | 98.31 ± 1.731 |
| Titanium dioxide | 101.83 ± 1.726 | 99.89 ± 1.858 | 97.94 ± 1.811 |
| Talc | 101.39 ± 1.614 | 97.13 ± 1.823 | 98.24 ± 1.297 |
| Environmental/co-contaminants |  |  |  |
| Humic acid | 96.6 ± 1.677 | 96.43 ± 1.138 | 105.77 ± 1.073 |
| Sodium dodecyl sulfate (SDS) | 97.74 ± 1.739 | 103.09 ± 1.148 | 103.81 ± 1.677 |
| Ibuprofen | 96.95 ± 1.155 | 101.86 ± 1.218 | 95.90 ± 1.148 |
| Paracetamol | 98.75 ± 1.696 | 102.77 ± 1.787 | 96.82 ± 1.378 |

Fig. S1: Chemical structures of favipiravir, molnupiravir and nirmatrelvir.

Fig. S2: Two-dimensional projection of experimental design space (favipiravir vs. molnupiravir). Calibration mixtures (teal circles, 25 samples) and validation mixtures (pink triangles, 20 samples) demonstrate comprehensive coverage with distinct separation between training and test sets.

Fig. S3: Two-dimensional projection of experimental design space (favipiravir vs. nirmatrelvir). Calibration mixtures (teal circles, 25 samples) and validation mixtures (pink triangles, 20 samples) demonstrate comprehensive coverage with distinct separation between training and test sets.

Fig. S4: Two-dimensional projection of experimental design space (molnupiravir vs. nirmatrelvir). Calibration mixtures (teal circles, 25 samples) and validation mixtures (pink triangles, 20 samples) demonstrate comprehensive coverage with distinct separation between training and test sets.