Deptuła Milena1,*, Zawrzykraj Małgorzata2, Sawicka Justyna3, Dzierżyńska Maria3, Skoniecka Aneta4, Czerwiec Katarzyna2, Kondej Karolina5, Zieliński Jacek6, Janus Łukasz7, Rodziewicz-Motowidło Sylwia3, Pikuła Michał1,8

Peptide-enriched hydrogel formulation for sensitive and damaged skin: from design to application testing

- Laboratory of Tissue Engineering and Regenerative Medicine, Division of Embryology, Medical University of Gdańsk, Poland

- Division of Clinical Anatomy, Medical University of Gdańsk, Poland

- Department of Biomedical Chemistry, Faculty of Chemistry, University of Gdańsk, Poland

- Division of Embryology, Medical University of Gdańsk, Poland

- Department of Plastic Surgery, Medical University of Gdańsk, Poland

- Department of Surgical Oncology, Transplant Surgery and General Surgery, Medical University of Gdańsk, Poland

- BioGel Sp. z o.o., Szybowcowa 8a, 80-298 Gdańsk, Poland

- Department of Biochemistry, Gdańsk University of Physical Education and Sport, Gdańsk, Poland

*- Correspondence: Milena Deptuła, PhD Eng., email: milenadeptula@gumed.edu.pl, Laboratory of Tissue Engineering and Regenerative Medicine, Division of Embryology, Medical University of Gdansk, Debinki 1, 80-211 Gdansk, Poland; ORCID: 0000-0003-4875-770X

Figure 1S. Levels of eotaxin (A), IL-8 (B), IL-1a (C), TNF-alpha (D), G-CSF (E) after IM2+NE1, P407 and composition stimulation of skin cells evaluated with Luminex technology. Graphs present data from 3 independent experiments, no statistical significance was detected (Mann Whitney U-test, p<0.05).

Table 1S. Validation of the neutralization of the preservative system in the test using Eugon LT100

| Tested stains | Nvf (unit/ml*) | Nvn (unit/ml) | Nvf ≥ 0,5 Nvn* |  | Nv (unit/ml) |  |
| --- | --- | --- | --- | --- | --- | --- |
| S. aureus ATCC 6538 | 3,5 x 103 | 5,2 x 103 |  | > 0,5 |  | 5,6 x 103 |
| E. coli ATCC 8739 | 3,3 x 103 | 4,9 x 103 |  | > 0,5 |  | 5,3 x 103 |
| Ps. Aeruginosa ATCC 9027 | 3,0 x 103 | 5,1 x 103 |  | > 0,5 |  | 5,4 x 103 |
| C. albicans ATCC 10231 | 3,3 x 103 | 5,3 x 103 |  | > 0,5 |  | 5,7 x 103 |
| A. brasiliensis ATCC 16404 | 2,5 x 103 | 3,4 x 103 |  | > 0,5 |  | 3,7 x 103 |

*unit/ml – colony forming unit in 1ml

** Nvf ≥ 0.5 Nvn – assumption according to PN-EN ISO 11930:2019

Nvf – number of microorganisms in 1ml of a mixture consisting of: 1ml of the test sample + 9ml of neutralizer + 1ml of the test strain suspension (N) from a dilution of 10-4 (bacteria)/10-3 (C. albicans and A. brasiliensis)

Nvn – Nvn – number of microorganisms in 1ml of a mixture of: 1ml of diluent + 9ml of neutralizer + 1ml of the test strain suspension (N) from a dilution of10-4 (bacteria)/10-3 (C. albicans  and A. brasiliensis)

Nv – Nv – number of microorganisms in the control: 10 ml of diluent + 1 ml of suspension of the test strain (N) from a dilution of10-4 (bakteria)/10-3 (C. albicans and A. brasiliensis)

Table 2S. Challange test

| Density of suspensions of test strains used for contamination of the tested samples |  |  |
| --- | --- | --- |
| Test strains | N (unit/ml) Number of microorganisms in a calibrated suspension | N0 = N/100 (unit/ml) The number of microorganisms in the sample tested immediately after contamination (T0) |
| S. aureus ATCC 6538 | 4,7 x 107 | 4,7 x 105 |
| E. coli ATCC 8739 | 4,2 x 107 | 4,2 x 105 |
| Ps. Aeruginosa ATCC 9027 | 4,0 x 107 | 4,0 x 105 |
| C. albicans ATCC 10231 | 3,6 x 106 | 3,6 x 104 |
| A. brasiliensis ATCC 16404 | 3,0 x 106 | 3,0 x 104 |

Table 3S. Number of microorganisms in 1 ml of sample at specified time intervals

| Test strains | T0 (unit/ml) | Ig N0 |  | T7 (unit/ml) | Ig N7 | T14 (unit/ml) | Ig N14 | T28 (unit/ml) | Ig N28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S. aureus ATCC 6538 | 4,7 x 105 |  | 5,67 | <10 | 1 | <10 | 1 | <10 | 1 |
| E. coli ATCC 8739 | 4,2 x 105 |  | 5,62 | <10 | 1 | <10 | 1 | <10 | 1 |
| Ps. Aeruginosa ATCC 9027 | 4,0 x 105 |  | 5,60 | <10 | 1 | <10 | 1 | <10 | 1 |
| C. albicans ATCC 10231 | 3,6 x 104 |  | 4,55 | <10 | 1 | <10 | 1 | <10 | 1 |
| A. brasiliensis ATCC 16404 | 3,0 x 104 |  | 4,47 | <10 | 1 | <10 | 1 | <10 | 1 |

Table 4S. The value of the reduction of the number of microorganisms in the sample at time intervals

Rx = IgN0 - IgNx

| Tested strains | T7 | T14 | T28 |
| --- | --- | --- | --- |
| S. aureus ATCC 6538 | 4,67 | 4,67 | 4,67 |
| E. coli ATCC 8739 | 4,62 | 4,62 | 4,62 |
| Ps. Aeruginosa ATCC 9027 | 4,60 | 4,60 | 4,60 |
| C. albicans ATCC 10231 | 3,55 | 3,55 | 3,55 |
| A. brasiliensis ATCC 16404 | 3,47 | 3,47 | 3,47 |
| Reduction values ​​(Rx) are expressed in logarithmic units |  |  |  |

Table 5S. Assessment criteria for the maintenance effectiveness test according to the standard PN-EN ISO 11930:2019

|  | Bacteria |  |  | C. albicans |  |  | A. brasiliensis |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time | T7 | T14 | T28 | T7 | T14 | T28 | T14 | T28 |
| Criterion A | ≥3 | ≥3 NIa | ≥3 NI | ≥1 | ≥1 NI | ≥1 NI | ≥0b | ≥1 |
| Criterion B | not tested | ≥3 | ≥3 NI | not tested | ≥1 | ≥1 NI | ≥0 | ≥0 NI |

Reduction values ​​calculated from the formula: Rx = IgN0 - IgNx

a NI – the number of microorganisms does not increase

b Rx = 0 when IgN0 = IgNx

Table 6. Stability and packaging compatibility testing

| Parameter | Requirements | Results |
| --- | --- | --- |
| Character | Transparent, clear liquid, odorless | Compatible |
| pH | - | 5,5 |
| Mechanical contamination | does not contain | absent |
| Sample stability at -5°C 10 cycles: 1. Placing the sample at a reduced temperature (-5°C) for 24 hours 2. Taking the sample out for 8 hours – return to room temperature 3. Evaluation of any changes / mass destabilization / reactivity with packaging 4. Placing the sample at a reduced temperature for 16 hours until the next evaluation | No signs of mass destabilization, reactivity with packaging or other changes | No changes observed |
| Sample stability at room temperature +18°C/+20°C without light 10 cycles: 1. Assessment of possible changes / mass destabilization / reactivity with packaging every 24h | No signs of mass destabilization, reactivity with packaging or other changes | No changes observed |
| Stability at +38°C 10 cycles: 1. Placing the sample at an elevated temperature for 24 hours 2. Taking the sample out for 8 hours – return to room temperature 3. Evaluation of any changes / mass destabilization / reactivity with packaging 4. Placing the sample in specified temperature conditions for 16 hours until the next evaluation | No signs of mass destabilization, reactivity with packaging or other changes | No changes observed |
| Stability of the preservation system: | ISO 11930:2019 | Compatible |
| Compatibility of weight with packaging | complete | Compatible |