Supplementary material

Table S1. Techno-economic assumptions for the production of bioactive-rich oil and defatted biomass from N. oculata.

| Variable | Unit | Value |
| --- | --- | --- |
| Operational details |  |  |
| Process capacity (microalgae) | kg h-1 | 720/360/180 |
| Capacity utilization rate | % | 85 |
| Operating labor productivity | employee-h kg-1 | 146 |
| Shift length | hours shift-1 | 8 |
| Production schedule | days year-1 | 330 |
| Construction time of the plant | year | 1 |
| Plant operation time (y) | year | 9 |
| Location | - | Brazil |
| Plant area | m² | 2000 |
| Financial criteria |  |  |
| Depreciation method | - | linear |
| Equipment | year | 10 |
| Buildings | year | 25 |
| Local tax | % profit before Tax | 0.34 |
| Minimum acceptable rate of return (MARR) | % | 20 |
| Weighted average cost of capital (WACC) | % | 20 |
| Salvage value | % of buildings | 60 |
| Lifetime project | year | 10 |
| Cost criteria |  |  |
| Maintenance and repairs | % direct fixed capital investment | 6 |
| Operating supplies | % of the maintenance and repairs | 10 |
| Laboratory charges | % of the operating labor | 10 |
| Patents and royalties | % of the total product cost. | 2 |
| Contingency | % direct fixed capital investment | 9 |
| Local taxes | % fixed capital investment | 2 |
| Insurance | % fixed capital investment. | 1 |
| Packaging | % for operating labor and maintenance | 6 |
| Indirect Labor | % for operating labor and maintenance | 40 |
| Warehouse and storage facilities | % for operating labor and maintenance | 3.5 |
| Administrative Expenses | % of the manufacturing cost | 8 |
| Selling and distribution expenses | % sales | 10 |
| Costs |  |  |
| Microalgae biomass cost | US$ kg-1 | 25 |
| Microalgae oil selling price | US$ kg-1 | 100 |
| Defatted biomass selling price | US$ kg-1 | 1.5 |
| Ethanol | US$ l-1 | 0.83 |
| Coagulant | US$ kg-1 | 0.3 |
| Electric energy | US$ kWh-1 | 0.14 |
| Thermal energy | US$ GJ-1 | 3.22 |
| Waste disposal | US$ t-1 | 36 |
| Water | US$ kg-1 | 0.008 |
| Operating labor cost including benefits | US$ h-1 | 25.0 |
| Research and development | % sales | 2 |

Table S2. Fatty acid methyl ester profile of Nannochloropsis oculata oil.

| Fatty acid | Content (%)* |
| --- | --- |
| C14:0 | 5.0 ± 0.4 |
| C16:0 | 24.6 ± 0.3 |
| C16:1 | 22.2 ± 0.5 |
| C18:1C | 5.5 ± 0.5 |
| C18:2C | 2.6 ± 0.1 |
| C18:3G | 11.0 ± 1.1 |
| C24:0 | 5.5 ± 0.4 |
| C20:5 | 25.6 ± 1.5 |
| ∑SFA | 35.05 ± 0.9 |
| ∑MUFA | 27.65 ± 0.2 |
| ∑PUFA | 39.11 ± 1.1 |

*Mean value ± standard deviation (n = 3); SFA: saturated fatty acids; MUFA: monounsaturated fatty acid; PUFA: polyunsaturated fatty acids.

Table S3. Detailed material balance of all process streams shown in Figure 2 for the production of bioactive-rich oil and defatted biomass from Nannochloropsis oculata. Mass flow rates are expressed in kg h⁻¹. Biomass is divided into lipids and other biomass components (non-lipid fraction).

| Stream | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mass Flow (kg h-1) | 144000.0 | 1.4 | 131015.5 | 12985.9 | 8657.0 | 4329.0 | 3607.0 | 721.0 | 728.0 | 725.0 | 724.0 | 520.9 | 204.0 | 446.5 | 276.2 |
| Biomass (kg h-1) | 720.0 | 0.0 | 720.0 | 648.0 | 0.0 | 648.0 | 0.0 | 648.0 | 0.0 | 203.2 | 444.8 | 0.2 | 203.0 | 444.8 | 0.0 |
| Lipids (kg h-1) | 360.0 | 0.0 | 360.0 | 324.0 | 0.0 | 324.0 | 0.0 | 324.0 | 0.0 | 203.2 | 120.8 | 0.2 | 203.0 | 120.8 | 0.0 |
| Other biomass components (kg h-1) | 360.0 | 0.0 | 360.0 | 324.0 | 0.0 | 324.0 | 0.0 | 324.0 | 0.0 | 0.0 | 324.0 | 0.0 | 0.0 | 324.0 | 0.0 |
| Water (kg h-1) | 143280.0 | 0.0 | 130943.4 | 12336.6 | 8657.0 | 3679.0 | 3607.0 | 72.0 | 0.0 | 48.0 | 25.0 | 47.5 | 0.1 | 0.1 | 24.4 |
| Flocculant (kg h-1) | 0.0 | 1.4 | 0.1 | 1.3 | 0.0 | 1.3 | 0.0 | 1.3 | 0.0 | 0.0 | 1.3 | 0.0 | 0.0 | 1.3 | 0.0 |
| Ethanol (kg h-1) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 728.0 | 474.0 | 253.4 | 473.2 | 0.9 | 1.6 | 251.8 |

Table S4. Equipment sizing for the production of bioactive-rich oil and defatted biomass from Nannochloropsis oculata.

| Equipment | 720 kg h-1 |  |  | 360 kg h-1 |  |  | 180 kg h-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Quantity | Capacity (kg h-1) | Energy demand (kWh year-1) | Quantity | Capacity (kg h-1) | Energy demand (kWh year-1) | Quantity | Capacity (kg h-1) | Energy demand (kWh year-1) |
| Decanter | 1 | 720 | - | 1 | 360 | - | 1 | 180 | - |
| Dewatering press | 3 | 648 | 5.2E+04 | 2 | 324 | 3.5E+04 | 1 | 162 | 1.7E+04 |
| Tray Dryers | 11 | 648 | 3.4E+07 | 6 | 324 | 1.8E+07 | 3 | 162 | 9.2E+06 |
| Extractor | 1 | 648 | - | 1 | 324 | - | 1 | 162 | - |
| Evaporator | 1 | 276 | 1.3E+06 | 1 | 138 | 6.4E+05 | 1 | 69 | 3.0E+05 |
| Desolventizer | 1 | 372 | 8.6E+04 | 1 | 186 | 4.4E+04 | 1 | 93 | 2.3E+04 |

Table S5. Annual production costs for microalgae bioactive-rich oil and defatted biomass obtained from Nannochloropsis oculata across three plant capacities (720, 360 and 180 kg h-1).

|  | Processing capacity (Kg/h) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 720 |  | 360 |  | 180 |  |
| Costs item | (Million US$ /year) | % | (Million US$/year) | % | (Million US$/year) | % |
| # Raw Materials | # 142.56 | # 77.67 | # 71.28 | # 77.64 | # 35.64 | # 77.61 |
| # Microalgae | # 142.56 | # 77.67 | # 71.28 | # 77.64 | # 35.64 | # 77.61 |
| # Reactant | # 0.01 | # 0.01 | # 0.01 | # 0.01 | # 0.00 | # 0.01 |
| # Coagulant | # 0.00 | # 0.00 | # 0.00 | # 0.00 | # 0.00 | # 0.00 |
| # Ethanol | # 0.01 | # 0.00 | # 0.00 | # 0.00 | # 0.00 | # 0.00 |
| # Operating Labor | # 2.38 | # 1.29 | # 1.19 | # 1.29 | # 0.59 | # 1.29 |
| # Utilities | # 0.29 | # 0.16 | # 0.15 | # 0.16 | # 0.07 | # 0.16 |
| # Electricity | # 0.01 | # 0.00 | # 0.00 | # 0.01 | # 0.00 | # 0.01 |
| # Thermal energy | # 0.10 | # 0.06 | # 0.05 | # 0.06 | # 0.03 | # 0.06 |
| # Waste disposal | # 0.18 | # 0.10 | # 0.09 | # 0.10 | # 0.04 | # 0.10 |
| # Water | # 0.00 | # 0.00 | # 0.00 | # 0.00 | # 0.00 | # 0.00 |
| # Maintenance and repairs | # 0.17 | # 0.10 | # 0.10 | # 0.11 | # 0.05 | # 0.12 |
| # Operating Supplies | # 0.02 | # 0.01 | # 0.01 | # 0.01 | # 0.01 | # 0.01 |
| # Laboratory chages | # 0.24 | # 0.13 | # 0.12 | # 0.13 | # 0.06 | # 0.13 |
| # Patents and Royalties | # 0.40 | # 0.22 | # 0.201 | # 0.22 | # 0.101 | # 0.22 |
| # Direct production costs | # 146.07 | # 79.58 | # 73.05 | # 79.57 | # 36.53 | # 79.55 |
| # Depreciation | # 0.11 | # 0.06 | # 0.07 | # 0.07 | # 0.04 | # 0.08 |
| # Local taxes | # 0.08 | # 0.05 | # 0.05 | # 0.05 | # 0.03 | # 0.06 |
| # Insurance | # 0.04 | # 0.02 | # 0.02 | # 0.03 | # 0.01 | # 0.03 |
| # Fixed Charges | # 0.24 | # 0.13 | # 0.14 | # 0.15 | # 0.08 | # 0.17 |
| # Packaging | # 0.15 | # 0.08 | # 0.08 | # 0.08 | # 0.04 | # 0.08 |
| # Indirect Labor | # 1.02 | # 0.56 | # 0.51 | # 0.56 | # 0.26 | # 0.56 |
| # Warehouse and storage facilities | # 0.09 | # 0.05 | # 0.05 | # 0.05 | # 0.02 | # 0.05 |
| # Plant-overhead | # 1.26 | # 0.69 | # 0.64 | # 0.69 | # 0.32 | # 0.70 |
| # Manufacturing Costs | # 147.57 | # 80.40 | # 73.82 | # 80.41 | # 36.93 | # 80.41 |
| # Administrative Expenses | # 11.81 | # 6.43 | # 5.91 | # 6.43 | # 2.95 | # 6.43 |
| # Selling and Distribution Expenses | # 20.14 | # 10.97 | # 10.07 | # 10.97 | # 5.03 | # 10.96 |
| # Research and development | # 4.03 | # 2.19 | # 2.01 | # 2.19 | # 1.01 | # 2.19 |
| # General Expenses | # 35.97 | # 19.60 | # 17.99 | # 19.59 | # 8.99 | # 19.59 |
| Total Product Cost | 183.538 | 100.00 | 91.810 | 100.00 | 45.922 | 100.00 |

*Production at 85% of capacity.