Flow Cytometry Analysis and Gating Strategy for E7-Specific CD8+ T Cells

To accurately quantify antigen-specific T cell responses, PBMCs were stained and analyzed using a rigorous sequential gating strategy to eliminate artifacts and non-specific signals. The detailed protocol is as follows:

(1) Singlet Gating (SSC-A vs. SSC-Width): To prevent false-positive signals from clumped cells, doublets were initially excluded. Single cells (Singlets) were stringently gated on an SSC-A (X-axis) versus SSC-Width (Y-axis) plot by delineating the primary linear population.

(2) Lymphocyte Gating (FSC-A vs. SSC-A): Following singlet selection, the target lymphocyte population was identified based on physical morphological properties. A morphological gate was applied using FSC-A (relative cell size) and SSC-A (internal granularity) to filter out smaller cell debris and larger, granular leukocyte subsets such as monocytes and granulocytes.

(3) Live/Dead Gating (live_dead BV510-A vs. SSC-A): Since dead cells exhibit non-specific antibody binding that increases background noise, live cells were selected. Dead cells were excluded by gating the negative population on the FL7-A :: live_dead BV510-A channel, with the threshold set strictly below the 104 fluorescence intensity mark.

(4) Effector Cell Identification (CD8 APC-A750-A vs. SSC-A): Within the live lymphocyte gate, cytotoxic T cells were identified. The CD8+ T cell population was precisely gated by defining the positive population with a fluorescence intensity threshold of >104 on the FL5-A :: CD8 APC-A750-A channel.

(5) Antigen-Specific T Cell Quantification (E7 PE-A vs. CD8 APC-A750-A): Finally, to evaluate the specific immune response, the E7-specific cells were quantified within the parental CD8+ T cell population. On a biaxial plot, quadrant gates were established with thresholds set at 104 for both FL10-A :: E7 PE-A (X-axis) and FL5-A :: CD8 APC-A750-A (Y-axis). The E7 tetramer-positive cells were identified in quadrant 2 (Q2, PE >10^4 and APC-A750 >10^4).