Supplementary Methods

Image Acquisition

The initial 10 participants obtained MR images using the General Electric 3T MRI Discovery 750 HD (MR console version DV26.0_R05_2008) and a 24-channel head coil. The remaining 18 participants received images using the General Electric SIGNA Ultra High Performance (UHP) 3.0 T MRI (MR console version MR30.1_R01_2322.c) and a 48-channel head coil.

T1-weighted images were collected using a BRAVO 3D inversion recovery-prepared fast spoiled echo gradient (FSPGR) sequence. Acquisition parameters included a repetition time (TR) = 8.4 ms, echo time (TE) = 3.2 ms, inversion time (TI) = 650 ms, flip angle 10˚, field of view of 256 mm x 256 mm x 188 mm, matrix size of 256 x 256 x 188, GRAPPA (ARC) factor of two in the phase-encoded direction, and sagittal-oblique orientation to approximate alignment with anterior commissure (AC)-posterior commissure (PC).

T2-weighted images used a 3D fast-spin echo (FSE) sequence. Parameters were as follows: TR = 3000 ms, TE = 60-90 ms, echo train length of 130, a field of view measuring 256 mm x 225 mm x 188 mm, matrix size of 256 x 256 x 188, a sagittal-oblique orientation approximating alignment with the AC-PC axis, and GRAPPA (ARC) factor of two utilized in the phase-encoded and slice direction.

The initial 10 participants obtained isotropic 3D ZTE images with the following parameters: TR = 698 ms, TE = 16 µs, flip angle = 1˚, receiver bandwidth  62.5 kHz, number of averages = 3.5, field of view measuring 256 mm x 256 mm x 256 mm, matrix size of 256 x 256 x 256. The remaining 18 participants obtained isotropic 3D ZTE images with the parameters TR = 531 ms, TE = 16 µs, flip angle = 1˚, receiver bandwidth  62.5 kHz, number of averages = 3.5, field of view measuring 256 mm x 256 mm x 256 mm, matrix size of 256 x 256 x 256.

Supplementary Figures

Supplementary Figure 1. Multifocal normalized acoustic intensity profiles in the brain using BabelBrain for 5 representative participants, showing reliable targeting of multiple focal spots. In situ ISPPA of 10 W/cm2 and ISPTA of 1 W/cm2.

Supplementary Tables

Supplementary Table 1. Post-sonication questionnaire on interoceptive experience. Number of reported sensations at baseline and post-TUS or -sham in unifocal and multifocal groups.

|  | Unifocal |  |  |  | Multifocal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sham |  | TUS |  | Sham |  | TUS |  |  |  |
|  | Baseline | Post-Sonication | Baseline | Post-Sonication | Baseline | Post-Sonication | Baseline |  | Post-Sonication | Area Targeted |
| Skin numbness, tingling, or itching? | 1 | 1 | 1 | 2 | 1 | 1 | 0 |  | 0 | Medial Lemniscus |
| Unusual muscle twitches, movements, tightness, or contraction? | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 4 |  | Cerebral Peduncle |
| Changes in hearing or vision (double vision)? | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | CN III and IV |
| Weakness in the face, hand, arm, leg, or foot? | 1 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | Cerebral Peduncle |
| Difficulty speaking? | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | Cerebral Peduncle |
| Decreased coordination? | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | Cerebral Peduncle |
| Feelings of anxiousness or nervousness? | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  | 0 | Medial Lemniscus |

Note: Participants were asked to focus on changes on the right side of their body as TUS was applied to the left STN.

Supplementary Table 2. Post-procedure safety questionnaire41

| Rate the following, based on how you feel right now | Value: - (1) absent - (2) mild - (3) moderate - (4) severe | Related to stimulation? - (1) definitely not related - (2) unlikely to be related - (3) possibly related - (4) definitely related |
| --- | --- | --- |
| Weakness in face, hand, arm, leg, or foot |  |  |
| Difficulty speaking or swallowing |  |  |
| Decreased coordination |  |  |
| Headache |  |  |
| Skin numbness, tingling, itching |  |  |
| Muscle pain |  |  |
| Fatigue |  |  |
| Difficulty paying attention |  |  |
| Tooth pain |  |  |
| Change in hearing or vision |  |  |
| Nausea |  |  |
| Unusual twitches or muscle movements |  |  |
| Dizziness |  |  |
| Feeling anxious or nervous |  |  |
| Feeling forgetful |  |  |
| Difficulty with balance/walking |  |  |
| Other/Comments: |  |  |

Note: Participants were asked to rate each side effect and indicate whether they thought it was related to stimulation.

Supplementary Table 3. Comparison of baseline values in unifocal and multifocal groups.

|  | Unifocal |  |  |  | Multifocal |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Outcome | Sham | TUS | Test statistic | p-value | Sham | TUS | Test statistic | p-value |
| Congruent response time (ms) | 630.79 90.53 | 610.92 108.91 | -1.07 | .286 | 631.61 106.63 | 628.84 98.18 | .11 | .915 |
| Incongruent response time (ms) | 692.94 112.34 | 668.28 125.98 | .79 | .441 | 679.96 131.17 | 670.22 125.21 | .32 | .759 |
| Stroop effect (ms) | 62.15 41.71 | 57.36 34.95 | -.07 | .948 | 48.35 34.86 | 41.38 34.58 | -.87 | .386 |
| Congruent accuracy (%) | 99.11 1.11 | 99.11 1.32 | -.22 | .823 | 99.03 1.05 | 97.64 3.21 | -1.43 | .152 |
| Incongruent accuracy (%) | 98.50 2.35 | 98.61 1.49 | -.25 | .805 | 98.68 1.24 | 98.13 2.54 | -.48 | .633 |
| Finger tapping frequency (Hz) | 5.53 .65 | 5.61 .54 | -.95 | .354 | 5.56 .54 | 5.56 .55 | .06 | .952 |

Supplementary Table 4. Two-way repeated measures ANOVA results in combined Unifocal x Multifocal groups.

|  | F-statistic | p-value | Conditional Result |  |
| --- | --- | --- | --- | --- |
| Congruent Response Time |  |  |  |  |
| Condition | 1.289 | .267 | Sham: 595.97 (± 93.78) ms | TUS: 581.57 (± 79.68) ms |
| Method | .241 | .628 |  |  |
| Interaction | .003 | .958 |  |  |
| Incongruent Response Time |  |  |  |  |
| Condition | 2.401 | .133 | Sham: 650.51 (± 119.34) ms | TUS: 625.44 (± 103.22) ms |
| Method | .004 | .953 |  |  |
| Interaction | .016 | .900 |  |  |
| Stroop Effect |  |  |  |  |
| Condition | 4.235 | .050 | Sham: 54.55 (± 40.32) ms | TUS: 43.87 (± 37.03) ms |
| Method | .872 | .359 |  |  |
| Interaction | .069 | .794 |  |  |
| Congruent Accuracy |  |  |  |  |
| Condition | .256 | .617 | Sham: 98.66 (± 1.24) % | TUS: 98.54 (± 2.00) % |
| Method | .111 | .742 |  |  |
| Interaction | .147 | .705 |  |  |
| Incongruent Accuracy |  |  |  |  |
| Condition | .001 | .981 | Sham: 97.87 (± 2.13) % | TUS: 97.82 (± 2.24) % |
| Method | .655 | .426 |  |  |
| Interaction | .218 | .644 |  |  |
| Finger Tapping Frequency |  |  |  |  |
| Condition | .386 | .540 | Sham: 5.46 (± .58) Hz | TUS: 5.49 (± .42) Hz |
| Method | .102 | .752 |  |  |
| Interaction | .921 | .346 |  |  |

Supplementary Table 5. Comparison between sham and TUS separately in unifocal and multifocal groups.

|  | Unifocal (n = 18) |  |  |  | Multifocal (n = 10) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Outcome | Sham | TUS | Test statistic | p-value | Sham | TUS | Test statistic | p-value |
| Congruent response time (ms) | 590.05 ± 94.33 | 576.14 ± 80.92 | -1.11 | .267 | 606.61 ± 96.85 | 591.34 ± 80.69 | 1.04 | .327 |
| Incongruent response time (ms) | 648.88 ± 117.31 | 625.30 ± 103.15 | 1.05 | .310 | 653.45 ± 129.29 | 625.68 ± 108.92 | 1.56 | .153 |
| Stroop effect (ms) | 58.83 ± 37.97 | 49.16 ± 35.19 | 1.31 | .208 | 46.84 ± 45.30 | 34.34 ± 40.24 | -1.99 | .047 |
| Congruent accuracy (%) | 98.69 ± 1.16 | 98.65 ± 1.57 | -.32 | .752 | 98.61 ± 1.43 | 98.33 ± 2.70 | -.68 | .497 |
| Incongruent accuracy (%) | 97.69 ± 2.41 | 97.53 ± 2.31 | -.40 | .686 | 98.19 ± 1.58 | 98.33 ± 2.13 | -.56 | .574 |
| Finger tapping frequency (Hz) | 5.51 ± .64 | 5.48 ± .40 | .25 | .810 | 5.38 ± .49 | 5.49 ± .48 | -1.64 | .136 |

Supplementary Table 6. Statistical comparison of average scores of side effects following sham and TUS.

|  | Sham | TUS | p-value |
| --- | --- | --- | --- |
| Weakness in face, hand, arm, leg, or foot | 1.04 .19 | 1.00 .00 | .322 |
| Difficulty speaking | 1.00 .00 | 1.00 .00 | N/A |
| Decreased coordination | 1.04 .19 | 1.00 .00 | .326 |
| Headache | 1.04 .19 | 1.04 .19 | 1.00 |
| Skin numbness, tingling, itching | 1.07 .26 | 1.07 .26 | 1.00 |
| Muscle pain | 1.00 .00 | 1.04 .19 | .326 |
| Fatigue | 1.54 .64 | 1.25 .44 | .057 |
| Difficulty paying attention | 1.32 .55 | 1.25 .44 | .593 |
| Tooth pain | 1.11 .57 | 1.00 .00 | .326 |
| Change in hearing or vision | 1.00 .00 | 1.07 .26 | .161 |
| Nausea | 1.00 .00 | 1.00 .00 | N/A |
| Unusual twitches or muscle movements | 1.00 .00 | 1.00 .00 | N/A |
| Dizziness | 1.00 .00 | 1.00 .00 | N/A |
| Feeling anxious or nervous | 1.04 .19 | 1.04 .19 | 1.00 |
| Feeling forgetful | 1.00 .00 | 1.00 .00 | N/A |
| Difficulty with balance/walking | 1.00 .00 | 1.00 .00 | N/A |