Case Study Parameters and Input Data

The probabilistic simulations and economic models rely on a set of well-defined parameters to ensure the results are realistic and applicable to the Egyptian context. These parameters, summarized in Table 1 and 2 , are derived from manufacturer specifications, high-resolution Egyptian meteorological data, international lifecycle assessment (LCA) databases, and national grid standards. The use of these standardized and cited values ensures the reproducibility and validity of the optimization framework.

Table 1: Key Parameters for Probabilistic Simulation and Economic Modeling

| Parameter | Symbol | Value / Range | Unit | Source / Justification |
| --- | --- | --- | --- | --- |
| A. Probabilistic & Renewable Models |  |  |  |  |
| Weibull shape parameter (Wind) | k | 2.1 | --- | Fitted from Red Sea coast meteorological data [20]; defines the input uncertainty distribution. |
| Weibull scale parameter (Wind) | c | 7.5 | m/s | Fitted from Red Sea coast meteorological data [20]; defines the input uncertainty distribution. |
| Wind turbine cut-in speed | vci | 3.0 | m/s | Manufacturer datasheet (2.9 MW turbine) |
| Wind turbine rated speed | vr | 12.0 | m/s | Manufacturer datasheet (2.9 MW turbine) |
| Wind turbine cut-out speed | vco | 25.0 | m/s | Manufacturer datasheet (2.9 MW turbine) |
| PV temperature coefficient | α | -0.0045 | 1/°C | Standard for crystalline silicon modules |
| PV Nominal Operating Cell Temp. | NOCT | 45 | °C | Manufacturer datasheet |
| PV Standard Test Condition Irradiance | G_STC | 1000 | W/m² | IEC 61215 standard |
| PV Standard Test Condition Temp. | T_STC | 25 | °C | IEC 61215 standard |
| B. Power System & Operational Limits |  |  |  |  |
| Minimum voltage limit | Vmin | 0.95 | p.u. | Egyptian Distribution Grid Code |
| Maximum voltage limit | Vmax | 1.05 | p.u. | Egyptian Distribution Grid Code |
| Base load scaling factor (stress test) | --- | 115 | % | Represents stressed Egyptian feeder conditions [5, 6] |
| C. Economic Parameters |  |  |  |  |
| Capital Cost - PV System | --- | 800 - 1000 | $/kW | Egyptian market estimates, 2024 |
| Capital Cost - Wind System | --- | 1200 - 1500 | $/kW | Egyptian market estimates, 2024 |
| Capital Cost - D-STATCOM | --- | 50 - 80 | $/kVAr | Industry estimates |
| Capital Cost - Capacitor Bank | --- | 15 - 25 | $/kVAr | Industry estimates |
| Discount Rate (for LCOE) | --- | 6.5 | % | Central Bank of Egypt benchmark |
| Project Lifetime | N | 20 | years | Standard for energy infrastructure |
| D. Life-Cycle Assessment (LCA) Parameters |  |  |  |  |
| Emission Factor - PV Manufacturing | EFM,PV | 80 | kg CO₂-eq/kW | Eco invent DB / Literature [23] |
| Emission Factor - Wind Manufacturing | EFM,WT | 40 | kg CO₂-eq/kW | Eco invent DB / Literature [23] |
| Emission Factor - Grid Electricity (Egypt) | EF grid | 0.55 | kg CO₂-eq/kWh | Egyptian Electricity Holding Co. report |
| Recycled Fraction (Metals) | R_REC | 0.85 | --- | Industry average for steel/aluminum |
| E. Social & Policy Parameters |  |  |  |  |
| Employment Factor (Renewables) | --- | 8.5 | FTE/MW | Derived from [22], contextualized for Egypt |
| Social Weight - Job Creation | α₁ | 0.7 | --- | Expert weighting, prioritizing local economic development |
| Social Weight - Grid Independence | α₂ | 0.3 | --- | Expert weighting, aligned with energy security goals |
| F. Optimization Algorithm (NSGA-II) |  |  |  |  |
| Population Size | Npop | 100 | individuals | Standard for problem scale; ensures diversity |
| Maximum Generations | Ngen | 200 | --- | Ensures convergence based on pilot runs |
| Crossover Probability | pc | 0.9 | --- | NSGA-II default / literature [12, 13] |
| Mutation Probability | P m | 1/n | --- | where *n* is number of decision variables |

Table 2 : Complete LCA Parameters

| Life-Cycle Assessment (LCA) Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Manufacturing Stage |  |  |  |  |
| Emission Factor - PV Manufacturing | EF_M,PV | 80 | kg CO₂-eq/kW | Eco invent DB / Literature [23] |
| Emission Factor - Wind Turbine Manufacturing | EF_M,WT | 40 | kg CO₂-eq/kW | Eco invent DB / Literature [23] |
| Emission Factor - D-STATCOM Manufacturing | EF_M,DST | 25 | kg CO₂-eq/kVAr | Estimated from [23] for power electronics |
| Transportation Stage |  |  |  |  |
| Emission Factor - Road Transport | EFT,road | 0.12 | kg CO₂-eq/ton/km | Standard LCA database value |
| Average Transport Distance | Davg | 300 | km | Estimated Egypt-specific distance |
| Installation Stage |  |  |  |  |
| Emission Factor - Installation | EF_INS | 5 | kg CO₂-eq/kW | Construction emissions estimate [23] |
| Operation & Maintenance Stage |  |  |  |  |
| Emission Factor - Grid Electricity (Egypt) | EFgrid | 0.55 | kg CO₂-eq/kWh | Egyptian Electricity Holding Co. report |
| Annual O&M Energy Fraction | --- | 0.5 | % of generation | Standard assumption for renewables |
| Decommissioning & Recycling Stage |  |  |  |  |
| Emission Factor - Decommissioning | EF_DEC | 10 | kg CO₂-eq/kW | Standard LCA value for dismantling |
| Recycled Fraction (Metals) | R_REC | 0.85 | --- | Industry average for steel/aluminum |
| Avoided Emission Credit | --- | -15 | kg CO₂-eq/kW | Credit from recycling |