Appendix A. Model Features and Definitions

Within the nested cross-validation framework, features passing Benjamini–Hochberg false discovery rate correction were ranked using an ensemble of mutual information, the ANOVA F-statistic, and Gini impurity. A sensitivity analysis across candidate feature-set sizes showed that model performance plateaued at 19 features, which were therefore retained as the final predictors for model development. The final feature set comprised four inter-lead R-peak delay features and 15 nonlinear heart rate variability features.

A.1 Inter-lead R-wave delay features

Inter-lead R-wave delay was defined as the temporal separation between corresponding R-wave fiducial points identified in two simultaneously recorded ECG leads. It was used as an ECG-derived multilead timing biomarker to characterize the spatial and beat-to-beat organization of ventricular depolarization waveforms across different lead orientations.

All ECG leads were recorded simultaneously using a common sampling clock. R-wave fiducial points were detected independently in each lead using the same prespecified adaptive-threshold QRS detection procedure.

For monophasic, biphasic, multiphasic, and fragmented QRS complexes, the algorithm-defined dominant positive peak within the detected QRS region was consistently used as the R-wave fiducial point. This uniform rule ensured that the same morphology-dependent fiducial definition was applied across all leads and recordings.

For lead l, the detected R-wave fiducial sequence was defined as

|  | Rl=tl,1tl,2…tl,Nl, | (A1) |
| --- | --- | --- |

where Rldenotes the ordered set of R-wave fiducial times detected in lead l; tl,ndenotes the occurrence time of the n-th R-wave fiducial in lead l; ndenotes the within-lead beat index; and Nldenotes the total number of detected R-wave fiducials in lead l.

A.1.1 Cross-lead beat matching

For each ordered lead pair ab, every R-wave fiducial in lead awas matched to the temporally nearest R-wave fiducial in lead b. The matched fiducial index was determined as

|  | j*i;a,b=argmin1≤j≤Nbtb,j-ta,i, | (A2) |
| --- | --- | --- |

where j*iabdenotes the index of the R-wave fiducial in lead bthat is temporally nearest to the i-th R-wave fiducial in lead a; idenotes the R-wave fiducial index in lead a; jdenotes a candidate R-wave fiducial index in lead b; ta,iand tb,jdenote the corresponding R-wave fiducial times; and Nbdenotes the total number of detected R-wave fiducials in lead b.

A matched pair was retained when the absolute temporal separation between the two R-wave fiducials was less than 200 ms:

|  | tb,j*iab-ta,i<τ, τ=200 ms, | (A3) |
| --- | --- | --- |

where τdenotes the prespecified cross-lead matching window.

The temporal matching window ensured that paired fiducials represented the same cardiac cycle. R-wave fiducials without an eligible temporal counterpart were excluded from the delay calculation. This procedure limited the influence of missed detections, additional detections, and unmatched beats on the recording-level features.

A.1.2 Beat-level inter-lead R-wave delay

For the k-th accepted R-wave pair between leads aand b, the beat-level inter-lead R-wave delay was calculated as

|  | dab,k=tb,j*ik;a,b-ta,k, k=1,2,…,Mab, | (A4) |
| --- | --- | --- |

where dab,kdenotes the absolute inter-lead R-wave delay for the k-th accepted beat pair between leads aand b; ta,kand tb,kdenote the matched R-wave fiducial times in leads aand b, respectively; kdenotes the accepted beat-pair index; and Mabdenotes the total number of accepted R-wave pairs between leads aand b.

The absolute temporal difference was used to quantify the magnitude of the lead-to-lead timing separation independently of the displayed order of the two leads.

A.1.3 Recording-level summary features

The mean inter-lead R-wave delay was calculated as

|  | μab=1Mabk=1Mabdab,k, | (A5) |
| --- | --- | --- |

where μabdenotes the mean inter-lead R-wave delay between leads aand b; Mabdenotes the number of accepted beat pairs; dab,kdenotes the beat-level delay for the k-th accepted pair; and kdenotes the accepted beat-pair index.

The beat-to-beat variability of the inter-lead R-wave delay was calculated as

|  | σab=1Mabk=1Mabdab,k-μab2, | (A6) |
| --- | --- | --- |

where σabdenotes the population standard deviation of the inter-lead R-wave delay between leads aand b; Mabdenotes the number of accepted beat pairs; dab,kdenotes the beat-level delay for the k-th accepted pair; μabdenotes the corresponding mean delay; and kdenotes the accepted beat-pair index.

The four inter-lead R-wave delay features retained in the final model were therefore defined as

|  | I - V5 RDelay SD&=σI,V5,I - V1 RDelay SD&=σI,V1,I - II RDelay Mean&=μI,II,I - II RDelay SD&=σI,II. | (A7) |
| --- | --- | --- |

where I, II, V1, and V5 denote the corresponding standard ECG leads; RDelay denotes the absolute temporal separation between matched R-wave fiducials; Mean denotes the arithmetic mean defined in Eq. (A5); and SD denotes the population standard deviation defined in Eq. (A6).

Table 1 Inter-lead R-wave Delay Features

| Feature | Full Feature Name | Feature Explanation |
| --- | --- | --- |
| I-V5 R-peak Delay SD | Standard Deviation of R-peak Time Delay between Lead I and V5 | Quantifies beat-to-beat dispersion in the relative timing of R-wave fiducials between leads I and V5. |
| I-V1 R-peak Delay SD | Standard Deviation of R-peak Time Delay between Lead I and V1 | Quantifies beat-to-beat dispersion in the relative timing of R-wave fiducials between leads I and V1. |
| I-II R-peak Delay Mean | Mean R-peak Time Delay between Lead I and II | Quantifies the average absolute temporal separation between matched R-wave fiducials in leads I and II. |
| I-II R-peak Delay SD | Standard Deviation of R-peak Time Delay between Lead I and II | Quantifies the stability or beat-to-beat dispersion of the relative R-wave fiducial timing between leads I and II. |

A.1.4 Temporal resolution and measurement uncertainty

The ECG sampling frequency was 500 Hz. The temporal resolution corresponding to one sample was therefore

|  | Δt=1000fs=2 ms, | (A8) |
| --- | --- | --- |

where Δtdenotes the duration represented by one ECG sample in milliseconds, and fsdenotes the ECG sampling frequency in samples per second.

A one-sample variation in R-wave fiducial localization corresponded to a timing variation of 2 ms in an individual lead. Under a conservative condition in which the fiducial locations in both leads varied by one sample in opposite temporal directions, the resulting pairwise delay variation was up to 4 ms.

The observed RDelay SD values of approximately 40–50 ms represent the dispersion of the complete set of accepted beat-level inter-lead delays over the recording. These values describe beat-to-beat variation in the multilead timing relationship and do not represent the duration of a single conduction event. Their magnitude was substantially greater than the 2–4 ms sampling-grid uncertainty.

The accepted beat-level delays were constrained by the matching rule to the following range:

|  | 0≤dab,k<200 ms, | (A9) |
| --- | --- | --- |

where dab,kdenotes the absolute inter-lead R-wave delay for an accepted beat pair.

Uniform ECG acquisition, simultaneous lead recording, identical preprocessing, consistent fiducial localization, and the same cross-lead matching criteria were applied to all participants. These procedures supported consistent estimation of the inter-lead timing features across recordings.

A.1.5 Interpretation of the inter-lead R-wave delay features

The R-wave peak is a morphology-dependent surface-ECG fiducial associated with the projection of ventricular depolarization onto the axis of each ECG lead. The temporal relationship between R-wave fiducials in different leads therefore reflects an integrated multilead timing pattern involving the ventricular depolarization sequence, QRS morphology, lead orientation, and beat-to-beat electrical variation.

The RDelay Mean feature characterizes the average temporal separation between two lead-specific R-wave fiducials. The RDelay SD feature characterizes the stability of this temporal relationship across successive cardiac cycles. Higher RDelay SD values indicate greater beat-to-beat dispersion of the multilead R-wave timing pattern.

Accordingly, these features were interpreted as candidate ECG-derived multilead timing biomarkers for POD risk prediction. Their predictive contribution reflects information contained in the spatiotemporal organization of the surface ECG. Further electrophysiological studies may clarify the specific myocardial and autonomic mechanisms represented by these features

A.2 Nonlinear Heart Rate Variability (HRV) Features

Nonlinear HRV features characterize the regulatory capacity of the autonomic nervous system over cardiac rhythm from multiple perspectives, including signal complexity, fractal properties, and symbolic dynamics.V5 HRV SampEn

Table 2 Nonlinear HRV Features

| Feature | Full Feature Name | Feature Explanation |
| --- | --- | --- |
| V5 HRV SampEn | Sample Entropy of RR Intervals (V5) | Quantifies the complexity and unpredictability of the heart rate time series in lead V5. Higher values indicate greater complexity in autonomic regulation. |
| V1 HRV SampEn | Sample Entropy of RR Intervals (V1) | Measures the irregularity of RR intervals in lead V1. Higher entropy suggests richer autonomic modulation. |
| V5 HRV KFD | Katz Fractal Dimension of RR Intervals (V5) | Evaluates the fractal complexity and self-similarity of heart rate dynamics in lead V5. Higher values indicate more complex regulation. |
| V5 HRV MFDFA | Multifractal Detrended Fluctuation Analysis Index of RR Intervals (V5) | Captures the multifractal scaling properties of heart rate dynamics in lead V5, reflecting complex, scale-dependent autonomic modulation. |
| V5 HRV PIP | Porta's Index of Parasympathetic Tone (V5) | A symbolic-dynamics measure of vagal modulation in lead V5. Higher values suggest stronger parasympathetic influence. |
| V1 HRV PIP | Porta's Index of Parasympathetic Tone (V1) | A symbolic-dynamics-based index for assessing parasympathetic (vagal) cardiac modulation in lead V1. |
| V5 HRV IALS | Index of Acceleration and Deceleration of Heart Rate (V5) | Describes the asymmetry between heart rate accelerations and decelerations in lead V5, which captures instability in autonomic control. |
| V1 HRV IALS | Inverse of the Average Length of Acceleration/Deceleration Segments (V1) | Reflects the symmetry characteristics of heart rate changes (V1 lead). |
| V1 HRV SD1SD2 | Ratio of SD1 to SD2(V1) | Represents the relative contribution of short-term (SD1) versus long-term (SD2) variability in lead V1. Higher ratios indicate relatively enhanced short-term fluctuations. |
| V1 HRV CSI | Cardiac Sympathetic Index (V1) | A nonlinear Poincaré-plot-derived indicator of sympathetic modulation in lead V1. Elevated values are associated with sympathetic predominance. |
| V5 HRV CSI | Cardiac Sympathetic Index (V5) | A nonlinear index quantifying sympathetic dominance in lead V5. Higher values reflect increased sympathetic drive. |
| V5 HRV PSS | Porta's Symbolic Slope (V5) | Analyzes RR-interval patterns based on symbolic dynamics in lead V5 to characterize directional trends in heart rate sequences. |
| V5 HRV PAS | Porta's Acceleration Slope (V5) | Characterizes the trend and intensity of heart rate accelerations in lead V5, reflecting short-term sympathetic-dominant changes. |
| V1 HRV PAS | Porta's Acceleration Slope (V1) | Characterizes the tendency and magnitude of heart rate accelerations in lead V1, associated with fast autonomic responses. |
| V1 HRV PI | Porta's Index of Heart Rate Asymmetry (V1) | Short-term heart-rate asymmetry in lead V1, quantified as the percentage of Poincaré points below the identity line. |

Table 3 Machine learning model performance

| Model | AUC | Accuracy, % | Sensitivity, % | Precision, % | F1-score, % |
| --- | --- | --- | --- | --- | --- |
| XGBoost | 0.960 (0.920-0.980) | 87.9 (81.7-92.2) | 88.2 (73.4-95.3) | 68.2 (53.4-80.0) | 76.9 (67.5-85.7) |
| RF | 0.966 (0.930-0.991) | 91.9 (86.5-95.3) | 85.3 (69.9-93.6) | 80.6 (65.0-90.2) | 82.9 (73.3-91.4) |
| ET | 0.950 (0.910-0.980) | 86.6 (80.2-91.1) | 79.4 (63.2-89.7) | 67.5 (52.0-79.9) | 73.0 (62.2-83.3) |
| GBM | 0.970 (0.940-0.990) | 89.3 (83.3-93.3) | 79.4 (63.2-89.7) | 75.0 (58.9-86.2) | 77.1 (66.7-87.0) |
| KNN | 0.910 (0.850-0.960) | 79.2 (72.0-84.9) | 82.4 (66.5-91.7) | 52.8 (39.7-65.6) | 64.4 (54.9-74.1) |
| LR | 0.770 (0.670-0.840) | 72.5 (64.8-79.0) | 73.5 (56.9-85.4) | 43.9 (31.8-56.7) | 54.9 (44.9-64.4) |