# Supplementary Information

Perceived Indoor Environmental Quality and Facility Satisfaction Across Four University Housing Facilities in Taiz, Yemen

Ghanima A. M. Al-Alali1,  Yaser Gamil2,  Saleem Ahmed Al-Azazi3,4*

1  Faculty of Engineering and Information Technology, Taiz University, Taiz 6803, Yemen;

ghghss3434@gmail.com

2 Department of Civil Engineering, School of Engineering, Monash University, Malaysia;

yaser.gamil@monash.edu

3 Faculty of Engineering and Information Technology, Taiz University, Taiz 6803, Yemen;

saleem.alazazi@taiz.edu.ye

4 Department of Biomedical Equipment Engineering, Faculty of Engineering, Al Rowad University, Taiz, Yemen

Corresponding author: Saleem Ahmed Al-Azazi; email: saleem.alazazi@taiz.edu.ye

This file reports complete supporting methods and results. It does not convert the study-specific questionnaire into a validated scale and does not support causal or institutional-ranking claims.

# S1 Data provenance and analytic populations

Resident sources contributed 138 and 115 complete records. Primary N=253; sensitivity N=235 after exclusion of 18 prespecified response-profile flags. No missing item values or exact duplicate analytic rows were found. Recruitment denominators were unavailable. Archived institutional facility codes were retained as Ofok Foundation Housing (code 1; N=76), Taiz University Housing (code 2; N=56), World Assembly Housing (code 3; N=102) and Al-Janad University Housing (code 4; N=19). Ofok Foundation has separate men’s and women’s physical buildings, but the analytic file does not contain a participant-level branch variable that can split its 76 responses reliably. No site code was inferred or reassigned from participant sex.

## Supplementary Table S1  Site metadata

| site_code | site | latitude | longitude | resident_n | construction_year | stories | structural_system | walls | cooling | ventilation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ofok Foundation Housing | Men 13.583657; Women 13.577823 | Men 43.967018; Women 43.982780 | 76 | 2009 | Men 4; Women 5 | Reinforced concrete frame | Hollow cement block masonry | Natural cooling through windows | Natural ventilation through window openings |
| 2 | Taiz University Housing | 13.578847 | 43.976776 | 56 | 2017 | 5 | Reinforced concrete frame | Hollow cement block masonry | Natural cooling through windows | Natural ventilation through window openings |
| 3 | World Assembly Housing | Not retained | Not retained | 102 | Not retained | Not retained | Not verified in retained site notes | Not verified in retained site notes | Not verified in retained site notes | Not verified in retained site notes |
| 4 | Al-Janad University Housing | 13.585086 | 44.006655 | 19 | 2000 | 4 | Reinforced concrete frame | Hollow cement block masonry | Natural cooling through windows | Natural ventilation through window openings |

# S2 Questionnaire dimensions

## Supplementary Table S2  Dimension summaries and consistency

| code | construct | items | N | mean | SD | median | IQR | alpha | omega |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC | Thermal comfort | 4 | 253 | 3.179 | 0.742 | 3.250 | 1.000 | 0.663 | 0.675 |
| VC | Visual comfort | 5 | 253 | 3.638 | 0.825 | 3.800 | 1.000 | 0.795 | 0.797 |
| AC | Acoustic comfort | 4 | 253 | 3.297 | 0.870 | 3.500 | 1.250 | 0.739 | 0.758 |
| IAQ | Indoor air quality/ventilation | 4 | 253 | 3.206 | 0.880 | 3.250 | 1.250 | 0.738 | 0.774 |
| FLS | Fire and life safety | 4 | 253 | 3.093 | 1.034 | 3.250 | 1.500 | 0.840 | 0.854 |
| RF | Room functionality | 4 | 253 | 3.602 | 0.842 | 3.750 | 1.000 | 0.770 | 0.777 |
| BC | Building condition and finishes | 7 | 253 | 3.564 | 0.798 | 3.571 | 1.000 | 0.855 | 0.861 |
| HS | Housing and utility services | 5 | 253 | 3.093 | 0.871 | 3.200 | 1.400 | 0.747 | 0.759 |
| MA | Mobility and accessibility | 6 | 253 | 3.686 | 0.724 | 3.667 | 0.667 | 0.754 | 0.753 |

## Supplementary Table S3  Complete resident item statistics

| item | label | N | mean | SD | median | dissatisfied_pct | neutral_pct | satisfied_pct |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Room temperature during summer | 253 | 2.213 | 1.114 | 2.000 | 66.8 | 14.2 | 19.0 |
| 2 | Room temperature during winter | 253 | 3.779 | 1.027 | 4.000 | 11.5 | 17.4 | 71.1 |
| 3 | Temperature in common areas | 253 | 3.277 | 0.965 | 3.000 | 20.6 | 30.4 | 49.0 |
| 4 | Fresh air from room windows | 253 | 3.447 | 1.096 | 4.000 | 19.8 | 23.3 | 56.9 |
| 5 | Natural daylight in room | 253 | 3.810 | 1.085 | 4.000 | 15.4 | 9.5 | 75.1 |
| 6 | Natural daylight in corridors | 253 | 3.700 | 1.034 | 4.000 | 15.4 | 13.4 | 71.1 |
| 7 | Artificial lighting in room at night | 253 | 3.585 | 1.214 | 4.000 | 22.1 | 15.0 | 62.8 |
| 8 | Artificial lighting in corridors/stairs | 253 | 3.577 | 1.140 | 4.000 | 20.9 | 14.2 | 64.8 |
| 9 | Control over room lighting | 253 | 3.518 | 1.079 | 4.000 | 17.8 | 22.5 | 59.7 |
| 10 | External-noise insulation | 253 | 2.992 | 1.291 | 3.000 | 38.3 | 21.7 | 39.9 |
| 11 | Noise insulation from adjacent rooms | 253 | 3.494 | 1.101 | 4.000 | 20.9 | 17.8 | 61.3 |
| 12 | Noise insulation from corridors/common areas | 253 | 3.375 | 1.094 | 4.000 | 22.1 | 26.1 | 51.8 |
| 13 | Noise insulation from building systems | 253 | 3.328 | 1.151 | 3.000 | 22.9 | 29.6 | 47.4 |
| 14 | Quality of natural ventilation in room | 253 | 3.458 | 1.166 | 4.000 | 23.3 | 18.2 | 58.5 |
| 15 | Control over natural ventilation in room | 253 | 3.336 | 1.155 | 4.000 | 23.3 | 24.5 | 52.2 |
| 16 | Ventilation quality in common areas | 253 | 3.585 | 1.038 | 4.000 | 17.8 | 19.0 | 63.2 |
| 17 | Control over mechanical ventilation in room | 253 | 2.443 | 1.328 | 2.000 | 56.5 | 17.8 | 25.7 |
| 18 | Identification of evacuation exits/routes | 253 | 3.123 | 1.233 | 3.000 | 28.1 | 26.5 | 45.5 |
| 19 | Clarity of emergency-exit signage | 253 | 3.146 | 1.224 | 3.000 | 29.2 | 25.7 | 45.1 |
| 20 | Emergency-corridor lighting | 253 | 3.123 | 1.181 | 3.000 | 28.1 | 28.5 | 43.5 |
| 21 | Alarm systems and fire extinguishers | 253 | 2.980 | 1.384 | 3.000 | 39.5 | 18.2 | 42.3 |
| 22 | Room-space layout | 253 | 3.609 | 1.092 | 4.000 | 18.2 | 14.2 | 67.6 |
| 23 | Visual and acoustic privacy | 253 | 3.427 | 1.127 | 4.000 | 19.8 | 22.9 | 57.3 |
| 24 | Ease of furniture arrangement | 253 | 3.834 | 1.033 | 4.000 | 12.3 | 11.9 | 75.9 |
| 25 | Location and adequacy of room windows | 253 | 3.538 | 1.125 | 4.000 | 18.2 | 23.3 | 58.5 |
| 26 | Room wall/paint finishes | 253 | 3.700 | 1.075 | 4.000 | 15.0 | 16.6 | 68.4 |
| 27 | Room floors and ceilings | 253 | 3.846 | 1.075 | 4.000 | 13.4 | 13.0 | 73.5 |
| 28 | Common-area wall/paint finishes | 253 | 3.692 | 1.080 | 4.000 | 16.2 | 17.8 | 66.0 |
| 29 | Common-area floors and ceilings | 253 | 3.723 | 1.040 | 4.000 | 14.6 | 17.0 | 68.4 |
| 30 | External landscaping quality | 253 | 2.917 | 1.230 | 3.000 | 37.9 | 23.3 | 38.7 |
| 31 | Condition of windows | 253 | 3.518 | 1.086 | 4.000 | 18.6 | 20.9 | 60.5 |
| 32 | Condition of doors | 253 | 3.553 | 1.044 | 4.000 | 17.8 | 21.3 | 60.9 |
| 33 | Continuity of electricity supply | 253 | 3.332 | 1.339 | 4.000 | 30.4 | 12.3 | 57.3 |
| 34 | Continuity of water supply | 253 | 2.787 | 1.298 | 3.000 | 45.8 | 20.2 | 34.0 |
| 35 | Condition/readiness of bathrooms and kitchens | 253 | 3.372 | 1.139 | 4.000 | 24.1 | 20.9 | 54.9 |
| 36 | Adequacy of shared facilities | 253 | 3.451 | 1.081 | 4.000 | 20.2 | 22.5 | 57.3 |
| 37 | Adequacy of recreational facilities | 253 | 2.526 | 1.296 | 2.000 | 51.4 | 22.9 | 25.7 |
| 38 | Ease of movement between building areas | 253 | 3.917 | 0.962 | 4.000 | 9.1 | 15.0 | 75.9 |
| 39 | Ease of movement in corridors/entrances | 253 | 4.028 | 0.936 | 4.000 | 7.5 | 11.5 | 81.0 |
| 40 | Adequacy of stairs/lifts | 253 | 3.751 | 1.053 | 4.000 | 13.8 | 17.4 | 68.8 |
| 41 | Accessibility for persons with disabilities | 253 | 3.051 | 1.279 | 3.000 | 36.8 | 22.1 | 41.1 |
| 42 | Location relative to lecture halls | 253 | 3.549 | 1.180 | 4.000 | 19.4 | 20.2 | 60.5 |
| 43 | Internal wayfinding signage | 253 | 3.818 | 1.031 | 4.000 | 10.3 | 18.6 | 71.1 |

# S3 Measurement-model audit

The primary ordinal model used lavaan WLSMV with ordered indicators and the explicitly specified delta parameterization. Primary fit was CFI=0.879, TLI=0.867, RMSEA=0.069 and SRMR=0.081; the N=235 sensitivity fit was similar. Maximum-likelihood and independently coded ordinal-DWLS results are retained as complementary audits. Across methods, the prespecified structure did not justify definitive scale validation.

## Supplementary Table S4  Full ordinal WLSMV fit

| analysis | chisq.scaled | df.scaled | pvalue.scaled | cfi.scaled | tli.scaled | rmsea.scaled | rmsea.ci.lower.scaled | rmsea.ci.upper.scaled | srmr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Primary_N253 | 1805.573 | 824.000 | 0.000 | 0.879 | 0.867 | 0.069 | 0.064 | 0.073 | 0.081 |
| Sensitivity_N235 | 1850.761 | 824.000 | 0.000 | 0.875 | 0.863 | 0.073 | 0.069 | 0.077 | 0.081 |

## Supplementary Table S5  Maximum-likelihood measurement audit

| analysis | model | chi2 | df | p | CFI | TLI | RMSEA | SRMR | converged |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Primary N=253 | item_full | 1958.019 | 824 | 0.000 | 0.780 | 0.759 | 0.074 | 0.093 | True |
| Primary N=253 | item_ieq | 495.284 | 179 | 0.000 | 0.854 | 0.829 | 0.084 | 0.085 | True |
| Primary N=253 | item_pffs | 658.972 | 203 | 0.000 | 0.811 | 0.785 | 0.094 | 0.097 | True |
| Primary N=253 | dimension | 65.108 | 26 | 0.000 | 0.959 | 0.943 | 0.077 | 0.047 | True |
| Sensitivity N=235 | item_full | 1884.648 | 824 | 0.000 | 0.795 | 0.776 | 0.074 | 0.092 | True |
| Sensitivity N=235 | item_ieq | 484.411 | 179 | 0.000 | 0.858 | 0.834 | 0.085 | 0.089 | True |
| Sensitivity N=235 | item_pffs | 587.324 | 203 | 0.000 | 0.844 | 0.823 | 0.090 | 0.093 | True |
| Sensitivity N=235 | dimension | 47.078 | 26 | 0.007 | 0.977 | 0.969 | 0.059 | 0.039 | True |

# S4 Alternative dimensionality and item analyses

## Supplementary Table S6  Parallel analysis

| block | component | observed_eigenvalue | 95th_percentile_null | retained | total_retained |
| --- | --- | --- | --- | --- | --- |
| Sensory IEQ | 1 | 5.800 | 1.579 | 1 | 2 |
| Sensory IEQ | 2 | 1.771 | 1.446 | 1 | 2 |
| Sensory IEQ | 3 | 1.333 | 1.361 | 0 | 2 |
| Sensory IEQ | 4 | 1.208 | 1.290 | 0 | 2 |
| Sensory IEQ | 5 | 0.913 | 1.225 | 0 | 2 |
| Sensory IEQ | 6 | 0.857 | 1.168 | 0 | 2 |
| Sensory IEQ | 7 | 0.721 | 1.113 | 0 | 2 |
| Sensory IEQ | 8 | 0.708 | 1.064 | 0 | 2 |
| Sensory IEQ | 9 | 0.638 | 1.017 | 0 | 2 |
| Sensory IEQ | 10 | 0.603 | 0.973 | 0 | 2 |
| Sensory IEQ | 11 | 0.537 | 0.927 | 0 | 2 |
| Sensory IEQ | 12 | 0.453 | 0.886 | 0 | 2 |
| Sensory IEQ | 13 | 0.397 | 0.845 | 0 | 2 |
| Sensory IEQ | 14 | 0.347 | 0.799 | 0 | 2 |
| Sensory IEQ | 15 | 0.280 | 0.752 | 0 | 2 |
| Sensory IEQ | 16 | 0.253 | 0.706 | 0 | 2 |
| Sensory IEQ | 17 | 0.182 | 0.655 | 0 | 2 |
| PFFS | 1 | 7.644 | 1.672 | 1 | 3 |
| PFFS | 2 | 2.122 | 1.541 | 1 | 3 |
| PFFS | 3 | 1.585 | 1.452 | 1 | 3 |
| PFFS | 4 | 1.273 | 1.381 | 0 | 3 |
| PFFS | 5 | 1.099 | 1.321 | 0 | 3 |
| PFFS | 6 | 1.019 | 1.266 | 0 | 3 |
| PFFS | 7 | 0.875 | 1.212 | 0 | 3 |
| PFFS | 8 | 0.722 | 1.161 | 0 | 3 |
| PFFS | 9 | 0.636 | 1.113 | 0 | 3 |
| PFFS | 10 | 0.617 | 1.073 | 0 | 3 |
| PFFS | 11 | 0.594 | 1.031 | 0 | 3 |
| PFFS | 12 | 0.540 | 0.987 | 0 | 3 |
| PFFS | 13 | 0.503 | 0.950 | 0 | 3 |
| PFFS | 14 | 0.422 | 0.911 | 0 | 3 |
| PFFS | 15 | 0.394 | 0.870 | 0 | 3 |
| PFFS | 16 | 0.360 | 0.834 | 0 | 3 |
| PFFS | 17 | 0.340 | 0.795 | 0 | 3 |
| PFFS | 18 | 0.295 | 0.757 | 0 | 3 |
| PFFS | 19 | 0.283 | 0.720 | 0 | 3 |
| PFFS | 20 | 0.274 | 0.679 | 0 | 3 |
| PFFS | 21 | 0.222 | 0.639 | 0 | 3 |
| PFFS | 22 | 0.181 | 0.588 | 0 | 3 |

## Supplementary Table S7  PCA-based adjusted model

| term | beta | robust_SE | CI_low | CI_high | p |
| --- | --- | --- | --- | --- | --- |
| Intercept | -0.4949 | 0.2225 | -0.931 | -0.05882 | 0.02613 |
| Sensory item PC1 | 0.598 | 0.06229 | 0.4759 | 0.7201 | 7.934e-22 |
| Fire and life safety | 0.2156 | 0.0561 | 0.1056 | 0.3255 | 0.0001214 |
| Female | 0.5322 | 0.239 | 0.06376 | 1.001 | 0.02596 |
| Academic level | -0.05227 | 0.06786 | -0.1853 | 0.08074 | 0.4412 |
| Residence duration | 0.07818 | 0.06612 | -0.0514 | 0.2078 | 0.237 |
| Coded batch | 0.07105 | 0.09757 | -0.1202 | 0.2623 | 0.4665 |
| Site 2 | -0.156 | 0.1249 | -0.4008 | 0.08879 | 0.2116 |
| Site 3 | 0.5266 | 0.2417 | 0.05299 | 1 | 0.02931 |
| Site 4 | 0.5439 | 0.2552 | 0.04371 | 1.044 | 0.03307 |

## Supplementary Table S8  Item-level adjusted associations with Holm correction

| item | beta | robust_SE | CI_low | CI_high | p_raw | p_Holm |
| --- | --- | --- | --- | --- | --- | --- |
| Q01 | 0.4512 | 0.0557 | 0.342 | 0.5604 | 5.509e-16 | 9.917e-15 |
| Q02 | 0.2609 | 0.07299 | 0.1178 | 0.4039 | 0.0003516 | 0.0003516 |
| Q03 | 0.2949 | 0.0656 | 0.1664 | 0.4235 | 6.928e-06 | 2.075e-05 |
| Q04 | 0.5003 | 0.05976 | 0.3831 | 0.6174 | 5.669e-17 | 1.134e-15 |
| Q05 | 0.3212 | 0.07144 | 0.1812 | 0.4613 | 6.917e-06 | 2.075e-05 |
| Q06 | 0.3621 | 0.06985 | 0.2252 | 0.499 | 2.174e-07 | 1.304e-06 |
| Q07 | 0.391 | 0.07232 | 0.2493 | 0.5327 | 6.423e-08 | 5.138e-07 |
| Q08 | 0.3809 | 0.07004 | 0.2436 | 0.5182 | 5.366e-08 | 4.829e-07 |
| Q09 | 0.4553 | 0.06627 | 0.3254 | 0.5852 | 6.426e-12 | 8.354e-11 |
| Q10 | 0.3207 | 0.06267 | 0.1979 | 0.4435 | 3.097e-07 | 1.549e-06 |
| Q11 | 0.4376 | 0.06175 | 0.3165 | 0.5586 | 1.385e-12 | 2.216e-11 |
| Q12 | 0.3879 | 0.06639 | 0.2577 | 0.518 | 5.154e-09 | 5.154e-08 |
| Q13 | 0.3559 | 0.07369 | 0.2114 | 0.5003 | 1.37e-06 | 5.481e-06 |
| Q14 | 0.5361 | 0.06367 | 0.4113 | 0.6609 | 3.747e-17 | 7.868e-16 |
| Q15 | 0.5318 | 0.065 | 0.4044 | 0.6592 | 2.8e-16 | 5.321e-15 |
| Q16 | 0.4437 | 0.06372 | 0.3188 | 0.5686 | 3.333e-12 | 4.999e-11 |
| Q17 | 0.4316 | 0.05623 | 0.3214 | 0.5418 | 1.644e-14 | 2.794e-13 |
| Q18 | 0.4023 | 0.0638 | 0.2772 | 0.5273 | 2.878e-10 | 3.166e-09 |
| Q19 | 0.4425 | 0.06363 | 0.3178 | 0.5672 | 3.523e-12 | 4.999e-11 |
| Q20 | 0.431 | 0.06498 | 0.3036 | 0.5583 | 3.31e-11 | 3.973e-10 |
| Q21 | 0.3632 | 0.06807 | 0.2297 | 0.4966 | 9.565e-08 | 6.696e-07 |

# S5 Association-model details

## Supplementary Table S9a  Primary N=253

| term | beta | robust_SE | CI_low | CI_high | p |
| --- | --- | --- | --- | --- | --- |
| Intercept | -0.4459 | 0.2224 | -0.8818 | -0.01001 | 0.04497 |
| Sensory IEQ | 0.613 | 0.05974 | 0.4959 | 0.7301 | 1.049e-24 |
| Fire and life safety | 0.2107 | 0.05636 | 0.1002 | 0.3212 | 0.0001851 |
| Female | 0.4962 | 0.2386 | 0.02857 | 0.9639 | 0.03755 |
| Academic level | -0.07782 | 0.067 | -0.2091 | 0.05349 | 0.2454 |
| Residence duration | 0.08952 | 0.0667 | -0.04121 | 0.2203 | 0.1796 |
| Coded batch | 0.05417 | 0.09667 | -0.1353 | 0.2436 | 0.5752 |
| Site 2 | -0.1147 | 0.1248 | -0.3592 | 0.1299 | 0.3581 |
| Site 3 | 0.4616 | 0.2412 | -0.01107 | 0.9342 | 0.05561 |
| Site 4 | 0.4401 | 0.2547 | -0.05911 | 0.9393 | 0.08401 |

## Supplementary Table S9b  Sensitivity N=235

| term | beta | robust_SE | CI_low | CI_high | p |
| --- | --- | --- | --- | --- | --- |
| Intercept | -0.4017 | 0.2611 | -0.9135 | 0.1101 | 0.1239 |
| Sensory IEQ | 0.635 | 0.06358 | 0.5103 | 0.7596 | 1.734e-23 |
| Fire and life safety | 0.1895 | 0.05716 | 0.07752 | 0.3016 | 0.0009127 |
| Female | 0.4512 | 0.2745 | -0.08681 | 0.9892 | 0.1002 |
| Academic level | -0.06399 | 0.06691 | -0.1951 | 0.06716 | 0.3389 |
| Residence duration | 0.08707 | 0.06825 | -0.0467 | 0.2208 | 0.2021 |
| Coded batch | 0.09132 | 0.09948 | -0.1037 | 0.2863 | 0.3586 |
| Site 2 | -0.1378 | 0.1256 | -0.3841 | 0.1084 | 0.2726 |
| Site 3 | 0.3827 | 0.2769 | -0.16 | 0.9254 | 0.1669 |
| Site 4 | 0.3302 | 0.2921 | -0.2423 | 0.9027 | 0.2583 |

## Supplementary Table S9c  Dimension model N=253

| term | beta | robust_SE | CI_low | CI_high | p |
| --- | --- | --- | --- | --- | --- |
| Intercept | -0.4318 | 0.2108 | -0.845 | -0.01855 | 0.04056 |
| Thermal comfort | 0.1882 | 0.05511 | 0.08017 | 0.2962 | 0.0006388 |
| Visual comfort | 0.08929 | 0.06962 | -0.04715 | 0.2257 | 0.1996 |
| Acoustic comfort | 0.175 | 0.06422 | 0.04911 | 0.3008 | 0.006437 |
| Indoor air quality and ventilation | 0.316 | 0.08604 | 0.1474 | 0.4846 | 0.0002397 |
| Fire and life safety | 0.2141 | 0.05582 | 0.1047 | 0.3235 | 0.0001252 |
| Female | 0.4513 | 0.2279 | 0.004606 | 0.898 | 0.04768 |
| Academic level | -0.08826 | 0.06801 | -0.2216 | 0.04503 | 0.1943 |
| Residence duration | 0.1008 | 0.06864 | -0.03375 | 0.2353 | 0.142 |
| Coded batch | 0.0789 | 0.1001 | -0.1174 | 0.2752 | 0.4307 |
| Site 2 | -0.1084 | 0.1233 | -0.3501 | 0.1332 | 0.3791 |
| Site 3 | 0.442 | 0.2327 | -0.014 | 0.8981 | 0.05746 |
| Site 4 | 0.4629 | 0.245 | -0.01732 | 0.9431 | 0.05885 |

## Supplementary Table S9d  Dimension model sensitivity N=235

| term | beta | robust_SE | CI_low | CI_high | p |
| --- | --- | --- | --- | --- | --- |
| Intercept | -0.4068 | 0.2549 | -0.9065 | 0.09292 | 0.1106 |
| Thermal comfort | 0.1843 | 0.05576 | 0.07504 | 0.2936 | 0.0009469 |
| Visual comfort | 0.1224 | 0.07144 | -0.01762 | 0.2624 | 0.08666 |
| Acoustic comfort | 0.1921 | 0.06214 | 0.07033 | 0.3139 | 0.001989 |
| Indoor air quality and ventilation | 0.2975 | 0.08146 | 0.1378 | 0.4571 | 0.0002602 |
| Fire and life safety | 0.1859 | 0.05743 | 0.07333 | 0.2984 | 0.001208 |
| Female | 0.4342 | 0.2688 | -0.09266 | 0.9611 | 0.1062 |
| Academic level | -0.07455 | 0.06782 | -0.2075 | 0.05838 | 0.2717 |
| Residence duration | 0.09738 | 0.07014 | -0.04009 | 0.2348 | 0.165 |
| Coded batch | 0.1106 | 0.1025 | -0.09034 | 0.3115 | 0.2808 |
| Site 2 | -0.1368 | 0.1273 | -0.3863 | 0.1127 | 0.2825 |
| Site 3 | 0.3859 | 0.2722 | -0.1477 | 0.9195 | 0.1563 |
| Site 4 | 0.3637 | 0.2864 | -0.1976 | 0.925 | 0.2041 |

# S6 Facility comparisons

## Supplementary Table S10  Omnibus facility comparisons

| construct | label | H | p | epsilon_squared | Ofok Foundation | Taiz University | World Assembly | Al-Janad University |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC | Thermal comfort | 3.626 | 0.3047 | 0.002515 | 3.224 | 3.009 | 3.238 | 3.184 |
| VC | Visual comfort | 8.426 | 0.03798 | 0.02179 | 3.724 | 3.343 | 3.7 | 3.832 |
| AC | Acoustic comfort | 1.747 | 0.6265 | 0 | 3.385 | 3.192 | 3.275 | 3.382 |
| IAQ | Indoor air quality and ventilation | 5.796 | 0.122 | 0.01123 | 3.378 | 3.067 | 3.162 | 3.158 |
| FLS | Fire and life safety | 24.4 | 2.058e-05 | 0.08596 | 3.539 | 2.929 | 2.804 | 3.342 |
| RF | Room functionality | 10.62 | 0.01398 | 0.0306 | 3.816 | 3.379 | 3.571 | 3.566 |
| BC | Building condition and finishes | 17.33 | 0.0006046 | 0.05755 | 3.579 | 3.258 | 3.637 | 4.015 |
| HS | Housing and utility services | 2.455 | 0.4836 | 0 | 3.158 | 2.982 | 3.133 | 2.947 |
| MA | Mobility and accessibility | 5.069 | 0.1668 | 0.008309 | 3.822 | 3.64 | 3.624 | 3.605 |

## Supplementary Table S11  Pairwise facility comparisons with Holm adjustment

| construct | site_A | site_B | U | p_raw | p_Holm |
| --- | --- | --- | --- | --- | --- |
| TC | Ofok Foundation Housing | Taiz University Housing | 2438 | 0.1527 | 0.7634 |
| TC | Ofok Foundation Housing | World Assembly Housing | 3778 | 0.7732 | 1 |
| TC | Ofok Foundation Housing | Al-Janad University Housing | 738 | 0.8847 | 1 |
| TC | Taiz University Housing | World Assembly Housing | 2338 | 0.05849 | 0.351 |
| TC | Taiz University Housing | Al-Janad University Housing | 466 | 0.4222 | 1 |
| TC | World Assembly Housing | Al-Janad University Housing | 1014 | 0.7525 | 1 |
| VC | Ofok Foundation Housing | Taiz University Housing | 2642 | 0.01748 | 0.08738 |
| VC | Ofok Foundation Housing | World Assembly Housing | 3846 | 0.9306 | 1 |
| VC | Ofok Foundation Housing | Al-Janad University Housing | 683 | 0.7187 | 1 |
| VC | Taiz University Housing | World Assembly Housing | 2154 | 0.01049 | 0.06293 |
| VC | Taiz University Housing | Al-Janad University Housing | 365 | 0.04149 | 0.166 |
| VC | World Assembly Housing | Al-Janad University Housing | 922.5 | 0.742 | 1 |
| AC | Ofok Foundation Housing | Taiz University Housing | 2410 | 0.1937 | 1 |
| AC | Ofok Foundation Housing | World Assembly Housing | 4132 | 0.4507 | 1 |
| AC | Ofok Foundation Housing | Al-Janad University Housing | 746.5 | 0.8225 | 1 |
| AC | Taiz University Housing | World Assembly Housing | 2698 | 0.5666 | 1 |
| AC | Taiz University Housing | Al-Janad University Housing | 468.5 | 0.4401 | 1 |
| AC | World Assembly Housing | Al-Janad University Housing | 920.5 | 0.7312 | 1 |
| IAQ | Ofok Foundation Housing | Taiz University Housing | 2648 | 0.01618 | 0.09706 |
| IAQ | Ofok Foundation Housing | World Assembly Housing | 4442 | 0.09537 | 0.4768 |
| IAQ | Ofok Foundation Housing | Al-Janad University Housing | 852 | 0.2262 | 0.9048 |
| IAQ | Taiz University Housing | World Assembly Housing | 2677 | 0.5147 | 1 |
| IAQ | Taiz University Housing | Al-Janad University Housing | 531 | 0.9951 | 1 |
| IAQ | World Assembly Housing | Al-Janad University Housing | 1000 | 0.8274 | 1 |
| FLS | Ofok Foundation Housing | Taiz University Housing | 2959 | 0.0001187 | 0.0005934 |
| FLS | Ofok Foundation Housing | World Assembly Housing | 5360 | 1.187e-05 | 7.122e-05 |
| FLS | Ofok Foundation Housing | Al-Janad University Housing | 804 | 0.4444 | 0.8889 |
| FLS | Taiz University Housing | World Assembly Housing | 3054 | 0.4713 | 0.8889 |
| FLS | Taiz University Housing | Al-Janad University Housing | 392.5 | 0.08862 | 0.2659 |
| FLS | World Assembly Housing | Al-Janad University Housing | 694 | 0.04989 | 0.1995 |
| RF | Ofok Foundation Housing | Taiz University Housing | 2828 | 0.001145 | 0.00687 |
| RF | Ofok Foundation Housing | World Assembly Housing | 4529 | 0.05329 | 0.2664 |
| RF | Ofok Foundation Housing | Al-Janad University Housing | 833 | 0.2997 | 0.7283 |
| RF | Taiz University Housing | World Assembly Housing | 2414 | 0.106 | 0.4238 |
| RF | Taiz University Housing | Al-Janad University Housing | 436.5 | 0.2428 | 0.7283 |
| RF | World Assembly Housing | Al-Janad University Housing | 964 | 0.9743 | 0.9743 |
| BC | Ofok Foundation Housing | Taiz University Housing | 2656 | 0.01485 | 0.05942 |
| BC | Ofok Foundation Housing | World Assembly Housing | 3668 | 0.5419 | 0.5419 |
| BC | Ofok Foundation Housing | Al-Janad University Housing | 488 | 0.02919 | 0.08756 |
| BC | Taiz University Housing | World Assembly Housing | 1997 | 0.001765 | 0.008825 |
| BC | Taiz University Housing | Al-Janad University Housing | 211.5 | 9.014e-05 | 0.0005408 |
| BC | World Assembly Housing | Al-Janad University Housing | 724.5 | 0.08149 | 0.163 |
| HS | Ofok Foundation Housing | Taiz University Housing | 2382 | 0.241 | 1 |
| HS | Ofok Foundation Housing | World Assembly Housing | 3952 | 0.825 | 1 |
| HS | Ofok Foundation Housing | Al-Janad University Housing | 834 | 0.2982 | 1 |
| HS | Taiz University Housing | World Assembly Housing | 2570 | 0.2982 | 1 |
| HS | Taiz University Housing | Al-Janad University Housing | 560.5 | 0.7321 | 1 |
| HS | World Assembly Housing | Al-Janad University Housing | 1122 | 0.276 | 1 |
| MA | Ofok Foundation Housing | Taiz University Housing | 2554 | 0.0491 | 0.2946 |
| MA | Ofok Foundation Housing | World Assembly Housing | 4445 | 0.09352 | 0.4676 |
| MA | Ofok Foundation Housing | Al-Janad University Housing | 881.5 | 0.1372 | 0.5487 |
| MA | Taiz University Housing | World Assembly Housing | 2770 | 0.7565 | 1 |
| MA | Taiz University Housing | Al-Janad University Housing | 563 | 0.7086 | 1 |
| MA | World Assembly Housing | Al-Janad University Housing | 1042 | 0.602 | 1 |

# S7 Specialist appraisal

## Supplementary Table S12a  Specialist denominators

| type | code | label | n |
| --- | --- | --- | --- |
| profession | 1.0 | Architectural engineer | 20 |
| profession | 2.0 | Electrical engineer | 3 |
| profession | 3.0 | Project manager | 5 |
| profession | 4.0 | University professor | 3 |
| gender | 1.0 | Male | 23 |
| gender | 2.0 | Female | 8 |
| facility | 1.0 | Facility A | 10 |
| facility | 2.0 | Facility B | 5 |
| facility | 3.0 | Facility C | 9 |
| facility | 4.0 | Facility D | 7 |
| domain |  | Architecture (Q1-Q10) | 21 |
| domain |  | Plumbing (Q11-Q19) | 9 |
| domain |  | Electrical (Q20-Q29) | 4 |
| domain |  | Sustainability (Q30-Q40) | 22 |

## Supplementary Table S12b  Complete specialist item summaries

| item | label | domain_n | mean | SD | median |
| --- | --- | --- | --- | --- | --- |
| 1 | Design compatibility with student-housing function | 21 | 3.381 | 1.117 | 4.000 |
| 2 | Efficiency of room/facility distribution | 21 | 3.381 | 0.921 | 4.000 |
| 3 | Natural lighting and ventilation | 21 | 3.571 | 0.811 | 4.000 |
| 4 | Visual and acoustic privacy | 21 | 3.190 | 0.814 | 3.000 |
| 5 | Future expansion potential | 21 | 2.524 | 1.289 | 2.000 |
| 6 | Quality of outdoor spaces | 21 | 2.238 | 1.221 | 2.000 |
| 7 | Heat and noise treatment | 21 | 2.762 | 0.944 | 3.000 |
| 8 | Facade aesthetics/coherence | 21 | 3.571 | 0.746 | 4.000 |
| 9 | Effective natural/mechanical ventilation | 21 | 3.143 | 0.910 | 3.000 |
| 10 | Noise from building systems | 21 | 3.333 | 0.913 | 3.000 |
| 11 | Water pressure on all floors | 9 | 3.000 | 0.866 | 3.000 |
| 12 | Safety of supply/drainage networks | 9 | 2.556 | 0.726 | 2.000 |
| 13 | Water leakage or odors | 9 | 3.222 | 1.093 | 3.000 |
| 14 | Quality of fixtures and installation | 9 | 2.889 | 0.928 | 3.000 |
| 15 | Isolation from contamination sources | 9 | 3.111 | 0.601 | 3.000 |
| 16 | Ease of maintenance | 9 | 3.111 | 0.928 | 3.000 |
| 17 | Availability/safety of water heaters | 9 | 2.222 | 1.394 | 2.000 |
| 18 | Efficiency of drainage and water systems | 9 | 2.556 | 1.014 | 2.000 |
| 19 | Rainwater drainage | 9 | 2.889 | 1.453 | 3.000 |
| 20 | Safety of electrical installations | 4 | 3.000 | 0.000 | 3.000 |
| 21 | Appropriate electrical-load distribution | 4 | 3.250 | 0.500 | 3.000 |
| 22 | Sufficient electrical outlets | 4 | 2.500 | 1.000 | 2.000 |
| 23 | Effectiveness of internal lighting | 4 | 2.500 | 1.000 | 2.000 |
| 24 | Emergency lighting | 4 | 2.000 | 0.000 | 2.000 |
| 25 | Electrical fire alarm | 4 | 1.750 | 0.500 | 2.000 |
| 26 | Effective earthing | 4 | 3.250 | 0.500 | 3.000 |
| 27 | Condition of distribution panels | 4 | 2.500 | 0.577 | 2.500 |
| 28 | Overload/short-circuit protection | 4 | 3.000 | 0.816 | 3.000 |
| 29 | Kitchen/bathroom exhaust fans | 4 | 2.500 | 1.000 | 2.000 |
| 30 | Environmentally friendly building materials | 22 | 3.000 | 1.024 | 3.000 |
| 31 | Waste management and recycling | 22 | 1.682 | 0.716 | 2.000 |
| 32 | Solar panels/alternative energy | 22 | 3.364 | 1.002 | 4.000 |
| 33 | Energy-saving techniques | 22 | 3.318 | 0.894 | 3.500 |
| 34 | Water-saving systems | 22 | 3.500 | 0.964 | 4.000 |
| 35 | Natural ventilation/reduced air-conditioning dependence | 22 | 3.273 | 1.077 | 3.500 |
| 36 | Natural/artificial shading | 22 | 2.591 | 1.054 | 2.500 |
| 37 | Green spaces/landscaping | 22 | 2.500 | 1.225 | 2.000 |
| 38 | Environmental awareness | 22 | 2.727 | 0.935 | 3.000 |
| 39 | Future green-building potential | 22 | 2.773 | 1.193 | 3.000 |
| 40 | Use of additional eco-friendly materials | 22 | 2.955 | 0.375 | 3.000 |

## Supplementary Table S13  Descriptive resident-specialist thematic concordance

| theme | resident_mean | specialist_mean | resident_items | specialist_items | included_in_rank_concordance |
| --- | --- | --- | --- | --- | --- |
| Room function and privacy | 3.602 | 3.317 | 22,23,24,25 | 1,2,4 | Yes |
| Ventilation | 3.254 | 3.357 | 4,14,15,16,17 | 3,9 | Yes |
| Acoustic and privacy | 3.323 | 3.095 | 10,11,12,13,23 | 4,7,10 | Yes |
| Outdoor space and landscaping | 2.917 | 2.372 | 30 | 6,37 | Yes |
| Water and plumbing | 3.079 | 2.917 | 34,35 | 11,12,13,14,15,16,18,19 | Yes |
| Fire and emergency readiness | 3.093 | 1.875 | 18,19,20,21 | 24,25 | No |

The resident and specialist samples are independent. Five pre-matched non-electrical themes were compared at the level of aggregate theme means. Theme-level Spearman ρ=0.700 (p=0.188) and Pearson r=0.832 (p=0.080), n=5. These coefficients are descriptive across themes, not participant-level correlations or agreement statistics. Fire and emergency readiness is shown separately but excluded from the coefficients because the corresponding specialist evidence depends on the electrical domain n=4.

# S8 Software reproducibility and data governance

The final end-to-end Python reproduction was verified under Python 3.12.13, NumPy 2.3.5, SciPy 1.17.0, pandas 3.0.5, statsmodels 0.14.6, Matplotlib 3.10.8, JAX 0.9.0.1 and Pillow 12.3.0. Full ordinal WLSMV was executed under R 4.4.3 and lavaan 0.7-2. The controlled package contains de-identified participant-level analytic data and is for editorial review or approved access only. The public Zenodo archive excludes participant-level data and contains code, aggregate results, figures and software manifests. Zenodo DOI: https://doi.org/10.5281/zenodo.22305617.

# S9 Ethics and administrative documentation

The master’s research plan, including the anonymous questionnaire procedure, was reviewed and approved by the Research Plan Discussion Committee before survey administration under the Faculty’s master’s research-plan approval process. Signed and stamped Dean’s letters subsequently requested institutional cooperation and site access; copies are supplied separately for confidential editorial verification. The letters document institutional authorization and do not replace participant informed consent. The Committee is reported by its institutional name and is not relabeled as an IRB.