| Algorithm S1. Pseudo-code for RS-based CCD inverse kinematics algorithm |  |
| --- | --- |
|  | Input: rigid body tree description, starting configuration, end-effector target |
|  | Output: trajectory of configurations moving end-effector to target |
| 1: | while iteration < maximum number of iterations |
| 2: | for each module in the tree, from end-effector to base |
| 3: | for each joint associated with the module |
| 4: | initialise best joint value to current joint value |
| 5: | generate M random joint values within max. step size, include limits in set |
| 6: | evaluate cost function for each sample |
| 7: | sort values in order of ascending cost |
| 8: | for each sorted value |
| 9: | if no self-collision in the tree |
| 10: | store value as best joint value |
| 11: | break |
| 12: | end |
| 13: | end |
| 14: | end |
| 19: | end |
| 15: | save full tree configuration for current iteration in trajectory |
| 16: | if cost < convergence tolerance |
| 17: | return converged trajectory |
| 18: | end |
| 20: | iteration++ |
| 21: | end |
| 22: | return unconverged trajectory |