Supplementary Information for:

Towards dynamic resilience from static control of urban rainfall-induced pollution: a multi-stress pathway framework for ecological infrastructure

Jiali Li 1, Hao Zhu 1, 2, Yukun Ma 3, Dan Liu 1, 2, Faith Ka Shun Chan 4, 5, Ronghua Zhong 6, Hua Chai 1, 2, Haiyan Li 7, Bin Chen 3, Hongtao Zhao 1, 2,

Author affiliations:

1 State Key Laboratory of Regional and Urban Ecology, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, China

2 University of Chinese Academy of Sciences, Beijing 100049, China

3 State Key Laboratory of Regional Environment and Sustainability, School of Environment, Beijing Normal University, Beijing 100875, China

4 School of Geographical Sciences, Faculty of Science and Engineering, University of Nottingham Ningbo China, Ningbo 315100, China

5 Water@Leeds Research Institute and School of Geography, University of Leeds, LS2 9JT, United Kingdom

6 Institute of International Rivers and Eco-security, Yunnan University, Kunming 650091, China

7 Beijing Engineering Research Center of Sustainable Urban Sewage System Construction and Risk Control, Beijing University of Civil Engineering and Architecture, Beijing 100044, China

7 Beijing Engineering Research Center of Sustainable Urban Sewage System Construction and Risk Control, Beijing University of Civil Engineering and Architecture, Beijing 100044, China

No. of pages: 16

No. of Figure: 1

No. of tables: 11

Supplementary Figure

Figure S1| Analysis of the impact of three types of EI on the existing spatial distribution of water environment resilience. a Spatial distribution of the three EI types. b–c Spatial distribution of resilience indices. d Correlation analysis between resilience indices and EI area. e–f Distribution of resilience indices along an urbanization gradient. The EI types examined are Sponge City Projects (SCP, 8.6% of total EI area), Park Green (PG, i.e., constructed urban vegetation, 31.4%), and Regional Green (RG, i.e., natural forests and wetlands, 60.1%).

Supplementary Tables

Table S1. Comparison of urban resilience approaches

| Aspect | Holistic Approaches1 | Technical Approaches2 |
| --- | --- | --- |
| Examples | City Resilience Indexes3; Spatialised Urban Flood Resilience Index4; Available resilience assessment frameworks5, 6 | Spatial Analysis Framework7, 8; Smart City Framework9; Global Resistance Analysis10 |
| Strength | Wide-ranging perspective | Excellent for mechanistic analysis, identifying weaknesses, and targeted improvements |
| Weakness | Complex, hard to interpret, obscures causality | Uneven application; poor for complex systems |
| Focus Area | System-wide interactions1 | Overwhelmingly on engineering resilience (e.g., water supply10/drainage11, 12) |
| Key Limitation | Hinders building actionable solutions | Severely limits framework applicability to complex systems like EI |

Table S2. Summary statistics of monthly mean values for EWIs (Wuxi City, 2017–2020, Definition of indices showed in Table S9)

| Month | PRCPTOT | SDII | R×1day | CWD | CDD | R×nd | R2 mm | RP = 60% | RP = 75 | RP = 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 68.75 | 9.72 | 21.20 | 4.17 | 13.83 | 36.37 | 7.50 | 1.17 | 0.33 | 0.00 |
| 2 | 53.23 | 7.11 | 17.27 | 3.17 | 11.67 | 28.10 | 6.67 | 0.83 | 0.33 | 0.00 |
| 3 | 69.03 | 8.32 | 22.23 | 3.83 | 11.50 | 29.68 | 8.17 | 1.00 | 0.83 | 0.17 |
| 4 | 83.32 | 15.27 | 37.73 | 1.83 | 9.17 | 45.00 | 5.83 | 2.00 | 0.83 | 0.50 |
| 5 | 71.72 | 11.25 | 27.43 | 2.00 | 12.50 | 32.60 | 6.67 | 1.67 | 0.50 | 0.33 |
| 6 | 161.50 | 17.27 | 60.97 | 3.17 | 8.00 | 73.62 | 8.67 | 3.50 | 2.33 | 1.50 |
| 7 | 210.45 | 20.81 | 59.73 | 3.83 | 9.83 | 107.50 | 9.00 | 4.83 | 3.17 | 1.83 |
| 8 | 167.57 | 19.06 | 62.37 | 3.33 | 8.50 | 69.22 | 9.00 | 4.67 | 1.83 | 1.17 |
| 9 | 143.85 | 22.83 | 81.25 | 3.17 | 14.33 | 113.32 | 6.17 | 2.83 | 1.83 | 1.17 |
| 10 | 53.52 | 10.31 | 22.33 | 2.00 | 16.00 | 36.55 | 4.83 | 1.33 | 0.33 | 0.33 |
| 11 | 60.05 | 11.57 | 26.00 | 1.83 | 13.17 | 35.35 | 5.00 | 1.33 | 0.83 | 0.17 |
| 12 | 39.95 | 6.18 | 11.73 | 2.50 | 13.67 | 18.70 | 5.50 | 0.67 | 0.17 | 0.00 |
| Mean | 98.58 | 13.31 | 37.52 | 2.90 | 11.85 | 52.17 | 6.92 | 2.15 | 1.11 | 0.60 |
| SD | 56.41 | 5.58 | 22.59 | 0.84 | 2.53 | 31.58 | 1.52 | 1.47 | 0.96 | 0.65 |

Table S3. Comparison of RP = 60 and CWD stress under Chinese VCR areas

| Area | VCR | RP = 60 stress (days) |  |  | CWD stress(days) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 40% | 60% | 80% | 40% | 60% | 80% |
| I | 85% | 0.26 | 0.99 | 9 | 1.50 | 3.50 | 29 |
| II | 80% | 1.17 | 3.00 | 13 | 2.56 | 5.17 | 25 |
| III | 75% | 2.26 | 4.73 | 17 | 3.26 | 5.61 | 24 |
| IV | 70% | 2.12 | 4.56 | 18 | 2.83 | 5.12 | 29 |
| V | 60% | 3.28 | 6.61 | 19 | 4.42 | 7.98 | 28 |

Table S4. SDGs relevant to the construction of resilient water environments13

| SDGs | Domination | Target |  |
| --- | --- | --- | --- |
| Goal 9 | Build resilient infrastructure, promote sustainable industrialization and foster innovation | 9.1 | Develop quality, reliable, sustainable and resilient infrastructure, including regional and transborder infrastructure, to support economic development and human well-being, with a focus on affordable and equitable access for all |
|  |  | 9.4 | Upgrade infrastructure and retrofit industries to make them sustainable, with increased resource-use efficiency and greater adoption of clean and environmentally sound technologies and industrial processes, with all countries taking action in accordance with their respective capabilities |
| Goal 11 | Make cities inclusive, safe, resilient and sustainable | 11.3 | Enhance inclusive and sustainable urbanization and capacity for participatory, integrated and sustainable human settlement planning and management in all countries |
|  |  | 11.B | Substantially increase the number of cities and human settlements adopting and implementing integrated policies and plans towards inclusion, resource efficiency, mitigation and adaptation to climate change, resilience to disasters, and develop and implement, in line with the Sendai Framework for Disaster Risk Reduction 2015-2030, holistic disaster risk management at all levels |
| Goal 13 | Take urgent action to combat climate change and its impacts | 13.1 | Strengthen resilience and adaptive capacity to climate-related hazards and natural disasters in all countries |
|  |  | 13.2 | Integrate climate change measures into national policies, strategies and planning |

Table S5. Parameters for WQI calculation

| Variable （mg/L） | Relative weight （Pi） | Normalization factor（Ci） |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 100 | 90 | 80 | 70 | 60 | 50 | 40 | 30 | 20 | 10 | 0 |
| DO | 4 | >=7.5 | >7 | >6.5 | >6 | >5 | >4 | >3.5 | >3 | >2 | >=1 | <1 |
| CODMn | 3 | <2 | - | <4 | - | <6 | - | <10 | - | <15 | - | >15 |
| NH4+-N | 3 | <0.01 | <0.05 | <0.1 | <0.2 | <0.3 | <0.4 | <0.5 | <0.75 | <1 | <1.25 | >1.25 |
| CODCr | 3 | <5 | <10 | <20 | <30 | <40 | <50 | <60 | <80 | <100 | <=150 | >150 |
| TP | 1 | <0.01 | <0.02 | <0.05 | <0.1 | <0.15 | <0.2 | <0.25 | <0.3 | <0.35 | <=0.4 | >0.4 |

Table S6. Catchment unit information and runoff control targets

| Unit | Unit code | Area （km2） | Water surface rate % | Green land rate % | Pervious surface rate% | Runoff coefficient | VCR before SCP （%） | VCR target after SCP（%） | Annual runoff pollution reduction rate (SS, %) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | BSH-1 | 2.4441 | 11 .04 | 35.54 | 45.2 | 0.61 | 44.1 | 77% | 62% |
| 2 | BSH-2 | 4.1703 | 8.80 | 27.95 | 46.2 | 0.64 | 39.9 | 75% | 60% |
| 3 | CGX-1 | 3.1218 | 12.50 | 37.78 | 37.8 | 0.62 | 43.4 | 70% | 56% |
| 4 | CGX-2 | 3.4528 | 2.14 | 73.37 | 73.4 | 0.35 | 66.6 | 75% | 60% |
| 5 | DQB-1 | 1.4437 | 1.55 | 18.52 | 25.0 | 0.75 | 18.5 | 65% | 52% |
| 6 | DQB-2 | 1.5131 | 2.20 | 22.53 | 27.7 | 0.72 | 22.5 | 65% | 52% |
| 7 | HDJ-1 | 0.1891 | 2.02 | 32.67 | 36.7 | 0.63 | 37.4 | 65% | 52% |
| 8 | HDJ-2 | 1.5588 | 7.84 | 65.57 | 66.4 | 0.41 | 64.4 | 76% | 61% |
| 9 | HDJ-3 | 1.8381 | 6.04 | 77.36 | 78.5 | 0.32 | 72.3 | 75% | 60% |
| 10 | HSHB | 4.0151 | 0.38 | 83.60 | 83.8 | 0.27 | 73.2 | 80% | 64% |
| 11 | HSHN-1 | 3.3568 | 10.16 | 49.74 | 50.4 | 0.53 | 52.4 | 76% | 61% |
| 12 | HSHN-2 | 1.5663 | 8.09 | 29.12 | 29.1 | 0.67 | 35.4 | 67% | 54% |
| 13 | HSHN-3 | 2.1272 | 14.37 | 59.03 | 59.5 | 0.47 | 62.3 | 75% | 60% |
| 14 | HSHZ-1 | 2.0993 | 1.08 | 67.71 | 67.8 | 0.39 | 62.0 | 75% | 60% |
| 15 | HSHZ-2 | 1.4059 | 2.17 | 21.03 | 21.9 | 0.72 | 28.7 | 68% | 54% |
| 16 | HSHZ-3 | 2.9983 | 1.64 | 33.29 | 33.9 | 0.64 | 37.0 | 64% | 51% |
| 17 | HSHZ-4 | 1.3547 | 5.39 | 69.15 | 69.1 | 038 | 65.3 | 75% | 60% |
| 18 | LHN | 1.9968 | 13.62 | 30.99 | 32.6 | 0.67 | 31.0 | 66% | 53% |
| 19 | LQGD-1 | 1.3333 | 4.64 | 18.51 | 18.6 | 0.75 | 26.1 | 64% | 51% |
| 20 | LQGD-2 | 1.2788 | 10.45 | 41.72 | 41.7 | 0.59 | 46.3 | 69% | 55% |
| 21 | LQGX-1 | 6.2114 | 6.14 | 43.35 | 43.7 | 0.57 | 46.0 | 68% | 54% |
| 22 | LQGX-2 | 1.9681 | 6.17 | 56.51 | 57.3 | 0.47 | 56.3 | 72% | 58% |
| 23 | LXHB | 7.0794 | 0.73 | 80.43 | 80.7 | 0.29 | 71.1 | 80% | 64% |
| 24 | LXHN-1 | 2.9662 | 4.19 | 32.50 | 32.5 | 0.65 | 36.5 | 65% | 52% |
| 25 | LXHN-2 | 2.9295 | 3.22 | 16.19 | 16.7 | 0.77 | 23.9 | 63% | 50% |
| 26 | LXHZ-1 | 2.7277 | 2.77 | 26.40 | 26.4 | 0.69 | 31.6 | 64% | 51% |
| 27 | LXHZ-2 | 1.4898 | 3.63 | 22.04 | 22.0 | 0.73 | 28.1 | 64% | 51% |
| 28 | LXHZ-3 | 2.5694 | 0.54 | 23.12 | 23.6 | 0.71 | 28.8 | 66% | 53% |
| 29 | LXHZ-4 | 2.7577 | 4.31 | 15.59 | 15.6 | 0.77 | 23.6 | 64% | 51% |
| 30 | MAH | 8.696 | 9.30 | 80.21 | 80.8 | 0.30 | 76.7 | 80% | 64% |
| 31 | MSB-1 | 7.9967 | 9.98 | 37.24 | 39.0 | 0.61 | 42.8 | 70% | 56% |
| 32 | MSB-2 | 5.1608 | 24.70 | 39.14 | 39.1 | 0.62 | 50.0 | 72% | 58% |
| 33 | MSB-3 | 4.3206 | 5.10 | 39.82 | 39.8 | 0.60 | 42.6 | 71% | 57% |
| 34 | MSN | 30.1061 | 6.42 | 78.46 | 79.0 | 0.31 | 73.3 | 85% | 68% |
| 35 | RZG-1 | 2.0101 | 8.27 | 28.88 | 31.2 | 0.67 | 35.9 | 63% | 50% |
| 36 | RZG-2 | 3.1118 | 3.59 | 20.11 | 28.0 | 0.71 | 29.8 | 63% | 50% |
| 37 | XCH-1 | 6.4718 | 24.06 | 48.37 | 44.5 | 0.56 | 49.5 | 74% | 59% |
| 38 | XCH-2 | 1.8899 | 5.97 | 33.65 | 35.5 | 0.65 | 33.7 | 66% | 53% |
| 39 | XLN | 11.8449 | 15.89 | 51.77 | 58.7 | 0.50 | 59.2 | 80% | 64% |
| 40 | XLX | 33.6179 | 19.41 | 65.72 | 45.2 | 0.42 | 71.5 | 80% | 64% |
| 41 | XSH-1 | 3.1824 | 16.27 | 32.95 | 33.1 | 0.66 | 40.3 | 67% | 54% |
| 42 | XSH-2 | 1.4621 | 7.36 | 23.57 | 30.0 | 0.70 | 32.3 | 68% | 54% |
| 43 | XSH-3 | 2.8802 | 3.37 | 19.62 | 26.7 | 0.73 | 28.3 | 64% | 51% |
| 44 | YH | 5.4458 | 16.24 | 56.41 | 56.7 | 0.49 | 61.4 | 80% | 64% |
| 45 | ZHGX | 3.6683 | 4.77 | 18.97 | 20.6 | 0.74 | 27.3 | 59% | 47% |
| 46 | ZSTB-1 | 4.1004 | 5.40 | 21.93 | 22.4 | 0.72 | 29.4 | 60% | 48% |
| 47 | ZSTB-2 | 2.2881 | 2.49 | 12.90 | 15.3 | 0.78 | 22.6 | 58% | 46% |
| 48 | ZSTB-3 | 2.3095 | 7.80 | 60.27 | 70.7 | 0.42 | 62.6 | 68% | 54% |
| 49 | ZSTD | 9.8883 | 5.91 | 74.56 | 76.8 | 0.34 | 70.4 | 80% | 64% |
| 50 | ZSTX-1 | 2.0427 | 6.61 | 38.13 | 43.3 | 0.59 | 43.4 | 71% | 57% |
| 51 | ZSTX-2 | 1.9895 | 7.49 | 48.68 | 48.7 | 0.53 | 50.6 | 66% | 53% |

Table S7. Catchment unit sample information

| Year Before | Year After | N | Catchment Unit ID |
| --- | --- | --- | --- |
| 2017 | 2018 | 5 | DQB-1、MLHN-1 、MLHZ-1、MLHZ-2、MLHZ-4 |
| 2018 | 2019 | 14 | BSH-1、CGX-1、DQB-1、HDJ-2、HDJ-3、LQGD-1、MLHN-1、MLHZ-1、MLHZ-2、MLHZ-4、ZSTX-1、XLN、MS、ZSTB-3 |
| 2019 | 2020 | 22 | BSH-1、CGX-1、CGX-3、DQB-1、DQB-2、HDJ-2、HDJ-3、HSHN-1、HSHN-2、LHN-2、LQGD-1、LWH-3、MLHN-1、MLHN-2、MLHZ-1、MLHZ-2 、MLHZ-4、ZSTB-2、ZSTB-3、STX-1、XLN、MS |
| 2020 | 2021 | 24 | BSH-1、CGX-1、CGX-3、DQB-1、DQB-2、HDJ-2、HDJ-3、HSHN-1、HSHN-2、LHN-2、LQGD-1、LWH-3、MLHN-1、MLHN-2、MLHZ-1、MLHZ-2、MLHZ-4、MW-2、XCH-2、ZSTB-2、ZSTB-3 、ZSTX-1、XLN、MS |
| 2021 | 2022 | 23 | BSH-1、CGX-1、CGX-3、HDJ-2、HDJ-3、LHN-2、LQGD-1、LQGD-2、LQGX-2、LQGX-3、LWH-1、LWH-3、MLHN-1、MLHN-2 、MLHZ-1、MLHZ-2 、MLHZ-4、ZSTB-3、ZSTX-1、ZSTX-2、YH、XLN、XLX |

Note: Paired data deletion was applied to catchment units lacking water quality data for more than 6 months across compared years.

Table S8. Runoff control rate vs. design rainfall depth in the study area

| N | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Design Rainfall（mm） | 4.0 | 6.0 | 8.0 | 9.8 | 10.0 | 11.5 | 12.0 | 13.5 |
| VCR（%） | 26 | 36 | 44 | 50 | 51 | 55 | 56 | 60 |
| N | 9 | 10 | 11 | 12 | 13 | 14 | 15 | - |
| Design Rainfall（mm） | 15.9 | 18.8 | 22.5 | 27.2 | 33.5 | 43.4 | 64.3 | - |
| VCRRControl（%） | 65 | 70 | 75 | 80 | 85 | 90 | 95 | - |

Table S9. Definition of 10 extreme precipitation indices

| No. | Descriptive Characteristic | Item | Definition | Unit |
| --- | --- | --- | --- | --- |
| 1 | Overall Statistical Parameters | PRCPTOT | Total precipitation on days with daily precipitation ≥ 2 mm per month | mm |
| 2 |  | SDII | Average precipitation on wet days (daily precipitation ≥ 2 mm) per month | mm/d |
| 3 |  | R×1d | Monthly maximum 1-day precipitation | mm |
| 4 | Precipitation Continuity | CWD | Maximum consecutive wet days with daily precipitation ≥ 2 mm | d/month |
| 5 |  | CDD | Maximum consecutive dry days with daily precipitation < 2 mm | d/month |
| 6 |  | R×nd | Monthly maximum consecutive precipitation | mm |
| 7 | Precipitation Intensity Categories. | R2.0 | Number of days per month with daily precipitation ≥ 2 mm | d |
| 8 |  | R13.5 | Number of days per month with daily precipitation ≥ 13.5 mm | d |
| 9 |  | R22.5 | Number of days per month with daily precipitation ≥ 22.5 mm | d |
| 10 |  | R33.5 | Number of days per month with daily precipitation ≥ 22.5 mm | d |

Table S10. Significance testing and classification of water quality improvement effects

| Year-month | Before SCP | After SCP | P | Increase value | Type |
| --- | --- | --- | --- | --- | --- |
| 2018-01 | 41.14286 | 54.0119 | 0.145 | 12.86904 | 2 |
| 2018-02 | 55.1786 | 55.0476 | 0.986 | -0.131 | 3 |
| 2018-03 | 61.0095 | 50.5429 | 0.194 | -10.4666 | 2 |
| 2018-04 | 45.95 | 57.25 | 0.147 | 11.3 | 2 |
| 2018-05 | 45.4643 | 43.7643 | 0.842 | -1.7 | 3 |
| 2018-06 | 35.5738 | 53.4881 | 0.013* | 17.9143 | 1 |
| 2018-07 | 37.3429 | 49.5357 | 0.051 | 12.1928 | 2 |
| 2018-08 | 31.0786 | 49.0833 | 0.017* | 18.0047 | 1 |
| 2018-09 | 34.65 | 47.3095 | 0.131 | 12.6595 | 2 |
| 2018-10 | 38.6429 | 53.0238 | 0.095 | 14.3809 | 2 |
| 2018-11 | 52.0024 | 52.6286 | 0.911 | 0.6262 | 2 |
| 2018-12 | 54.9857 | 53.4786 | 0.861 | -1.5071 | 3 |
| 2019-01 | 56.9273 | 58.3132 | 0.758 | 1.3859 | 2 |
| 2019-02 | 61.2863 | 59.5175 | 0.571 | -1.7688 | 3 |
| 2019-03 | 58.1056 | 61.6852 | 0.298 | 3.5796 | 2 |
| 2019-04 | 61.384 | 66.1049 | 0.098 | 4.7209 | 2 |
| 2019-05 | 48.6807 | 64.9189 | 0.002** | 16.2382 | 1 |
| 2019-06 | 57.5487 | 59.484 | 0.657 | 1.9353 | 2 |
| 2019-07 | 61.399 | 56.6694 | 0.203 | -4.7296 | 3 |
| 2019-08 | 59.9515 | 61.4694 | 0.694 | 1.5179 | 2 |
| 2019-09 | 58.9605 | 58.0347 | 0.799 | -0.9258 | 3 |
| 2019-10 | 61.1583 | 61.058 | 0.974 | -0.1003 | 3 |
| 2019-11 | 58.3885 | 63.1214 | 0.112 | 4.7329 | 2 |
| 2019-12 | 52.1199 | 70.2223 | <0.001*** | 18.1024 | 1 |
| 2020-01 | 57.3592 | 65.1648 | 0.006** | 7.8056 | 1 |
| 2020-02 | 59.3113 | 66.0682 | <0.001*** | 6.7569 | 1 |
| 2020-03 | 61.886 | 70.6183 | <0.001*** | 8.7323 | 1 |
| 2020-04 | 65.5341 | 71.252 | 0.017* | 5.7179 | 1 |
| 2020-05 | 63.593 | 67.0185 | 0.144 | 3.4255 | 2 |
| 2020-06 | 60.6336 | 64.7265 | 0.092 | 4.0929 | 2 |
| 2020-07 | 56.3405 | 61.1279 | 0.068 | 4.7874 | 2 |
| 2020-08 | 59.5386 | 60.6423 | 0.687 | 1.1037 | 2 |
| 2020-09 | 59.7438 | 62.7424 | 0.132 | 2.9986 | 2 |
| 2020-10 | 61.8906 | 67.1162 | 0.016* | 5.2256 | 1 |
| 2020-11 | 64.732 | 71.7898 | 0.004** | 7.0578 | 1 |
| 2020-12 | No data |  |  |  |  |
| 2021-01 | 64.8832 | 68.4939 | 0.089 | 3.6107 | 2 |
| 2021-02 | 65.875 | 71.2169 | 0.004** | 5.3419 | 1 |
| 2021-03 | 70.9091 | 70.7835 | 0.944 | -0.1256 | 3 |
| 2021-04 | 70.8203 | 72.894 | 0.348 | 2.0737 | 2 |
| 2021-05 | 67.3414 | 68.8267 | 0.455 | 1.4853 | 2 |
| 2021-06 | 64.5708 | 68.8371 | 0.061 | 4.2663 | 2 |
| 2021-07 | 60.9298 | 68.6014 | 0.001** | 7.6716 | 1 |
| 2021-08 | 61.318 | 60.7424 | 0.839 | -0.5756 | 3 |
| 2021-09 | 62.3651 | 68.3623 | 0.001** | 5.9972 | 1 |
| 2021-10 | 67.0143 | 65.0447 | 0.564 | -1.9696 | 3 |
| 2021-11 | 71.4025 | 67.9167 | 0.087 | -3.4858 | 3 |
| 2021-12 | 68.9376 | 75.5239 | 0.001** | 6.5863 | 1 |
| 2022-01 | No data |  |  |  |  |
| 2022-02 | 70.7233 | 73.0869 | 0.243 | 2.3636 | 2 |
| 2022-03 | 71.6101 | 62.0445 | <0.001*** | -9.5656 | 3 |
| 2022-04 | 72.3529 | 72.6718 | 0.851 | 0.3189 | 2 |
| 2022-05 | 67.1672 | 66.4009 | 0.737 | -0.7663 | 3 |
| 2022-06 | 68.4481 | 57.7882 | <0.001*** | -10.6599 | 3 |
| 2022-07 | 69.5559 | 59.8649 | <0.001*** | -9.691 | 3 |
| 2022-08 | 60.0028 | 62.1488 | 0.432 | 2.146 | 2 |
| 2022-09 | 67.5616 | 68.9263 | 0.505 | 1.3647 | 2 |
| 2022-10 | 65.8865 | 69.3416 | 0.200 | 3.4551 | 2 |
| 2022-11 | 67.874 | 78.2362 | <0.001*** | 10.3622 | 1 |
| 2022-12 | 77.8173 | 73.7906 | 0.003** | -4.0267 | 3 |

Note: Data from January 2020 and January 2022 were excluded due to insufficient sample size caused by equipment maintenance and river discontinuity. *P < 0.05, **P < 0.01, ***P < 0.001

Table S11. Definition of stress level

| Stress | Stress Degree | xi Scope | Description |
| --- | --- | --- | --- |
| 20% | Low Risk | [0, x̄ - S) | Values within this range indicate that the impact of this failure mode on the urban water environment is minimal and falls within the range of normal fluctuations. |
| 40% | Relatively Low Risk | [x̄ - S, x̄) | Values within this range indicate that the impact of this failure mode on the urban water environment is slightly above the normal level but remains manageable. |
| 60% | Medium Risk | [x̄, x̄ + S) | Values within this range indicate that the impact of this failure mode on the urban water environment has reached an average level, requiring attention. |
| 80% | Relatively High Risk | [x̄ + S, xmax) | Values within this range indicate that the impact of this failure mode on the urban water environment is significantly above the normal level, necessitating corresponding measures for intervention. |
| 100% | High Risk | [xmax, +∞) | Values within this range indicate that the impact of this failure mode on the urban water environment is extremely severe, potentially leading to serious water quality deterioration or other issues, and requiring immediate emergency measures. |

Reference

1.	Shi L, Zheng Q, Yang M, Liu L. A review of definitions, influence factors and assessment of urban resilience. Acta Ecologica Sinica 42, 6016–6029 (2022).

2.	Asghari F, et al. Resilience Assessment in Urban Water Infrastructure: A Critical Review of Approaches, Strategies and Applications. Sustainability 15,  (2023).

3.	Rockefeller Foundation. CITY RESILIENCE INDEX The Rockefeller Foundation: New York, 2011.

4.	Xie Z, et al. Assessment of urban flood resilience based on the socio-ecological composite index model: a case study in Wuhua District, Kunming City, China. Water Science and Technology 89, 252–270 (2023).

5.	Saja AMA, Goonetilleke A, Teo M, Ziyath AM. A critical review of social resilience assessment frameworks in disaster management. International Journal of Disaster Risk Reduction 35, 101096 (2019).

6.	Beceiro P, Brito RS, Galvao A. The Contribution of NBS to Urban Resilience in Stormwater Management and Control: A Framework with Stakeholder Validation. Sustainability 12,  (2020).

7.	Vercruysse K, Dawson DA, Wright N. Interoperability: A conceptual framework to bridge the gap between multifunctional and multisystem urban flood management. Journal of Flood Risk Management 12,  (2019).

8.	Gu T, et al. Spatial heterogeneity of urban resilience: Quantifying key determinants by spatial machine learning model embedded in density-structure-function framework. Cities 167, 106305 (2025).

9.	Oberascher M, Rauch W, Sitzenfrei R. Towards a smart water city: A comprehensive review of applications, data requirements, and communication technologies for integrated management. Sustainable Cities and Society 76,  (2022).

10.	Mugume SN, Gomez DE, Fu GT, Farmani R, Butler D. A global analysis approach for investigating structural resilience in urban drainage systems. Water Res. 81, 15–26 (2015).

11.	Diao KG, Sweetapple C, Farmani R, Fu GT, Ward S, Butler D. Global resilience analysis of water distribution systems. Water Res. 106, 383–393 (2016).

12.	Rodriguez M, Fu GT, Butler D, Yuan ZG, Cook L. Global resilience analysis of combined sewer systems under continuous hydrologic simulation. J. Environ. Manage. 344,  (2023).

13.	UNPE. Embedding the Environment in Sustainable Development Goals. 2013.