Appendix

Fig.A1. Calculation process of the level of economic development

# 1. Calculating the water footprint of energy production

Considering the energy production life cycle process, the above equation can be further expanded as:

WFEi=p=1n(WFEp(direct,i)blue+WFEp(direct,i)grey)+j=1nWFEj(indirect,i)blue+WFEj(indirect,i)grey	(A.1)

where: p indicates the production process; j indicates the energy category; and the other variables are the same as in Eq.(6) to Eq.(9).

The grey WFE is calculated as follow:

WFEdirect,igrey=GCmax-Cnat=L×VpCmax-Cnat	(A.2)

Where, the G represents the volume of pollutants emitted per unit of energy production (mg/GJ); L and Vp are represent the amount of wastewater discharged per unit of energy production (m³/GJ) and the concentration of pollutants in wastewater per unit volume (mg/m³) respectively. Cmax represents the maximum permissible concentration of pollutants allowed in water quality standards (mg/m³). Cnat represents the initial concentration of pollutants in water bodies (mg/m³), which sets as 0. Due to the presence of multiple pollutants in industrial wastewater, COD serves as a significant indicator for measuring organic pollution in the industrial sector, particularly in the energy sector. Therefore, this paper selects COD as the evaluation index when calculating the grey WFE.

Table A.1

WF per unit of production in energy production life cycle in China

| Energy | Energy production processes |  | Values | References |
| --- | --- | --- | --- | --- |
| Coal | Raw coal mining | Water consumption COD emissions values The maximum cod value (Cmax) | 0.3 m3/t 300g/t 50mg/L | (MEPC, 2008) |
|  | Raw coal processing | Water consumption COD emissions values The maximum cod value (Cmax) | 0.15 m3/t 40g/t 70mg/L | (MEPC, 2006) |
| Crude oil | Crude oil extraction | Water consumption COD emissions values The maximum cod value (Cmax) | 7 m3/t 46.2g/t 50mg/L | (Gerbens-Leenes et al., 2008) |
|  | Crude oil processing | Water consumption | 0.68 m3/GJ |  |
| Natural gas | Natural gas extraction | Direct blue WF The maximum cod value (Cmax) | 0.003 m3/ m3 20 mg/L | (Ding et al., 2018) |
|  | Natural gas purification | Direct blue WF | 0.109 m3/GJ | (MEPC, 2002) |
| Thermal power |  | Direct blue WF Direct grey WF Indirect blue WF Indirect grey WF | 0.681 m3/GJ 0.083 mg/GJ 0.063 m3/GJ 0.387 mg/GJ | (Guan et al., 2019) |
| Hydropower | Hydropower generation The construction of dam | Direct blue WF Indirect blue WF | 6.75 m3/GJ 0.00168 m3/GJ | (Ding et al., 2018) |
| Nuclear power | Power generation in the plant | Direct blue WF COD emissions values | 0.015 m3/GJ 36.63 mg/MWh | (Zhang et al., 2019) |
|  | Uranium ore mining and the conversion, enrichment, fabrication of fuel. | Indirect blue WF COD emissions values The maximum cod value (Cmax) | 0.086 m3/GJ 874 mg/MWh 20mg/L | (Fthenakis & Kim, 2010) |
| Wind power |  | Indirect blue WF | 0.0005 m3/kWh | (Gerbens-Leenes et al., 2008) |
| Solar power |  | Indirect blue WF | 0.265 m3/GJ |  |

Notes: The direct grey WF for processing was not included in this study due to the unavailability of data. The grey WF of the processing plant was not calculated due to the unavailability of data. Since the China Environmental Statistics Yearbook includes the COD emissions from oil and gas extraction as one total value, this study will take the proportion of direct blue WF in oil and gas extraction as the proportion of COD emissions from oil and gas extraction.

# 2. Prediction model

In the sparrow algorithm, the basic parameters such as the number of iterations required for the population and the population size are specified, and the percentage of producers is also set. Suppose the number of sparrows in the population is N, and the optimal solution dimension is d. Each sparrow's position can be represented as X=x1,x2,⋯,xd, with a fitness value fi=fx1,x2,⋯,xd. The initial population can thus be represented as:

X=x11x12⋯x1dx21x22⋯x2d⋯⋯⋯⋯xn1xn2⋯xnd	(A.3)

Subsequently, the positions of the various subgroups within the population are iteratively updated according to specific rules. Additionally, it is essential to monitor for any warning indicators:

1)Discoverers

In each generation, the top PN sparrows with the highest fitness values are selected as discoverers. The position update formula for these discoverers is given by:

xi,jt+1=xi,jt∙exp-iα∙itermaxR2<STxi,jt+Q∙LR2≥ST	(A.4)

where, xi,jt represents the position information of the dimension j of the individual 𝑖 in the generation t. 𝛼 is a uniformly distributed random number in the interval [0,1], ∙itermax is the maximum number of iterations, Q is a random number following a normal distribution, and R2 is a uniformly distributed random number in the interval [0,1], representing the current warning value. ST denotes the alert threshold.

2) Followers

In addition to the discoverers, the remaining N-PN individuals serve as followers. The position update formula for the followers is given by:

xi,jt+1=Q∙expxworstt-xi,jt/i2i>N/2xpt+1+|xi,jt-xpt+1|∙A+∙Lotherwise	(A.5)

where, xp represents the optimal position of the current discoverers, and xworst represents the current worst position. A is a 1×D matrix, with each element randomly assigned a value of 1 or -1.

3) Scouts

Each generation randomly selects SD sparrows to act as scouts for early warning and exploration. The position update formula for scouts is as follows:

xi,jt+1=xbestt+β∙|xi,jt-xbestt|fi>fgxi,jt+K∙xi,jt-xworstt/fi-fw+εfi=fg	(A.6)

In the Equal (A.6), xbest denotes the current global optimal position, β is the step control parameter, K is a random number in the interval [-1,1], fi represents the fitness value of the sparrow i, and fg and fw are the global best and worst fitness values, respectively.

Upon completing the specified number of iterations, the final optimal solution is derived and subsequently integrated into the GRU neural network for parameter optimization and testing.

# 3. Validation Tests of Predict Models

Table A.2 reports the validation performance of the two SSA-based recurrent architectures considered in this study (SSA-LSTM and SSA-GRU) using RMSE, MAE, and R2. The SSA-GRU achieves substantially lower prediction errors (RMSE = 0.017; MAE = 0.013) than the SSA-LSTM (RMSE = 0.060; MAE = 0.047), indicating more accurate and stable out-of-sample fitting. In addition, SSA-GRU yields a positive coefficient of determination (R2 = 0.625), whereas SSA-LSTM produces a negative R2 (-4.102), implying that the latter performs worse than a mean-baseline predictor on the validation set. These validation results support our selection of SSA-GRU for the subsequent forecasting analysis.

Table A.2

Validation Tests

|  | RMSE | MAE | R2 |
| --- | --- | --- | --- |
| SSA-LSTM | 0.06 | 0.047 | -4.102 |
| SSA-GRU | 0.017 | 0.013 | 0.625 |

References

Ding, N., Liu, J., Yang, J., & Lu, B. (2018). Water footprints of energy sources in China: Exploring options to improve water efficiency. Journal of Cleaner Production, 174, 1021–1031. https://doi.org/10.1016/j.jclepro.2017.10.273

Fthenakis, V., & Kim, H. C. (2010). Life-cycle uses of water in U.S. electricity generation. Renewable and Sustainable Energy Reviews, 14(7), 2039–2048. https://doi.org/10.1016/j.rser.2010.03.008

Gerbens-Leenes, P. W., Hoekstra, A. Y., & Meer, Th. V. der. (2008). Water Footprint of Bio-energy and other primary energy carriers (NameValue of Water Report Series). https://research.rug.nl/en/publications/water-footprint-of-bio-energy-and-other-primary-energy-carriers

Guan, W., Zhao, X., & Xu, S. (2019). Spatiotemporal feature of the water footprint of energy and its relationship with water resources in China (In Chinese). Resources Science, 41(11). https://doi.org/10.18402/resci.2019.11.05

MEPC. (2002). Ministry of environmental protection of China.,GB3838-2002 quality standard of surface water in China. In: Ministry of Environmental Protection of China.

MEPC. (2006). Ministry of environmental protection of China.,GB20246-2006: Emission standard for pollutants from coal industry. In: Ministry of Environmental Protection of China. Beijing.

MEPC. (2008). Ministry of environmental protection of China.,HJ 446-2008: Cleaner production standardeCoal mining and processing industry. In: Ministry of Environmental Protection of China. Beijing.

Zhang, J., Lei, X., Chen, B., & Song, Y. (2019). Analysis of blue water footprint of hydropower considering allocation coefficients for multi-purpose reservoirs. Energy, 188, 116086. https://doi.org/10.1016/j.energy.2019.116086