Additional file 1: Table S1. Unit technologies and accessories of the MMV system.

| Unit Technologies |  | Content | Category | Comments | Level Obtained |
| --- | --- | --- | --- | --- | --- |
| Manipulation | Dump transfer | Well-to-well vertical transfer of solution from the donor to acceptor MMV using a spacer. | Simple manipulation | High transfer accuracy | Successfully achieved |
|  | Selective transfer | MMV-to-MMV selective transfer of solutions in specific wells using either of the following: 1. Packing (different patterns of holes) or 2. MMV dispenser system | In some instances, automation is required. | Aids generation of 1,024 different conditions. | Successfully achieved |
|  | Addition/mixing | Addition and mixing using packing of different patterns of holes and thickness. In addition, mixing by vigorous moving of small steel beads under a magnetic force. | Relatively complex manipulation |  | Successfully achieved |
|  | Division | Division of solution between two MMVs using the following: 1. By mainly specific filter or packing or stainless steel beads mediated well-to-well vertical (Z-type) or horizontal (X-type) transfer of a fraction of the solution. 2. Partially by MMV dispenser | Simple manipulation, but careful handling required. | Transfer rate needs to be controlled in some instances. | Method successfully developed, but optimization is required in some cases. |
|  | PCR | Isolation of cells, DNA extraction, and amplification of DNA, all in a single MMV chip. | Simple manipulation | MMV chip found to be compatible to the conventional thermocycler. | Successfully achieved (up to single molecule/0.5 µl level) |
|  | Coating | To prevent biofouling of molecules, MMV surface was coated using the following: 1. BSA (spin coating), 2. Silanization solution (coating by vapor deposition), 3. mPEG Thiol (spin coating) | Simple manipulation | BSA coating found to be most suitable. | Successfully developed and tested |
|  | Evaporation | Exposing MMV in a laminar air flow to evaporate excess solution for the prevention of cross-contamination and for further addition of chemicals. | Simple manipulation | Control of evaporation rate in each well needs to be achieved. | Successfully achieved |
|  | Pipetting | Recovery of solutions from MMV using the following: 1. MMV dispenser system, 2. Pipetting under a microscope | Slightly complex manipulation | Manual handling requires care and time. | Successfully developed and tested |
|  | Culture | Addition of media inoculated by cells of either of the following: 1. Bacteria, 2. Fungi, 3. Animal cells, followed by incubation and parallel observation of growth under different conditions. | Simple manipulation | Single cell culture is possible without traditional laborious colony-picking method. | Successfully achieved |
|  | Replica | Partial well-to-well transfer and back transfer of solutions leading to the distribution of the original solution to each well of the original and the replica MMVs (see Methods) 1. Replica of DNA, 2. Replica of cells | Simple manipulation | Useful in culture collection and library stock preparation. | Successfully achieved |
|  | Storage | Preservation of MMV at the following: 1. 4 °C, 2. −20 °C, 3. −80 °C, 4. Liquid nitrogen | Simple handling |  | Successfully achieved |
| Detection | Fluorescence | Staining of reaction products using fluorescent dyes (SYBR green I, SYBR gold, etc.), or detection of sample-specific fluorescence using a fluorescence microscope or fluoroimager. | Simple manipulation |  | Successfully achieved |
|  | Microscope | Visualization of MMV wells under optical microscope to observe sample charging and manipulation. | Simple handling |  | Successfully achieved |
|  | 4SR | Direct 3D electrophoresis of reaction products in each well of MMV using stacking on layers of sliced gels. | Relatively simple, multiparallel, and large-scale analysis of DNA/RNA/proteins/peptide. | Needs to be tested in polycarbonate MMV. | Successfully achieved (in polyacrylamide MMV) |
| Accessories | Frame | Silicon/urethane frame confines edges of MMV and helps in initial addition of solution. | Transfer accessory |  | Currently in use |
|  | Packing | Silicon/urethane packing for addition of solution to MMV and generation of multiple conditions. | Transfer accessory |  | Currently in use |
|  | Filter/spacer | Aids well-to-well transfer of solution from the donor to acceptor MMVs. Mainly constructed of silicon/urethane. | Transfer accessory |  | Currently in use |
|  | Beads and magnet-embedded centrifugal stand | 0.5-mm stainless steel beads and magnet were used for mixing the solution in MMV. | Transfer and centrifugation accessory | Stand was built in-house | Currently in use |
|  | Plastic case | To keep MMV in centrifuge basket for general transfer processes. | Basic accessory (centrifugation, transfer, storage, etc.) |  | Currently in use |
|  | Centrifuge flat bucket | For centrifugation of MMV. | Centrifugation accessory |  | Currently in use |
|  | Horizontal (X-mode) transfer MMV stand | To keep the donor and acceptor MMVs together and prevent leakage during centrifugal X-mode transfer. | Transfer and centrifugation accessory | Built in-house | Currently in use |
|  | Silicone tape | Used for sealing MMV and prevention of evaporation. | Basic accessory |  | Currently in use |
|  | MMV holder | To set MMV in MMV PCR machine. | PCR accessory |  | Currently in use |
|  | Silicone rubber and copper plate | For MMV PCR in conventional thermocycler. | PCR accessory |  | Currently in use |