Closed-loop Real-time Simulation Model of Hemodynamics and Oxygen transport in the Cardiovascular System

SUPPLEMENT

Supplement

Detailed model properties are provided below. Additional file 1: Figure S1 shows an electrical analogue of the overall model, while Additional file 14: Figure S2 shows the details of the cardiac, valvular and vascular compartments in an electrical analogue sketch.

The cardiac model

The time-varying elastance function e(t) [11]describing each cardiac chamber contractile and passive properties is shown in Equation S1.

et=emax(ved,q)∙a∙tα1∙Tn11+tα1∙Tn1∙11+tα2∙Tn2+eminv

Equation S1.

T is cardiac cycle length. α1, n1, α2 and n2 are dimensionless factors defining the shape of the chamber-specific elastance curves during contraction and relaxation phases (Additional file 3: Figure S3). The factor a is a normalizing scaling factor.

The systolic cardiac elastance contribution emax (ved, q) defines chamber contractility, as determined by the end-diastolic volume ved, output flow q, the chamber-specific constants maximum elastance Emax, maximum end-diastolic volume Ved,max and maximum output flow Qmax (Equation S2). This relation determines both the reduction of contractile force due to myocardial fiber stretch as determined by the quotient of end-diastolic volume ved and maximum end-diastolic volume Ved,max according to the “heart law of Starling” in a similar way to Sun et al  [8] (Figure 2) and the effect of an internal source resistance restricting maximum flow to Qmax in each cardiac chamber in accordance with previous publications  [11] making ventricular outflow more realistic.

emaxved, q=Emax∙1-vedVed,max4∙1-qQmax

Equation S2.

The term emin(v)  in Equation S1 is determined by a chamber-specific passive exponential pressure-volume relation defined by the minimum chamber elastance Emin, a constant σ and the zero volume v0 as previously described  [10] (Equation S3, Figure 2).

eminv=Emin∙eσ∙(v-v0)

Equation S3.

The presence of the term emin(v) in Equation S1 both during systole and diastole, adds further realism during blood-volume overload and severely depressed ventricular function with cardiac dilatation.

Viscous properties of the cardiac chamber walls are simulated with a pressure-dependent resistor Rwall  [9] in series with the variable elastance/capacitance e(t) determining contractile properties (Additional file14: Figure S2).

The heart valves

Valve pressure gradients are composed of a Bernoulli resistance and an inertial term (Equation S4). The valve constants determining valve leaflet opening and closure speed is separate from the inertial parameter L determining blood flow inertia [11].

∆P=B∙q∙q+L∙dqdt

Equation S4.

L is calculated as described in the main text (Equation 1) to better describe inertia when valve area is changing. Bernoulli resistance B is determined by density ρ and squared valve area A and is the dominating term in creating the valvular pressure gradient (Equation S5, Additional file 6: Figure S6).

B=ρ2∙A2

Equation S5.

The valve area A used in the calculations above should be considered as an effective area, rather than the real area, but to simplify formulas they are considered identical [11]. Parameter values are found in Table S4.

The pericardium

A function (Equation S6, Additional file 7: Figure S7) relating intra-pericardial pressure ppc to total heart volume vpc is adopted from the literature [9]. Pericardial volume and stiffness can be modified and fluid added by volume to simulate pericardial restrictive properties and tamponade respectively. Total heart volume vpc includes the four heart chambers and the pericardial fluid volume, but excludes the myocardial volume, which is considered constant. The parameters ppc,0, Kpc, Vpc,0 and φpc are constants (Table S3).

ppcv=ppc,0+Kpc∙e(vpc-Vpc,0)φpc

Equation S6.

As a further development of the previous model the minimal pericardial pressure ppc,0 is allowed to be a negative value in agreement with what is found experimentally when measured during intrathoracic pressure changes or hypovolemia [16].

The blood vessels

An electrical analogue (Additional file 1: Figure S1 and Additional file 14: Figure S2) can be used to simplify the understanding of the fluid mechanical properties of each vascular segment as earlier described [9, 11]. All parameters and output of this vascular model can be expressed both in rheological unit equivalents and as properties of an electrical circuit. The individual vessel segments are modeled in the electrical analogue way with a non-linear resistance (R) in series with a non-linear inductor (I), a non-linear capacitance (C) and a non-linear resistance (Ω) in series with the capacitor. The parameter R corresponds to its counterpart with the same name in rheology, while I represents mass flow inertia and C the volume-dependent vascular compliance. The parameter Ω is the resistance for loading and unloading the capacitor in the electric analogue and represents viscous dampening of the pressure and flow pulse in rheology.

The volume constant V0 is the volume and E0 is the elastance at a normal mean pressure P0 in each compartment (see table 9). The parameter φ (Equation S7) is a vessel-specific volume constant determining the non-linearity of the exponential pressure-volume relation.

Vascular elastance evascular in each segment can be calculated by derivation of Equation S7 as seen in Equation S8. The relation describes an increase in elastance/stiffness with increasing volume corresponding to progressive distension of the vascular wall (Additional file 8: Figure S12). Compliance C is identical to inverted vascular elastance. Equation S9 is derived from Equation S8 and helpful when calculating φ from P0 and E0.

pv=P0∙ev-V0φ

Equation S7.

evascularv=dpdv=P0φ∙ev-V0φ=E0∙ev-V0φ

Equation S8.

φ=P0E0

Equation S9.

The vascular dimensions used as input parameters are shown in Table S5. Properties derived from these parameters are showed in Table S6-7.

The coronary circulation

Coronary circulation is simulated with a left and right coronary artery emptying in the right atrium. Flow is dependent on a fixed coronary vascular resistance Rcoronary, aortic root pressure paortic root, and the maximum value of right atrial pressure pra and intraventricular pressure pventricular to create a vascular waterfall mechanism  [29] resulting in realistic coronary flow (Equation S10, Additional file 9: Figure S8).

qcoronary=paortic root-maximum(pventricular, pra)∙Rcoronary

Equation S10.

Baroreceptor reflex

A baroreceptor reflex based on Sun et al. can be activated in the model that affects heart rate, cardiac contractility (maximum elastance) and arterial vascular resistance in a linear fashion with an adjustable gain (G) according to the example in Equation S11 that shows how cardiac cycle time Tnew is determined based on normal heart cycle time Tsetpoint , normal aortic pressure psetpoint and present aortic pressure paorta  [9] (Figure 9).

Tnew=GHR∙paorta-psetpoint+Tsetpoint

Equation S11.

Table S1. Basic cardiac parameters.

|  | Contractility Emax | Time delay | Cycle time T | α1 | n1 | α2 | n2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | mmHg/ml | ms | ms | --- | --- | --- | --- |
| Right atrium | 0.04 | 85 | 833 | 0.100 | 1.2 | 0.200 | 10 |
| Right ventricle | 0.60 | 245 | 833 | 0.200 | 1.2 | 0.250 | 10 |
| Left atrium | 0.05 | 95 | 833 | 0.100 | 1.2 | 0.200 | 10 |
| Left ventricle | 2.80 | 245 | 833 | 0.200 | 1.2 | 0.250 | 20 |

Table S2. Basic diastolic and other cardiac parameters.

|  | Stiffness Emin | Sigma σ | Maximal volume Ved,max | Maximal flow Qmax | Viscous damping Rwall |
| --- | --- | --- | --- | --- | --- |
|  | mmHg/ml | ml-1 | ml | ml/s | mmHg·s/ml |
| Right atrium | 0.06 | 0.0015 | 400 | 4000 | 0.00050 |
| Right ventricle | 0.04 | 0.0030 | 400 | 2000 | 0.00020 |
| Left atrium | 0.07 | 0.0015 | 400 | 4000 | 0.00050 |
| Left ventricle | 0.05 | 0.0030 | 400 | 2000 | 0.00005 |

Table S3. Pericardial and septal parameters.

| Minimum pericardial pressure ppc,0 | Pericardial pressure constant Kpc | Pericardial volume constant Vpc,0 | Pericardial volume constant φpc | Atrial septal stiffness Esa0 | Ventricular septal stiffness Esv0 |
| --- | --- | --- | --- | --- | --- |
| mmHg | ml/s | ml | ml | mmHg/ml | mmHg/ml |
| -2 | 1.0 | 320 | 50 | 12 | 46 |

Table S4. Valve parameters.

|  | Closed area | Open area | Bernoulli resistance | Opening constant | Closure constant |
| --- | --- | --- | --- | --- | --- |
|  | cm2 | cm2 | mmHg·s/ml | (mmHg·s)-1 | (mmHg·s) -1 |
| Tricuspid valve | 0.0 | 5.0 | 0.00040 | 40 | 30 |
| Pulmonary valve | 0.0 | 5.0 | 0.00040 | 30 | 30 |
| Mitral valve | 0.0 | 5.0 | 0.00040 | 40 | 20 |
| Aortic valve | 0.0 | 5.0 | 0.00040 | 20 | 20 |

Table S5. Vascular parameters – dimensions – input data.

|  | Length | Radius | Thickness | Young’s modulus | Number | Normal mean pressure |
| --- | --- | --- | --- | --- | --- | --- |
|  | l | r | h | Y | n | P0 |
|  | cm | cm | cm | mmHg | --- | mmHg |
| Aortic root | 2,00 | 1,6000 | 0,16300 | 3000 | 1 | 80 |
| Ascending aorta | 4,00 | 1,4700 | 0,16300 | 3000 | 1 | 80 |
| Proximal aortic arch | 2,00 | 1,2600 | 0,12600 | 3000 | 1 | 80 |
| Distal aortic arch | 3,90 | 1,1900 | 0,11500 | 3000 | 1 | 80 |
| Descending aorta | 20,00 | 1,1000 | 0,10000 | 3000 | 1 | 80 |
| Peripheral arteries | 30,00 | 0,1600 | 0,05000 | 3000 | 80 | 80 |
| Resistance arteries | 4,00 | 0,0250 | 0,01000 | 3000 | 7000 | 80 |
| Right carotid artery | 20,00 | 0,4730 | 0,06300 | 3000 | 1 | 80 |
| Right carotid resistance arteries | 4,00 | 0,0250 | 0,02000 | 3000 | 1200 | 80 |
| Right carotid capillaries | 0,02 | 0,0005 | 0,00001 | 3000 | 2400000000 | 6 |
| Right carotid vein | 12,00 | 0,1000 | 0,00100 | 3000 | 1500 | 4 |
| Left carotid artery | 20,00 | 0,4130 | 0,06300 | 3000 | 1 | 80 |
| Left carotid resistance arteries | 4,00 | 0,0250 | 0,02000 | 3000 | 1200 | 80 |
| Left carotid capillaries | 0,02 | 0,0005 | 0,00001 | 3000 | 2400000000 | 6 |
| Left carotid vein | 12,00 | 0,1000 | 0,00100 | 3000 | 1500 | 4 |
| Systemic capillaries | 0,02 | 0,0005 | 0,00001 | 3000 | 11200000000 | 6 |
| Capacitance vessels | 12,00 | 0,1000 | 0,00100 | 3000 | 7000 | 4 |
| Superior caval vein | 12,00 | 1,4000 | 0,15000 | 3000 | 1 | 4 |
| Inferior caval vein | 20,00 | 1,4000 | 0,15000 | 3000 | 1 | 4 |
| Pulmonary artery | 10,00 | 1,5000 | 0,12000 | 3000 | 1 | 12 |
| Pulmonary resistance arteries | 1,00 | 0,0600 | 0,01000 | 3000 | 2000 | 12 |
| Pulmonary capillaries | 0,02 | 0,0005 | 0,00001 | 3000 | 8000000000 | 10 |
| Pulmonary small veins | 6,00 | 0,0800 | 0,00100 | 3000 | 3000 | 8 |
| Pulmonary veins | 6,00 | 0,7000 | 0,10000 | 3000 | 4 | 6 |

Table S6. Vascular parameters – calculated from values in table S5.

|  | Volume | Volume | Elastance | Resistance | Inductance |
| --- | --- | --- | --- | --- | --- |
|  | Vₒ | φ | Eₒ | Rₒ | Lₒ |
|  | ml | ml | mmHg/ml | mmHg·s/ml | mmHg·s2/ml |
| Aortic root | 16.08 | 8.42 | 9.50034 | 0.00019 | 0.00019910 |
| Ascending aorta | 27.15 | 13.06 | 6.12515 | 0.00053 | 0.00047108 |
| Proximal aortic arch | 9.98 | 5.32 | 15.03732 | 0.00049 | 0.00032115 |
| Distal aortic arch | 17.35 | 9.58 | 8.35475 | 0.00120 | 0.00070170 |
| Descending aorta | 38.01 | 22.30 | 3.58726 | 0.00421 | 0.00210095 |
| Peripheral arteries | 193.02 | 32.94 | 2.42851 | 0.35053 | 0.00371177 |
| Resistance arteries | 54.98 | 7.33 | 10.91348 | 0.96403 | 0.00024028 |
| Right carotid artery | 14.06 | 5.63 | 14.21244 | 0.24415 | 0.02262250 |
| Right carotid resistance arteries | 9.42 | 0.63 | 127.32394 | 5.25036 | 0.00135436 |
| Right carotid capillaries | 37.70 | 7.54 | 0.79577 | 0.08381 | 0.00000001 |
| Right carotid vein | 565.49 | 150.80 | 0.02653 | 0.04674 | 0.00019797 |
| Left carotid artery | 10.72 | 3.75 | 21.35019 | 0.42048 | 0.02968836 |
| Left carotid resistance arteries | 9.42 | 0.63 | 127.32394 | 5.26930 | 0.00135680 |
| Left carotid capillaries | 37.70 | 7.54 | 0.79577 | 0.08413 | 0.00000001 |
| Left carotid vein | 565.49 | 150.80 | 0.02653 | 0.04685 | 0.00019821 |
| Systemic capillaries | 175.93 | 35.19 | 0.17052 | 0.01797 | 0.00000000 |
| Capacitance vessels | 2 638.94 | 703.72 | 0.00568 | 0.00991 | 0.00004219 |
| Superior caval vein | 73.89 | 1.84 | 2.17504 | 0.00194 | 0.00156017 |
| Inferior caval vein | 123.15 | 3.07 | 1.30502 | 0.00320 | 0.00259118 |
| Pulmonary artery | 70.69 | 7.07 | 1.69765 | 0.00128 | 0.00115793 |
| Pulmonary resistance arteries | 22.62 | 1.09 | 11.05243 | 0.02426 | 0.00003566 |
| Pulmonary capillaries | 125.66 | 41.89 | 0.23873 | 0.02674 | 0.00000000 |
| Pulmonary small veins | 361.91 | 154.42 | 0.05181 | 0.03046 | 0.00007991 |
| Pulmonary veins | 36.95 | 1.03 | 5.80011 | 0.00393 | 0.00078567 |
| Right coronary artery | - | - | - | 80.00 | - |
| Left coronary artery | - | - | - | 40.00 | - |

Table S7. More properties of the vascular system – calculated from values in table S5.

|  | Volume | Vascular surface area | Vascular wall volume |
| --- | --- | --- | --- |
|  | ml | m2 | ml |
| Systemic arteries | 438 | 0.87 | 230 |
| Systemic capillaries | 251 | 101 | 10 |
| Systemic veins | 3967 | 7.57 | 118 |
| Pulmonary arteries | 93 | 0.08 | 19 |
| Pulmonary capillaries | 126 | 50.3 | 5 |
| Pulmonary veins | 399 | 0.92 | 20 |
| TOTAL | 5274 | 160 | 401 |