Additional file 1

Title: Comparative evaluation of three interfaces for non-invasive ventilation: a randomized cross-over design physiologic study on healthy volunteers.

Version 24 Nov 2013

Rosanna Vaschetto1,2 MD, PhD, Audrey De Jong2 MD, Matthieu Conseil MD2,

Fabrice Galia2, Martin Mahul2 MD, 2 MD, Yannael Coisel2 MD, Albert Prades2,

Paolo Navalesi3,4,5 MD, Samir Jaber2 MD, PhD.

1Anesthesia and Intensive Care Medicine, Maggiore della Carità Hospital, Novara, Italy.

2Intensive Care Unit and Transplantation, Critical Care and Anesthesia Department (DAR B), Hôpital Saint-Éloi, CHU de Montpellier, Montpellier, France.

3Dipartimento di Medicina Traslazionale, Università del Piemonte Orientale “Amedeo Avogadro”, Alessandria-Novara-Vercelli, Italy.

4Anesthesia and Intensive Care Medicine, Sant’Andrea Hospital, Vercelli, Italy.

5CRRF Mons. L. Novarese, Moncrivello, Vercelli, Italy.

An additional file in appendix contains two additional tables: Table E1 shows the main ventilatory variables and inspiratory efforts parameters obtained at baseline settings and following interventions at increased settings, obtained in spontaneous breathing (SB), facial mask (FM), standard helmet (HS) and helmet next (HN) with data presented as median [interquartiles]; and Table E2 shows the inspiratory and expiratory trigger delay and synchrony time obtained at baseline settings and following interventions at increased settings, obtaned in facial mask (FM), standard helmet (HS) and helmet next (HN) with data presented as median [interquartiles].

|  | SBI | Baseline Settings |  |  | Increased Settings |  | SBE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FM | HS | HN | HS | HN |  |
| VT ml | 730[591-967] | 579[503-662] | 760[617-885] | 773[648-1012] | 827[625-1060] | 830[634-1073] | 747[648-1143] |
| RRn breath/min | 12[8-16] | 13[10-17] | 13[10-15] | 13[12-16] | 13[10-15] | 14[11-16] | 12[10-15] |
| VE L/min | 8.1[6.2-11.1] | 8.1[6.5-9.2] | 9.4[7.7-13.3] | 9.1[8.3-14.1]† | 9.1[8.3-13.3]† | 10.0[8.1-15.0]† | 8.9[7.4-13.5] |
| PTPdi/breath | 17.9[10.8-29.7] | 17.3[5.6-22.9] | 15.3[6.2-26.5] | 13.2[6.1-25.6] | 9.0[4.8-18.5]* | 6.9[3.1-19.5]*† | 12.0[6.3-32.5] |
| PTPdi/min | 205[152-354] | 215[155-266] | 192[115-277] | 184[116-266] | 112[76-204]* | 94[61-205]*† | 181[98-284] |
| PTPdi/L | 22.4[17.3-36.1] | 31.3[16.9-34.2] | 19.4[11.0-35.4] | 18.4[6.7-31.4] | 13.3[6.4-25.1]*† | 8.6[3.8-23.2]*† | 15.2[6.7-32.3] |
| WOB J/breath | 0.493[0.243-0.609] | 0.331[0.236-0.377] | 0.337[0.240-0.430] | 0.359[0.211-0.650] | 0.282[0.165-0.599] | 0.301[0.130-0.609] | 0.393[0.173-0.535] |
| WOB J/min | 4.7[3.7-7.1] | 4.7[2.5-5.5] | 4.1[2.9-6.1] | 5.0[2.7-6.4] | 3.6[2.2-6.5] | 3.2[2.1-7.7] | 5.5[2.2-6.2] |
| WOB J/L | 0.539[0.392-0.760] | 0.509[0.355-0.676] | 0.441[0.391-0.590] | 0.538[0.336-0.675] | 0.407[0.263-0.672] | 0.363[0.226-0.585] | 0.479[0.348-0.636] |

Table E1. Main ventilatory variables and inspiratory efforts parameters obtained at baseline settings and following interventions at increased settings, obtained in spontaneous breathing (SB), facial mask (FM), standard helmet (HS) and helmet next (HN) with data presented as median [interquartiles].

Table E1. N=12 healthy volunteers included. Baseline settings for FM, HS, HN: PEEP 5cmH2O; PS 5cmH2O, Ramp 0; Exp trigger: 25%; FiO2: 21%. Increased settings for HS and HN: PEEP 8cmH2O; PS 8cmH2O, Ramp 0; Exp trigger: 25%; FiO2: 21%. Data are presented as median [interquartiles]. Data were analyzed with Friedman test and Dunn's Multiple Comparison Test, * p<0.05 vs. SBI. † p<0.05 vs. FM. VT: tidal volume; VE: minute ventilation; RRn: neural respiratory rate; PTPdi: pressure-time product of the transdiaphragmatic pressure; WOB: work of breathing; FM: face mask; HS: helmet standard; HN: helmet next; SBI: initial spontaneous breathing; SBE: end spontaneous breathing; PEEP: positive end-expiratory pressure; PS: pressure support; Exp trigger: expiratory trigger.

Table E2. Inspiratory and expiratory trigger delay and synchrony time obtained at baseline settings and following interventions at increased settings, obtained in facial mask (FM), standard helmet (HS) and helmet next (HN) with data presented as median [interquartiles].

|  | Baseline Settings |  |  | Increased Settings |  |
| --- | --- | --- | --- | --- | --- |
|  | FM | HS | HN | HS | HN |
| Delay,TR-insp (s) | 0.144[0.122-0.217] | 0.372[0.283-0.390]*† | 0.253[0.240-0.297]* | 0.325[0.300-0.415]*† | 0.228[0.171-0.254] |
| Delay,TR-exp (s) | 0.260[0.257-0.290] | 0.403[0.349-0.445]*† | 0.301[0.274-0.345] | 0.379[0.306-0.420]* | 0.305[0.231-0.344] |
| TI,Synchrony (%) | 92[86-93] | 79[74-83]* | 83[81-87] | 79[75-84]* | 85[83-88] |

Table E2. N=12 healthy volunteers included. Baseline settings for FM, HS, HN: PEEP 5cmH2O; PS 5cmH2O, Ramp 0; Exp trigger: 25%; FiO2: 21%. Increased settings for HS and HN: PEEP 8cmH2O; PS 8cmH2O, Ramp 0; Exp trigger: 25%; FiO2: 21%. Data are presented as median [interquartiles].

Data were analyzed with Friedman test and Dunn's Multiple Comparison Test, * p<0.05 vs. FM; † p<0.05 vs. HN Increased Settings. FM: face mask; HS: helmet standard; HN: helmet next; Delay,TR-insp: inspiratory trigger delay; Delay,TR-exp: expiratory trigger delay; TI,Synchrony: synchrony time as % of patient inspiratory time.