Additional file

Seasonal variability in metal and metalloid burdens of mussels: using data from the German Environmental Specimen Bank to evaluate implications for long-term mussel monitoring programs

Burkhard Knopf1*, Annette Fliedner1, Georg Radermacher1, Heinz Rüdel1, Martin Paulus2, Ulrike Pirntke3, Jan Koschorreck3

1 Fraunhofer Institute for Molecular Biology and Applied Ecology (Fraunhofer IME), Department Environmental Specimen Bank and Elemental Analysis, 57392 Schmallenberg, Germany.

2 Biogeography, University of Trier; 54286 Trier, Germany.

3 German Environment Agency (Umweltbundesamt), 06813 Dessau-Rosslau, Germany

*: Corresponding author. Tel: +49-2972-302-208, E-mail: Burkhard.knopf@ime.fraunhofer.de

Captions:

Table S1: ICP-MS parameters for metal and metalloid measurements.

Table S2: GC parameters, GC temperature program and ICP-MS parameters for organotin measurements.

Table S3: Correlation (p ≤ 0.05, non-directional) between the shell length of mussels and concentrations of analyses elements/compounds.

Table S4: Correlation (p ≤ 0.05, non-directional) between the condition index of mussels and concentrations of elements/compounds. NS: North Sea, BS: Baltic Sea.

Table S5: Correlation (p ≤ 0.05, non-directional) between the condition index of mussels and wet weight-based concentrations of elements/compounds - comparison between annual and monthly samples. NS: North Sea, BS: Baltic Sea.

Table S6: Cd-concentrations (ng g-1 dry weight dw and wet weight ww) of monthly samples and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea sites.  
For comparison: BAC and EC maximum food level used by HELCOM and OSPAR (wet weight based data without respiratory water).

Table S7: Pb-concentrations (ng g-1 dry weight dw and wet weight ww) of monthly samples and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea sites. For comparison: BAC and EC maximum food level used by HELCOM and OSPAR (wet weight based data without respiratory water).

Table S8: Hg concentrations (ng g-1 wet weight) of monthly samples and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea site. 
For comparison: BAC and EC maximum food level used by HELCOM and OSPAR and the Water Framework Directive (Directive 2013/39/EU) (wet weight based data without respiratory water).

Table S9: Comparison of Cd-thresholds used by HELCOM and OSPAR with Cd-concentrations in seasonal and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea sites (wet weight based data without respiratory water).

Table S10: Comparison of Pb-threshold values used by HELCOM and OSPAR with Pb-concentrations in monthly samples and annual composite samples of blue mussels (Mytilus edulis Complex) from sampling sites North Sea and Baltic Sea sites (wet weight based data without respiratory water).

Table S11: Comparison of Hg-thresholds used by HELCOM, OSPAR and the WFD with THg-concentrations in monthly and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea sites (wet weight based data without respiratory water).

Figure S1: Biometrical parameters of blue mussels (Mytilus edulis Complex) from the sampling sites of the German environmental specimen bank in the North Sea (Eckwarderhoerne, Koenigshafen) and the Baltic Sea (Darsser Ort). Shown are data of monthly samples and of annual composite samples of the respective year (dark columns).

Figure S2: Long-term trends and seasonal variations in concentrations of metals in blue mussel (Mytilus edulis Complex) sampled at the North Sea site Eckwarderhoerne. Upper row: long-term trends based on annual composite samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on bi-monthly samples from 2013, 2015, and 2017.

Figure S3: Long-term trends and seasonal variations in concentrations of metals in blue mussel (Mytilus edulis Complex) sampled at the North Sea site Koenigshafen. Upper row: long-term trends based on annual composite samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on bi-monthly samples from 2013, 2015, and 2017.

Figure S4: Long-term trends and seasonal variations in concentrations of metals in blue mussel (Mytilus edulis Complex) sampled at the Baltic Sea site Darsser Ort. Upper row: long-term trends based on annual composite samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on six-month samples from 2013, 2015, and 2017.

Table S1: ICP-MS parameters for metal and metalloid measurements.

| Rf power | 1450 |
| --- | --- |
| Carrier gas flow rate | 0.85 L/min |
| Makeup gas flow rate | 0.25 L/min |
| Nebulizer | Micro Mist |
| Spray chamber | Double pass quartz Scott type |
| Monitoring mass | 59Co, 60Ni, 61Ni, 63Cu, 65Cu, 75As, 78Se, 82Se, 111Cd, 114Cd,206Pb, 207Pb and 208Pb |
| Integration time | Depending on element and gas mode used |
| Collision/Reaction gas mode | noGas, Helium and Hydrogen mode (Hydrogen only for Se) |

Table S2: GC parameters, GC temperature program and ICP-MS parameters for organotin measurements.

| GC-parameters |  |  |  |
| --- | --- | --- | --- |
| Injector |  | Splitless |  |
| Injector temperature |  | 200 °C |  |
| Injection volume |  | 2 µL |  |
| Carrier flow |  | He 2.00 mL/min |  |
| Additional flow |  | Ar 70.01% |  |
| Column |  | RTX-5 (fused silica), 5% polydiphenyl-/95% polydimethylsiloxane, length 30 m, ID 0.32 mm, df 0.25 µm |  |
| Transfer line |  | Silcosteel, Temperature 300 °C |  |
| GC temperature program |  |  |  |
| Rate [°C/min] |  | End temperature [°C] | Dwell time [min] |
| 0.0 |  | 60.0 | 1.20 |
| 80.0 |  | 100.0 | 0.00 |
| 20.0 |  | 160.0 | 0.00 |
| 80.0 |  | 300.0 | 0.00 |
| 90.0 |  | 310.0 | 1.00 |
| ICP-MS parameters |  |  |  |
| Dwell time |  | 70 ms |  |
| Plasma power |  | 1550 W |  |
| Additional gas flow |  | 0.8 L/min |  |
| Measured Isotopes |  | 118Sn, 119Sn, 120Sn |  |

Table S3: Correlation (p ≤ 0.05, non-directional) between the shell length of mussels and concentrations of analyses elements/compounds.

|  | NS - Eckwarderhoerne |  | NS - Koenigshafen |  | BS – Darsser Ort |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wet weight | dry weight | wet weight | dry weight | wet weight | dry weight |
| Cobalt | - | - | +* | - | +* | +* |
| Nickel | - | - | - | + | +* | +* |
| Copper | - | - | +* | - | - | +* |
| Cadmium | + | - | +* | - | - | - |
| Mercury | - | - | - | + | +* | +* |
| Lead | - | - | - | - | +* | +* |
| Arsenic | - | - | +* | - | - | + |
| Selenium | - | - | +* | - | - | +* |
| MeHg | - | - | +* | - | + | - |
| Tributyl tin | - |  | +* |  | - |  |

*negative correlation

Table S4: Correlation (p ≤ 0.05, non-directional) between the condition index of mussels and concentrations of elements/compounds. NS: North Sea, BS: Baltic Sea.

|  | NS - Eckwarderhoerne |  | NS - Koenigshafen |  | BS – Darsser Ort |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wet weight | dry weight | wet weight | dry weight | wet weight | dry weight |
| Cobalt | - | - | - | +* | + | - |
| Nickel | - | - | - | +* | - | +* |
| Copper | - | - | + | - | + | +* |
| Cadmium | + | - | - | +* | - | - |
| Mercury | - | - | - | - | - | - |
| Lead | - | - | - | +* | - | - |
| Arsenic | - | - | - | - | - | - |
| Selenium | - | +* | - | - | + | - |
| MeHg | - | - | + | - | - | - |
| Tributyl tin | - |  | - |  | +* |  |

*negative correlation

Table S5: Correlation (p ≤ 0.05, non-directional) between the condition index of mussels and wet weight-based concentrations of elements/compounds - comparison between annual and monthly samples. NS: North Sea, BS: Baltic Sea.

|  | NS - Eckwarderhoerne |  | NS - Koenigshafen |  | BS – Darsser Ort |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | annual pool sample | monthly sample | annual pool sample | monthly sample | annual pool sample | monthly sample |
| Cobalt | - | - | - | - | + | - |
| Nickel | - | - | - | - | - | - |
| Copper | - | + | + | + | + | - |
| Cadmium | + | - | - | - | - | - |
| Mercury | - | - | - | + | - | - |
| Lead | - | - | - | - | - | - |
| Arsenic | - | - | - | + | - | - |
| Selenium | - | - | - | + | + | + |
| Methyl-Hg | - | - | + | + | - | - |
| Tributyl tin | - | - | - | + | +* | + |

*negative correlation

Table S6: Cd-concentrations (ng g-1 dry weight and wet weight ww) of monthly samples and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea sites.  
For comparison: BAC and EC maximum food level used by HELCOM and OSPAR (wet weight based data without respiratory water).

| sampling site | Monthly sample or annual pool sample | Cd - HELCOM: 960 ng g-1 dry weight; BAC based |  |  | Cd - OSPAR: 1000 ng g-1 wet weight; EC maximum food level based |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Year 2013 | Year 2015 | Year 2017 | Year 2013 | Year 2015 | Year 2017 |
| North Sea Koenigshafen | February | 1235 | 1049 | 1137 | 90.6 | 68.4 | 67.4 |
|  | April | 1169 | 1112 | 1231 | 101 | 77.4 | 75.4 |
|  | June | 1291 | 1021 | 1336 | 101 | 50.5 | 63.0 |
|  | August | 1094 | 1052 | 1480 | 68.6 | 61.9 | 57.9 |
|  | October | 872 | 1311 | 1479 | 68.0 | 56.8 | 61.2 |
|  | December | 993 | 1320 | 1483 | 70.8 | 62.3 | 68.1 |
|  | annual composite sample | 1099 | 1192 | 1406 | 73.7 | 63.7 | 66.7 |
| North Sea Eckwarderhoerne | February | 1762 | 1946 | 1824 | 118 | 70.9 | 71.7 |
|  | April | 1574 | 2056 | 2026 | 119 | 76.6 | 81.3 |
|  | June | 1562 | 1769 | 1834 | 126 | 79.8 | 92.8 |
|  | August | 1223 | 1346 | 1500 | 121 | 82.9 | 90.8 |
|  | October | 1487 | 1688 | 1706 | 95.4 | 80.3 | 77.9 |
|  | December | 1569 | 1636 | 1626 | 85.6 | 74.8 | 73.9 |
|  | annual composite sample | 1541 | 1620 | 1688 | 96.5 | 82.3 | 80.3 |
| Baltic Sea Darsser Ort | June | 1326 | 1004 | 2230 | 81.3 | 42.3 | 33.7 |
|  | November/December | 1572 | 1061 | 1201 | 42.9 | 68.5 | 56.0 |
|  | annual composite sample | 1432 | 1014 | 1515 | 64.2 | 53.4 | 42.3 |

Table S7: Pb-concentrations (ng g-1 dry weight dw and wet weight ww) of monthly samples and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea sites. For comparison: BAC and EC maximum food level used by HELCOM and OSPAR (wet weight based data without respiratory water).

| sampling site | Monthly sample or annual pool sample | Pb - HELCOM: 1300 ng g-1 dry weight; BAC based |  |  | Pb - OSPAR: 1500 ng g-1 wet weight; EC maximum food level based |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Year 2013 | Year 2015 | Year 2017 | Year 2013 | Year 2015 | Year 2017 |
| North Sea Koenigshafen | February | 1115 | 1230 | 1359 | 81.8 | 80.3 | 80.5 |
|  | April | 1074 | 1298 | 1313 | 92.7 | 90.4 | 80.4 |
|  | June | 905 | 1334 | 1443 | 70.6 | 66.0 | 68.0 |
|  | August | 818 | 1348 | 1683 | 51.3 | 79.3 | 65.9 |
|  | October | 687 | 1195 | 1498 | 53.5 | 51.8 | 62.0 |
|  | December | 869 | 1373 | 1645 | 61.9 | 64.8 | 75.5 |
|  | annual composite sample | 851 | 1291 | 1501 | 57.1 | 69.0 | 71.2 |
| North Sea Eckwarderhoerne | February | 2264 | 3137 | 1978 | 152 | 114 | 77.8 |
|  | April | 2831 | 3564 | 2997 | 213 | 133 | 120 |
|  | June | 2140 | 3354 | 2300 | 172 | 151 | 116 |
|  | August | 1263 | 2507 | 1750 | 125 | 154 | 106 |
|  | October | 2187 | 2323 | 2119 | 140 | 111 | 96.8 |
|  | December | 1965 | 2705 | 2855 | 107 | 124 | 130 |
|  | annual composite sample | 1919 | 2412 | 2305 | 120 | 123 | 110 |
| Baltic Sea Darsser Ort | June | 1005 | 753 | 1695 | 61.6 | 31.7 | 25.6 |
|  | November/December | 1103 | 631 | 1334 | 30.1 | 40.7 | 62.2 |
|  | annual composite sample | 875 | 655 | 1424 | 39.2 | 34.5 | 39.8 |

Table S8: Hg concentrations (ng g-1 wet weight) of monthly samples and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea site. 
For comparison: BAC and EC maximum food level used by HELCOM and OSPAR and the Water Framework Directive (Directive 2013/39/EU) (wet weight based data without respiratory water).

| sampling site | Monthly sample or annual pool sample | Hg - HELCOM: 20 ng g-1 wet weight; EQS based (WFD); Hg - OSPAR: 20 ng g-1 wet weight; EQS based (WFD) and 500 ng g-1 wet weight EC maximum food level based |  |  |
| --- | --- | --- | --- | --- |
|  |  | Year 2013 | Year 2015 | Year 2017 |
| North Sea Koenigshafen | February | 18.9 | 13.5 | 14.3 |
|  | April | 19.7 | 15.4 | 15.8 |
|  | June | 17.8 | 10.8 | 11.3 |
|  | August | 13.3 | 8.90 | 10.9 |
|  | October | 13.3 | 7.68 | 9.60 |
|  | December | 15.6 | 9.08 | 10.3 |
|  | annual composite sample | 15.5 | 10.7 | 11.0 |
| North Sea Eckwarderhoerne | February | 17.3 | 10.9 | 11.5 |
|  | April | 20.1 | 11.6 | 13.0 |
|  | June | 16.3 | 12.1 | 15.3 |
|  | August | 14.7 | 12.3 | 14.3 |
|  | October | 14.0 | 9.79 | 11.6 |
|  | December | 11.9 | 10.4 | 12.1 |
|  | annual composite sample | 14.9 | 11.6 | 12.3 |
| Baltic Sea Darsser Ort | June | 3.17 | 1.62 | 1.06 |
|  | November/December | 1.18 | 1.55 | 1.87 |
|  | annual composite sample | 2.23 | 1.89 | 1.39 |

Table S9: Comparison of Cd-thresholds used by HELCOM and OSPAR with Cd-concentrations in seasonal and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea sites (wet weight based data without respiratory water).

| sampling site | seasonal or annual composite samples | Comparison with | Year 2013 | Year 2015 | Year 2017 |
| --- | --- | --- | --- | --- | --- |
| North Sea Koenigshafen | seasonal samples | Cd - HELCOM: 960 ng g-1 dry weight; BAC based | all > BAC (except Oct.) | all > BAC | all > BAC |
|  | annual composite sample |  | > BAC | > BAC | > BAC |
| North Sea Eckwarderhoerne | seasonal samples |  | all > BAC | all > BAC | all > BAC |
|  | annual composite sample |  | > BAC | > BAC | > BAC |
| Baltic Sea Darsser Ort | seasonal samples |  | all > BAC | all > BAC | all > BAC |
|  | annual composite sample |  | > BAC | > BAC | > BAC |
| North Sea Koenigshafen | seasonal samples | Cd - OSPAR: 1000 ng g-1 wet weight; EC maximum food level based | all < EC | all < EC | all < EC |
|  | annual composite sample |  | < EC | < EC | < EC |
| North Sea Eckwarderhoerne | seasonal samples |  | all < EC | all < EC | all < EC |
|  | annual composite sample |  | < EC | < EC | < EC |
| Baltic Sea Darsser Ort | seasonal samples |  | all < EC | all < EC | all < EC |
|  | annual composite sample |  | < EC | < EC | < EC |

Table S10: Comparison of Pb-threshold values used by HELCOM and OSPAR with Pb-concentrations in monthly samples and annual composite samples of blue mussels (Mytilus edulis Complex) from sampling sites North Sea and Baltic Sea sites (wet weight based data without respiratory water).

| sampling site | seasonal or annual composite samples | Comparison with | Year 2013 | Year 2015 | Year 2017 |
| --- | --- | --- | --- | --- | --- |
| North Sea Koenigshafen | seasonal samples | Pb - HELCOM: 1300 ng g-1 dry weight; BAC based | all < BAC | Feb., April and Oct. <BAC; June, Aug. and Dec. > BAC | all > BAC |
|  | annual composite sample |  | < BAC | < BAC | > BAC |
| North Sea Eckwarderhoerne | seasonal samples |  | all > BAC | all > BAC | all > BAC |
|  | annual composite sample |  | > BAC | > BAC | > BAC |
| Baltic Sea Darsser Ort | seasonal samples |  | all < BAC | all < BAC | all > BAC |
|  | annual composite sample |  | < BAC | < BAC | > BAC |
| North Sea Koenigshafen | seasonal samples | Pb - OSPAR: 1500 ng g-1 wet weight; EC maximum food level based | all < EC | all < EC | all < EC |
|  | annual composite sample |  | < EC | < EC | < EC |
| North Sea Eckwarderhoerne | seasonal samples |  | all < EC | all < EC | all < EC |
|  | annual composite sample |  | < EC | < EC | < EC |
| Baltic Sea Darsser Ort | seasonal samples |  | all < EC | all < EC | all < EC |
|  | annual composite sample |  | < EC | < EC | < EC |

Table S11: Comparison of Hg-thresholds used by HELCOM, OSPAR and the WFD with THg-concentrations in monthly and annual composite samples of blue mussels (Mytilus edulis Complex) from North Sea and Baltic Sea sites (wet weight based data without respiratory water).

| sampling site | seasonal or annual composite samples | Comparison with | Year 2013 | Year 2015 | Year 2017 |
| --- | --- | --- | --- | --- | --- |
| North Sea Koenigshafen | seasonal samples | Hg - HELCOM: 20 ng g-1 wet weight; EQS based (WFD) | all < EQS (except April) | all < EQS | all < EQS |
|  | annual composite sample |  | < EQS | < EQS | < EQS |
| North Sea Eckwarderhoerne | seasonal samples |  | all < EQS | all < EQS | all < EQS |
|  | annual composite sample |  | < EQS | < EQS | < EQS |
| Baltic Sea Darsser Ort | seasonal samples |  | all < EQS | all < EQS | all < EQS |
|  | annual composite sample |  | < EQS | < EQS | < EQS |
| North Sea Koenigshafen | seasonal samples | Hg - OSPAR: 20 ng g-1 wet weight; EQS based (WFD) and 500 ng g-1 wet weight EC maximum food level based | all < EC and EQS (except April) | all < EC and EQS | all < EC and EQS |
|  | annual composite sample |  | < EC and EQS | < EC and EQS | < EC and EQS |
| North Sea Eckwarderhoerne | seasonal samples |  | all < EC and EQS | all < EC and EQS | all < EC and EQS |
|  | annual composite sample |  | < EC and EQS | < EC and EQS | < EC and EQS |
| Baltic Sea Darsser Ort | seasonal samples |  | all < EC and EQS | all < EC and EQS | all < EC and EQS |
|  | annual composite sample |  | < EC and EQS | < EC and EQS | < EC and EQS |

Figure S1: Biometrical parameters of blue mussels (Mytilus edulis Complex) from the sampling sites of the German environmental specimen bank in the North Sea (Eckwarderhoerne, Koenigshafen) and the Baltic Sea (Darsser Ort). Shown are data of monthly samples and of annual composite samples of the respective year (dark columns).

Figure S2: Long-term trends and seasonal variations in concentrations of metals in blue mussel (Mytilus edulis Complex) sampled at the North Sea site Eckwarderhoerne. Upper row: long-term trends based on annual pool samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on bi-monthly samples from 2013, 2015, and 2017.

Figure S2 (cont.): Long-term trends and seasonal variations in concentrations of metals, metalloids, and TBT in blue mussel (Mytilus edulis Complex) sampled at the North Sea site Eckwarderhoerne. Upper row: long-term trends based on annual pool samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on bi-monthly samples from 2013, 2015, and 2017.

Figure S3: Long-term trends and seasonal variations in concentrations of metals in blue mussel (Mytilus edulis Complex) sampled at the North Sea site Koenigshafen. Upper row: long-term trends based on annual pool samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on bi-monthly samples from 2013, 2015, and 2017.

Figure S3 (cont.): Long-term trends and seasonal variations in concentrations of metals, metalloids, and TBT in blue mussel (Mytilus edulis Complex) sampled at the North Sea site Koenigshafen. Upper row: long-term trends based on annual pool samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on bi-monthly samples from 2013, 2015, and 2017.

Figure S4: Long-term trends and seasonal variations in concentrations of metals in blue mussel (Mytilus edulis Complex) sampled at the Baltic Sea site Darsser Ort. Upper row: long-term trends based on annual pool samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on six-month samples from 2013, 2015, and 2017.

Figure S4 (cont.): Long-term trends and seasonal variations in concentrations of metals, metalloids, and TBT in blue mussel (Mytilus edulis Complex) sampled at the Baltic Sea site Darsser Ort. Upper row: long-term trends based on annual pool samples (continuous line = significant linear or non-linear trend; dashed line = trend not significant; p = 0.05); lower row: seasonal variation based on six-month samples from 2013, 2015, and 2017.