Table A5: Current available aquatic invertebrates test guidelines

| Main study parameters compared |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Acute |  | Chronic |  |  |  |  |
| Parameter | OECD 202 Daphnia sp. Acute Immobilisation Test | OECD 235 Chironomus sp., Acute Immobilisation Test | OPPTS 885.4240 Freshwater Aquatic Invertebrate Testing | OECD 211 Daphnia magna Reproduction Test | OECD 218 Sediment-Water Chironomid Toxicity Using Spiked Sediment | OECD 219 Sediment-Water Chironomid Toxicity Using Spiked Water | OECD 233 Sediment-Water Chironomid Life-Cycle Toxicity Test Using Spiked Water or Spiked Sediment |
| Species | Daphnia sp., (D. magna is preferred) | Chironomus sp., (C. riparius is preferred) | one species of benthic invertebrate; Additionally: one planktonic species (usually D. magna) in case of direct aquatic exposure | Daphnia magna | Chironomus sp., (C. riparius is preferred) | Chironomus sp., (C. riparius is preferred) | Chironomus sp., (C. riparius is preferred) |
| Controls | Non-dosed control, solvent control if relevant | Non-dosed control, solvent control if relevant | Non-dosed control, sterile filtrate, (note: attenuated control not mentioned) | Non-dosed control, solvent control if relevant | Non-dosed control, solvent control if relevant | Non-dosed control, solvent control if relevant | Non-dosed control, solvent control if relevant |
| Test conditions | (semi-) static or flow-through | (semi-) static or flow-through | (semi-)static | semi-static or flow-through | static, in exceptional cases semi-static or flow-through | static, in exceptional cases semi-static or flow-through | static |
| Test duration (exposure period) | 48 hours | 48 hours | 21 days; should continue until recovery of effects if pathogenicity and/or toxicity are apparent at the 21st day | 21 days | 28 days | 28 days | approx. 44 days (for C. riparius and C. yoshimatsui); approx. 100 days (for C. dilutes). |
| Biological endpoints | EC50 (immobilisation) | EC50 (immobilisation) | LC50 (mortality) | NOEC, ECx: - reproductive output of daphnia - growth (e.g., length) - survival | NOEC, ECx: - total number of adults emerged - time to emergence | NOEC, ECx: - total number of adults emerged - time to emergence | NOEC, ECx: - total number of adults emerged (for both 1st and 2nd generations), - development rate (for both 1st and 2nd generations), - sex ratio |
| Validity criteria indicated | Yes | Yes | No | Yes | Yes | Yes | Yes |
| Fit for purpose? | No. Reasoning: - Study duration too short to detect pathogenic effects | No. Reasoning: - Study duration too short to detect pathogenic effects | Yes, with few adaptations (extended study descriptions, list of recommended test species, increased food levels, possible inclusion of filtered test item groups, validity criteria) | Yes, with adaptations (e.g., attenuated/sterile filtrate controls, CFU counts, increased food levels, possible inclusion of filtered test item groups) | Yes, with adaptations (e.g., attenuated/sterile filtrate controls, CFU counts) | Yes, with adaptations (e.g., attenuated/sterile filtrate controls, CFU counts) | Yes, with adaptations (e.g., attenuated/sterile filtrate controls, CFU counts) |