Supplementary Materials S1a - LOD Data Imputation

Results below the limit of detection in the England GCMS/LCMS dataset are not reported (EA, 2024a). As such there is potentially a large tranche of missing <LOD data from any assessment of the UK monitoring dataset which may skew the assessment statistics towards higher concentrations given these “absent concentrations” are not captured. This limitation may also affect other UK nation datasets where the England laboratory facilities are used by the other nations either for specific compounds or additional capacity (EIONET, 2016).

Only the data missing from the England dataset was imputed as this issue was confirmed by the EA through email correspondence. They also noted that they had included the dates when different analytes had been added to and periods when excluded from, their analytical suite (see EA, 2024b) to the documentation distributed with their GCMS/LCMS dataset. They confirmed by email correspondence that after this date (07/04/2014 for IMI LCMS dataset), for a given sampling occasion, where a result for an analyte is not made available it is reasonable to assume that this is because the result was <LOD (the LOD being 1 ng/L for IMI LCMS) and not reported by their systems.

The algorithm for imputing the missing data was as such:

Inputdata = England LCMS data (complete dataset)

Get list of unique sampling sites in Inputdata

For each sampling site in Inputdata

Get list of unique sampling occasions for sampling site

For each sampling occasion at that sampling site

Get list of analytes for which results are published for the site/sampling occasion

If list of analytes for that site/sampling occasion does not include IMI then

IMI Result = <1 ng/L for that site and sampling occasion

End if

Next sampling occasion

Next sampling site

References

EA, 2024a. England Environment Agency (EA) README documentation associated with dataset https://www.data.gov.uk/dataset/0c63b33e-0e34-45bb-a779-16a8c3a4b3f7/water-quality-monitoring-data-gc-ms-and-lc-ms-semi-quantitative-screen, accessed October 2024.

EA, 2024b. England Environment Agency (EA) LCMS EA NLS Target Database documentation associated with dataset https://www.data.gov.uk/dataset/0c63b33e-0e34-45bb-a779-16a8c3a4b3f7/water-quality-monitoring-data-gc-ms-and-lc-ms-semi-quantitative-screen, accessed October 2024.

EIONET (European Environment Information and Observation Network) (2016). Description of the watch list monitoring stations and the monitoring  strategy for surface waters sampled in the Administrations of United Kingdom. Retrieved August 2024 from   https://cdr.eionet.europa.eu/gb/eea/wise_soe/wise4/envwfizrq/Summary_of_WFD_Watch_List_sampling_in_the_UK__Dec_2016__final.pdf