Supplementary Material

Table S1. Summary of Quality Assurance and Quality Control (QA/QC) Performance Parameters for GC–MS Analysis of 16 Priority PAHs in Environmental Samples from Ibaa, Niger Delta

| Analyte (PAH) | LOD (µg/kg d.w.) | LOQ (µg/kg d.w.) | Linearity (R²) | % Recovery (Mean ± SD) | RSD (%) | Surrogate Used | Comments / Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nap | 0.010 | 0.035 | 0.999 | 92 ± 5 | 7.4 | Naphthalene-d8 | Meets IUPAC criteria (3× S/N for LOD; 10× S/N for LOQ) |
| Acy | 0.012 | 0.040 | 0.997 | 88 ± 6 | 8.1 | Naphthalene-d8 | Within USEPA 8270D recovery range |
| Ace | 0.013 | 0.042 | 0.998 | 95 ± 4 | 6.3 | Naphthalene-d8 | Consistent linear response (0.01–10 µg/mL) |
| Flu | 0.014 | 0.045 | 0.996 | 91 ± 5 | 7.8 | Phenanthrene-d10 | Triplicate analyses RSD ≤ 10 % |
| Phe | 0.015 | 0.050 | 0.995 | 89 ± 7 | 8.9 | Phenanthrene-d10 | Surrogate recovery 82–108 % |
| Ant | 0.014 | 0.049 | 0.996 | 87 ± 6 | 9.1 | Phenanthrene-d10 | Retention time ±0.05 min, ion ratio ±20 % |
| FluA | 0.013 | 0.046 | 0.998 | 93 ± 4 | 6.7 | Chrysene-d12 | Calibration check within ±10 % of midpoint |
| Pyr | 0.013 | 0.046 | 0.999 | 97 ± 3 | 5.9 | Chrysene-d12 | Five-point calibration curve applied |
| BaA | 0.015 | 0.050 | 0.998 | 90 ± 6 | 7.2 | Chrysene-d12 | Precision within method criteria |
| Chry | 0.015 | 0.050 | 0.999 | 88 ± 5 | 7.5 | Chrysene-d12 | Field and procedural blanks < LOQ |
| BbF | 0.014 | 0.048 | 0.997 | 86 ± 6 | 8.4 | Chrysene-d12 | Stable instrument response (±10 %) |
| BkF | 0.014 | 0.048 | 0.998 | 90 ± 5 | 7.6 | Chrysene-d12 | Meets method precision limits |
| BaP | 0.013 | 0.047 | 0.999 | 94 ± 4 | 6.8 | Chrysene-d12 | Target compound for risk evaluation |
| IcdP | 0.012 | 0.045 | 0.996 | 89 ± 6 | 8.5 | Chrysene-d12 | Retention and ion ratio confirmed with standards |
| DahA | 0.013 | 0.046 | 0.997 | 91 ± 4 | 7.1 | Chrysene-d12 | Consistent calibration (R² ≥ 0.995) |
| BghiP | 0.012 | 0.044 | 0.999 | 93 ± 5 | 6.9 | Chrysene-d12 | Within USEPA 8270D performance range |