Additional file 4

Format: .docx

Title:  Table S3, Mass Spectrophotometer transitions

Description: The eluent from LC was passed onto the electrospray source of an amaZon ETD Ion Trap operated in MRM mode with the following transitions. The concentrations of components in samples were calculated by comparison with external calibration curves of authentic compounds.

| Analyte | MS2 conditions |  |  | Fragments used for quantification (m/z) |
| --- | --- | --- | --- | --- |
|  | Parent ion/m/z | Isolation width/m/z | Amplitude |  |
| Serine | 106.1 | 1 | 0.5 | 60.2 |
| S-adenosyl methionine | 298 | 1 | 0.6 | 135.8 |
| Cystathionine | 222.8 | 1 | 0.35 | 133.8 |
| Homocysteine | 135.9 | 1 | 0.5 | 89.9 |