## Supplementary Methods

UDiTaS protocol

1. Equipment

            - 96 well PCR plates (ThermoFisher, #AB-1400-L)

            - Plate Seals – EmporeTM Sealing Tape (Sigma-Aldrich, #66881-U)

            - 96/384-well plate spinner (ThermoFisher, 14-100-143)

            - Qubit 3.0 Fluorometer (ThermoFisher, #Q33216)

            - Veriti Thermal Cycler, 96-well (ThermoFisher, #4375786)

            - Agilent 2100 BioAnalyzer (Agilent, #G2939AA); Agilent High Sensitivity DNA Kit (# 5067-4626)

            - BluePippin (Sage Science)

- Bench Cleanup

- DNAse AWAY spray (Molecular BioProducts #7002)

- Kimwipes (Kimberly Clark #34256)

- Spray down bench and all pipettors. Wipe dry

- Adapter oligo annealing

3.1 Materials:

- 96 well PCR plate

- 96 Thermal Cycler

- Top oligo (IDT):  refer to appendix A for the 8-possible top oligos with the full length Illumina i5 adapter sequence

- Bottom oligo (IDT): refer to appendix A for single bottom oligo

3.2 Oligo annealing set up (Refer to Appendix A for adapter oligo info)

- Set up in a 96 well PCR plate

- For each 100 uL reaction, add 50 uL the top and bottom oligo

| Reagents | 100 µL reaction | Final conc. (µM) |
| --- | --- | --- |
| Top oligo (100 µM) | 50 µL | 50 |
| Bottom oligo (100 µM) | 50 µL | 50 |
| Total vol. | 100 µL |  |

- Set the PCR thermocycler 95° C for 2 min and slow ramp down to 25° C (~12 hours)

- Aliquot into labeled 0.7 mL tubes in single use volumes (15 or 30 µL)

- Store at -20° C

- Transposome assembly

4.1 Materials:

- DNA LoBind Tube 1.5 mL tubes (Eppendorf, #022431021)

- pTXB1-Tn5 vector was synthesized by GeneWiz.  Academic institutes can purchase the vector from addgene (https://www.addgene.org/60240/). The vector was expressed according to Picelli, S. et al. Genome Res. 24, 2033–2040 (2014). Final protein concentration was 1.85 mg/ml and stored at -20°C in 1 mL aliquots:

  - unassembled Tn5 storage buffer: 55%glycerol stock: 100% glycerol + 2x Tn5 dialysis buffer

  - 2x Tn5 dialysis buffer: 100 HEPES-KOH at pH7.2, 0.2M NaCl, 0.2mM EDTA, 2mM DTT, 0.2% Triton X-100, 20% glycerol

- Annealed oligos from step 3

4.2 Transposome assembly:

- Keep Tn5 enzyme at -20°C until ready to use. In LoBind 1.5mL Eppendorf tubes, add pre-annealed oligo and transposome.

| Reagents | % reaction volume | 100 µL reaction | 200 µL reaction |
| --- | --- | --- | --- |
| Tn5 enzyme (1.85mg/mL) | 0.857 | 85.7 µL | 171.4 µL |
| Pre-annealed oligos (100µM) | 0.143 | 14.3 µL | 28.6 µL |
| Total volume (µL) | 1 | 100 µL | 200 µL |

- Incubate at room temperature for 60 minutes

- Test transposome activity by setting up tagmentation reactions (step 5)

- Store aliquots in -20° C

- Tagmentation reaction

5.1 Materials:

- 96 well PCR plate

- Human genomic DNA, 10 ng/µL (Promega, # G3041)

- 5X TAPS-DMF: Boston BioProducts (custom buffer)

  - 50mM TAPS-NaOH @ pH 8.5, 25 mM MgCl2 and 50% DMF (dimethylformamide)

- Transposome (8 different transposome can be generated using the bottom oligo with each of the i5 barcoded top oligos listed in Appendix A)

- DNA clean & concentrator-5 columns (Zymo Research, # D4013)

5.2 Tagmentation setup:

NOTE: Accurate quantification of DNA is essential for reproducible tagmentation resulting in ~1.5-2 kb products (see Appendix B for tagmentation profile)

- Samples are processed in small batches (12) to reduce any premature activity of the transposase before incubation.

- Amount of transposome may vary depending on your activity assay

- For tagmentation activity assay: set up reactions below doing a titration of the transposome keeping the gDNA constant (50ng). Then clean up with Zymo column and run on the Bioanalyzer to check the size of the library.

- Preheat the thermocycler to 55° C

- Make a mastermix with water, 5x TAPS-DMF buffer and transposome

- Mix well but gently

| Reagents | 1 reaction |
| --- | --- |
| Nuclease free water | 4 µL |
| 5x TAPS-DMF | 4 µL |
| Genomic DNA (5 ng/µL) | 10 µL |
| Assembled transposome | 2 µL |
| Total reaction (µL) | 20 µL |

- Aliquot 10 µL of mix to each well

- Add 10 µL of gDNA (50 ng total) to each well

- Pipet up and down to mix slowly. Avoid bubbles as possible

- Seal plate and spin briefly

- Incubate the plate in thermocycler at 55° C for 7 minutes

- Place on ice

  - Tagmentation reaction clean up using Zymo columns

- Remove plate from ice and add 100 µL of DNA binding buffer (5x reaction volume)

- Mix briefly by pipetting up and down or vortex briefly

- Transfer mixture to Zymo-Spin column in a 2mL collection tube

- Centrifuge for 30 sec (13,000xg); do not need to discard, flow through volume is low

- Add 200 ul DNA Wash Buffer to the column.

- Centrifuge for 30 sec (13,000xg); do not need to discard, flow through volume is low

- Add another 200 µL DNA Wash Buffer to the column.

- Centrifuge for 30 sec (15,000xg)

- Place filter column in a clean 1.5 mL tubes

- Add 11 µL of nuclease free water to each column

- Incubate at room temperature for 1 minute

- Centrifuge for 30 sec (15,000xg)

- Round 1 PCR

6.1 Materials:

- 96 well PCR plate

- 2x Platinum SuperFi PCR Master mix (ThermoFisher, # 12358050)

- 5M Tetramethylammonium chloride solution (TMAC; Sigma, # T3411-500ML)

- VEGFA gene specific primer (GSP) for assay validation.  This can be replaced with gene specific primer of interest:

  - VEGFA_chr6_43737470_F:  IDT; 5’-GTGACTGGAGTTCAGACGTGTGCTCTTCCGATCTCAACTCCAGTCCCAAATATGTAGCTGTT-3’; Illumina i7 tail in red

- i5 amplification primer: IDT: 5’-AATGATACGGCGACCACCGAGATCTACAC- 3’

- Tagmented DNA (from step 5)

6.2 Round 1 PCR setup:

| Reagents | 1 reaction | Final concentration |
| --- | --- | --- |
| Water (SuperFi kit) | To 25 µL |  |
| 2x SuperFi master mix | 12.5 µL | 1x |
| TMAC (0.5M) | 1.5 µL | 30 mM |
| GSP (10 µM) | 1.25 µL | 500 nM |
| i5 primer | 0.625 µL | 250 nM |
| Tagmented DNA | 9 µL |  |
| Total reaction | 25 µL |  |

- Make a master mix for each gene specific primer (GSP), add components as listed aboveAdd 15.875 µL of the mix into the wells of the 96 well plate

- Add 9.125 µL of the Zymo cleaned tagmentation products to each well

- Mix well and slowly to avoid bubbles

- Seal plate and spin briefly

- Place the plate in thermocycler with the following program:

| Step | Temp (oC) | Time | Cycle # |
| --- | --- | --- | --- |
| Initial Denaturation | 98 | 2 min | 1 |
| Denature | 98 | 10 sec | 12 |
| Anneal | 65 | 10 sec |  |
| Extend | 72 | 90 sec |  |
| Final Extension | 72 | 5 min | 1 |
| Hold | 4 |  | 1 |

6.3 Clean up using Agencourt Ampure XP SPRI beads (0.9X reaction volume)

6.3.1 Materials:

- Agencourt AMPure XP (#A63882)

- DynaMag™-96 Side Magnet (#12331D)

- 200 Proof Pure Ethanol (KOPTEC #V1016)

- DNase-RNase Free Water Non-DEPC Treated (Boston Bioproducts #R-100DR)

6.3.2 Ampure XP SPRI beads clean up

- Add 22.5 µL of Ampure XP beads to each well (25 µL x 0.9x reaction volume = 22.5 µL)

- Mix beads with reaction by pipetting up and down 15 times

- Let stand in room temperature for 5 minutes

- Place the plate on the magnet and let stand for 5 minutes or until the solution is clear

- Remove and discard supernatant

- Wash two times with 200 µL of 70% ethanol (made fresh before use). Let the ethanol sit in the beads for 30 seconds then remove ethanol.

- Let the beads air dry for 3-5 minutes (until the pellet is not glossy).

- Elute with 11 µL of water (use nuclease free water)

- Round 2 PCR - refer to Appendix A for barcoded PCR 2 primers

7.1 Materials:

- 2x Platinum SuperFi PCR Master mix (Thermo Fisher #12358050)

- i7 barcoded primer (see Appendix A)

•	i5 amplification primer: IDT: 5’-AATGATACGGCGACCACCGAGATCTACAC- 3’

- Round 1 PCR product

  - Round 2 PCR set up

- In a new 1.5mL tube, make a master mix with water, P5 primer and 2X Platinum SuperFi PCR   master mix

| Reagents | 1 reaction | Final concentration |
| --- | --- | --- |
| Water (SuperFi kit) | 5 µL |  |
| 2x SuperFi master mix | 25 µL | 1x |
| i5 amplification primer (10 µM) | 2.5 µL | 500 nM |
| 1st round PCR product | 10 µL |  |
| i7 barcode primer (100 µM) | 7.5 µL | 15 µM |
| Total reaction | 50 µL |  |

- Aliquot 32.5 µL into each well with round 1 PCR product

- Add 7.5 µL  of unique i7 barcode primer to each well

- Add 10 µL of purified Round 1 PCR product to each well

- Mix well

- Place plate in thermocycler and run the following program:

| Step | Temp (oC) | Time | Cycle # |
| --- | --- | --- | --- |
| Initial Denaturation | 98 | 2 min | 1 |
| Denature | 98 | 10 sec | 15 |
| Anneal | 65 | 10 sec |  |
| Extend | 72 | 90 sec |  |
| Final Extension | 72 | 5 min | 1 |
| Hold | 4 |  | 1 |

7.3 Clean up using Agencourt Ampure XP SPRI bead (0.9X reaction volume)

7.3.1 Materials:

- Agencourt AMPure XP (#A63882)

- DynaMag™-96 Side Magnet (#12331D)

- 200 Proof Pure Ethanol (KOPTEC #V1016)

- DNase-RNase Free Water Non-DEPC Treated (Boston Bioproducts #R-100DR)

    - Round 2 PCR Ampure Cleanup:

- Add 45 µL of Ampure XP beads to each well (50 µL x 0.9x = 45 µL)

- Mix beads with reaction by pipetting up and down 15 times.

- Let stand in room temperature for 5 minutes.

- Place the plate on the magnet and let stand for 5 minutes or until the solution is clear.

- Remove and discard supernatant.

- Wash two times with 200 µL of 70% ethanol (made fresh before use). Let the ethanol sit in the beads for 30 seconds then remove ethanol.

- Let the beads air dry for 3-5 minutes (until the pellet is not glossy).

- Elute with 25 µL of 1x low TE (10 mM Tris, 0.1 mM EDTA)

- QC with Bioanalyzer and Qubit

8.1 Bioanalyzer QC following manufacturer’s specifications

http://www.agilent.com/cs/library/usermanuals/Public/G2938-90322_HighSensitivityDNA_QSG.pdf

8.2 Qubit Quantification following manufacturer’s specifications for the dsDNA High Sensitivity kit

https://tools.thermofisher.com/content/sfs/manuals/Qubit_dsDNA_HS_Assay_UG.pdf

- Pool and concentrate samples

9.1 Materials

- Agencourt AMPure XP (#A63882)

- DynaMag™-96 Side Magnet (#12331D)

- 200 Proof Pure Ethanol (KOPTEC #V1016)

- DNase-RNase Free Water Non-DEPC Treated (Boston Bioproducts #R-100DR)

9.2 Pooling and SPRI clean up

- Calculate the molar amount of each product based on the concentration from qubit and the size from the bioanalyzer.

- Pool equimolar amounts of each product into a 1 mL Eppendorf tube.

- Clean and concentrate pool using Agencourt Ampure XP SPRI beads (0.9x)

  - Add 0.9 times the reaction volume of Ampure XP beads

  - Mix beads with reaction by pipetting up and down 15 times

  - Let stand in room temperature for 5 minutes

  - Place the plate on the magnet and let stand for 5 minutes or until the solution is clear

  - Remove and discard supernatant

  - Wash two times with 200 µL of 70% ethanol (made fresh before use). Let the ethanol sit in the beads for 30 seconds then remove ethanol

  - Let the beads air dry for 3-5 minutes (until the pellet is not glossy)

  - Elute with 35 µL of 1x low TE

  - Set 2 µL aside for QC. Proceed with size selection with remaining

- Size selection using Sage BluePippin following manufacturer’s specifications

10.1 Reagents

    - BluePippin Gel cassettes: 1.5% agarose, 250bp – 1.5kb (Sage Science)

10.2 Size Selection (400-850bp)

- Create size selection protocol for 400-850bp on all lanes

- Load only 1.5 µg of product per lane (run multiples lanes if necessary)

11. Clean up size selected library using Ampure XP SPRI beads (0.9x reaction volume)

11.1 Materials:

- Agencourt AMPure XP (#A63882)

- DynaMag™-96 Side Magnet (#12331D)

- 200 Proof Pure Ethanol (KOPTEC #V1016)

- DNase-RNase Free Water Non-DEPC Treated (Boston Bioproducts #R-100DR)

11.2 Ampure XP setup

- Add 0.9 times reaction volume of size selected product from BluePippin

- Mix beads with reaction by pipetting up and down 15 times

- Let stand in room temperature for 5 minutes

- Place the plate on the magnet and let stand for 5 minutes or until the solution is clear

- Remove and discard supernatant

- Wash two times with 200 µL of 70% ethanol (made fresh before use). Let the ethanol sit in the beads for 30 seconds then remove ethanol

- Let the beads air dry for 3-5 minutes (until the pellet is not glossy)

- Elute with 25 ul of 1x low TE

- Quantification of final size selected library

  - KAPA library quant qPCR kit

    - KAPA library quant kit (illumina) Universal qPCR mix; cat# KK4824

    - Vortex to mix all reagents well

    - Dilute sample according to the table below:

| Library dilution | dilution factor |
| --- | --- |
| 1 | 1:10 |
| 2 | 1:100 |
| 3 | 1:1,000 |
| 4 | 1:10,000 |
| 5 | 1:100,000 |
| 6 | 1:1,000,000 |
| 7 | 1:2,000,000 |

- Set up KAPA library quant qPCR according to the manual

- Add 6 ul of KAPA master mix into each well

- Use 4 ul of each dilution or standard in triplicate

- Place on the qPCR instrument (2 hours)

| Temp (oC) | Time | Cycle # |
| --- | --- | --- |
| 95 | 5 min | 1 |
| 95 | 30 sec | 35 |
| 60 | 45 |  |
| Melt curve |  |  |

- Use the KAPA analysis excel sheet provided online to copy the Cq values and the size of the library from the bioanalyzer to calculate the nM concentration of the sample

  - QC final pooled library before and after size selection on Agilent Bioanalyzer

- Dilute all samples 1:10 before running on the Bioanalzyer high sensitivity chip

- Obtain the median size of the library

Example:

13. MiSeq Sequencing

13.1 Follow manufacturer’s specifications for loading library on MiSeq

- Load 10 pM denatured library with 15% PhiX

- Generate MiSeq run sheet:

  - Read 1: 300 bases

  - Index read 1: 8 bases

  - Read 2: 300 bases

  - Index read 2: 18 bases

Appendix A:

Adapter oligo sequences used to anneal (step 3) and complexing with the Tn5: Oligos were ordered from Integrated DNA Technologies (IDT)

| Oligo Name | sequence 5'-3' |
| --- | --- |
| Tn5-A bottom | [Phos]CTGTCTCTTATACA[ddC] |
| i5_N501_UMI_Tn5-A | AATGATACGGCGACCACCGAGATCTACACTAGATCGCNNNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |
| i5_N502_UMI_Tn5-A | AATGATACGGCGACCACCGAGATCTACACCTCTCTATNNNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |
| i5_N503_UMI_Tn5-A | AATGATACGGCGACCACCGAGATCTACACTATCCTCTNNNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |
| i5_N504_UMI_Tn5-A | AATGATACGGCGACCACCGAGATCTACACAGAGTAGANNNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |
| i5_N505_UMI_Tn5-A | AATGATACGGCGACCACCGAGATCTACACGTAAGGAGNNNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |
| i5_N506_UMI_Tn5-A | AATGATACGGCGACCACCGAGATCTACACACTGCATANNNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |
| i5_N507_UMI_Tn5-A | AATGATACGGCGACCACCGAGATCTACACAAGGAGTANNNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |
| i5_N508_UMI_Tn5-A | AATGATACGGCGACCACCGAGATCTACACCTAAGCCTNNNNNNNNNNTCGTCGGCAGCGTCAGATGTGTATAAGAGACAG |

| Oligo Name | i5 Barcode Sequence | i5 Barcode READ (Tx10 for UMI) |
| --- | --- | --- |
| i5_N501_UMI_Tn5-A | TAGATCGC | TAGATCGCTTTTTTTTTT |
| i5_N502_UMI_Tn5-A | CTCTCTAT | CTCTCTATTTTTTTTTTT |
| i5_N503_UMI_Tn5-A | TATCCTCT | TATCCTCTTTTTTTTTTT |
| i5_N504_UMI_Tn5-A | AGAGTAGA | AGAGTAGATTTTTTTTTT |
| i5_N505_UMI_Tn5-A | GTAAGGAG | GTAAGGAGTTTTTTTTTT |
| i5_N506_UMI_Tn5-A | ACTGCATA | ACTGCATATTTTTTTTTT |
| i5_N507_UMI_Tn5-A | AAGGAGTA | AAGGAGTATTTTTTTTTT |
| i5_N508_UMI_Tn5-A | CTAAGCCT | CTAAGCCTTTTTTTTTTT |

P5 / i5 oligo sequence for Round 1 and 2 PCR:

| Oligo Name | sequence 5'-3' |
| --- | --- |
| i5 | AATGATACGGCGACCACCGAGATCTACAC |

i7 Barcoded oligo sequences for Round 2 PCR:

| Oligo Name | Sequence (5'-3') | i7 BC Seq | i7 BC READ |
| --- | --- | --- | --- |
| i7_N701_SBS12 | CAAGCAGAAGACGGCATACGAGATAGCGGAATGTGACTGGAGTTCAGACGTGT | AGCGGAAT | ATTCCGCT |
| i7_N702_SBS12 | CAAGCAGAAGACGGCATACGAGATGATCATGCGTGACTGGAGTTCAGACGTGT | GATCATGC | GCATGATG |
| i7_N703_SBS12 | CAAGCAGAAGACGGCATACGAGATAAGACGGAGTGACTGGAGTTCAGACGTGT | AAGACGGA | TCCGTCTT |
| i7_N704_SBS12 | CAAGCAGAAGACGGCATACGAGATCGAGTCCTGTGACTGGAGTTCAGACGTGT | CGAGTCCT | AGGACTCG |
| i7_N705_SBS12 | CAAGCAGAAGACGGCATACGAGATTCCTCAGGGTGACTGGAGTTCAGACGTGT | TCCTCAGG | CCTGAGGA |
| i7_N706_SBS12 | CAAGCAGAAGACGGCATACGAGATGTACGGATGTGACTGGAGTTCAGACGTGT | GTACGGAT | ATCCGTAC |
| i7_N707_SBS12 | CAAGCAGAAGACGGCATACGAGATCATCTCTCGTGACTGGAGTTCAGACGTGT | CATCTCTC | GAGAGATG |
| i7_N710_SBS12 | CAAGCAGAAGACGGCATACGAGATGTCGGAGCGTGACTGGAGTTCAGACGTGT | GTCGGAGC | GCTCCGAC |
| i7_N711_SBS12 | CAAGCAGAAGACGGCATACGAGATACGGAGAAGTGACTGGAGTTCAGACGTGT | ACGGAGAA | TTCTCCGT |
| i7_N712_SBS12 | CAAGCAGAAGACGGCATACGAGATAGGAGATGGTGACTGGAGTTCAGACGTGT | AGGAGATG | CATCTCCT |
| i7_N714_SBS12 | CAAGCAGAAGACGGCATACGAGATAGTACTCGGTGACTGGAGTTCAGACGTGT | AGTACTCG | CGAGTACT |
| i7_N715_SBS12 | CAAGCAGAAGACGGCATACGAGATGGACTCTAGTGACTGGAGTTCAGACGTGT | GGACTCTA | TAGAGTCC |

Appendix B: Tagmentation Profile viewed on the Agilent Bioanalyzer

50 ng Promega gDNA using 2 µL of transposome in 20 µL tagmentation reaction and incubated in a preheated thermal cycler at 55oC for 7 min

AMP-seq protocol

A modified AMP-seq protocol previously described (Tsai, S. Q. et al. GUIDE-seq enables genome-wide profiling of off-target cleavage by CRISPR-Cas nucleases. Nat. Biotechnol. 33, 187–197 (2014)) was used to generate the AMP-seq libraries.

- Equipment

            - 96 well PCR plates (ThermoFisher, #AB-1400-L)

            - Plate Seals – EmporeTM Sealing Tape (Sigma-Aldrich, #66881-U)

            - 96/384-well plate spinner (ThermoFisher, 14-100-143)

            - Qubit 3.0 Fluorometer (ThermoFisher, #Q33216)

            - Veriti Thermal Cycler, 96-well (ThermoFisher, #4375786)

            - Agilent 2100 BioAnalyzer (Agilent, #G2939AA); Agilent High Sensitivity DNA Kit (# 5067-4626)

            - BluePippin (Sage Science)

            - Covaris M220 instrument

- Materials:

- End repair Kit (Enzymatics, Y9140-LC-L)

- Ligation Kit: (Enzymatics, L6030-LC-L)

- Platinum Taq: (Invitrogen, 10966-026)

- Nuclease free water

- 1x TE (10mM Tris, 1mM EDTA, pH 8.0)

- Pre-annealed Y adapters

- TetraMethylAmmonium Chloride (TMAC) 5M

- Agencourt AMPure XP (#A63882)

- DynaMag™-96 Side Magnet (#12331D)

- 200 Proof Pure Ethanol (KOPTEC #V1016)

- DNase-RNase Free Water Non-DEPC Treated (Boston Bioproducts #R-100DR)

- AMP-seq adapter sequences – Oligos ordered from IDT

| Y-Adapters | Sequence 5’ 3’ |
| --- | --- |
| OLI2909 | AATGATACGGCGACCACCGAGATCTACACACTGCATANNWNNWNNACACTCTTTCCCTACACGACGCTCTTCCGATC*T |
| OLI2910 | ATGATACGGCGACCACCGAGATCTACACAAGGAGTANNWNNWNNACACTCTTTCCCTACACGACGCTCTTCCGATC*T |
| bottom | [Phos]GATCGGAAGAGC*C*A |

* Phosphorothioate bond

100uM of each top and bottom oligos are heated to 95oC for 1sec, 60oC for 1sec and ramped down 2oC/min to 4oC and hold at 4oC.

- Method:

50 ng  of into mouse genomic DNA containing the CEP290 plasmids (PLA370, PLA367, PLA371, PLA368, PLA372, PLA369) ranging from ~714,000 to 2,200 genome equivalents was used to generate AMP-seq libraries. The DNA (50ng in 130uL total TE buffer) was sheared using the Covaris M220 (shearing conditions- peak power:50, cycles/burst:200, duration:60s, duty factor:10) to obtain 500bp fragments. The sample was concentrated using AmPure XP beads (130uL) following the manufacturer’s specifications and eluted in 15uL 1x TE.

The DNA fragment were end repaired as follows:

| Reagent | Volume/reaction (µL) |
| --- | --- |
| Nuclease free water | 0.5 |
| dNTP (5mM) | 1 |
| SLOW Ligation buffer (10x) | 2.5 |
| End repair mix low concentration | 2 |
| 10x Buffer for Platinum Taq Mg2+ free | 2 |
| Platinum Taq 5U/uL) | 0.5 |
| Fragmented DNA | 14 |
| Total | 22.5 |

End Repair thermocycler program used was: 12°C for 15min, 37°C for 15min; 72°C for 15min; hold at 4°C. Pre-annealed unique Illumina adapters (1uL at 10uM adapters: either OLI2909, OLI2910) were added to each reaction along with 2uL of T4 DNA ligase and incubated on thermal cycler (16°C for 30min, 22°C for 30min, hold at 4°C).  The reaction was cleaned using AmPure XP beads (0.9X volume, 22.5uL) and eluted in 12uL 1x TE buffer.

- Round 1 PCR:

The CEP290 library was amplified with gene specific primer (OLI6062):

| Reagent | Volume/reaction (µL) |
| --- | --- |
| Nuclease free water | 11.15 |
| 10x Buffer for Platinum Taq Mg2+ free | 3 |
| 50mM Mg2+ | 1.2 |
| Platinum Taq (5U/uL) | 0.3 |
| dNTP 10mM | 0.6 |
| TMAC 0.5M | 1.5 |
| i5 amplification primer (10uM) | 0.75 |
| Gene specific primer (10uM) | 1.5 |
| Adapter ligated DNA | 10 |
| Total | 30 |

Cycling conditions:

| Temp | Time | Cycle |
| --- | --- | --- |
| 95 degree C | 5 minutes |  |
| 95 degree C | 30 seconds | 15x |
| 70 degree C, -1 C per cycle | 2 minutes | 15x |
| 72 degree C | 30 seconds | 15x |
| 95 degree C | 30 seconds | 10x |
| 55 degree C | 1 minute | 10x |
| 72 degree C | 30 seconds | 10x |
| 4 degree C | hold |  |

PCR products were cleaned using AmPure XP beads (1.2x volume, 36uL) following the manufacturer’s specifications and eluted in 17uL 1x TE.

- Round 2 PCR:

The CEP290 library was amplified with i7 barcode primers (i7_N701_SBS12, i7_N702_SBS12):

| Reagent | Volume/reaction (µL) |
| --- | --- |
| Nuclease free water | 4.65 |
| 10x Buffer for Platinum Taq Mg2+ free | 3 |
| 50mM Mg2+ | 1.2 |
| Platinum Taq (5U/uL) | 0.3 |
| dNTP 10mM | 0.6 |
| TMAC 0.5M | 1.5 |
| i5 amplification primer (10uM) | 0.75 |
| i7_barcode_SBS12 (10uM) | 3 |
| Round 1 PCR product | 15 |
| Total | 30 |

Cycling conditions:

| Temp | Time | Cycle |
| --- | --- | --- |
| 95 degree C | 5 minutes |  |
| 95 degree C | 30 seconds | 15x |
| 70 degree C, -1 C per cycle | 2 minutes | 15x |
| 72 degree C | 30 seconds | 15x |
| 95 degree C | 30 seconds | 10x |
| 55 degree C | 1 minute | 10x |
| 72 degree C | 30 seconds | 10x |
| 4 degree C | hold |  |

PCR products were cleaned using AmPure XP beads (0.7x volume, 21uL) following the manufacturer’s specifications and eluted in 25uL 1x TE.

- QC with Bioanalyzer and Qubit

8.1 Bioanalyzer QC following manufacturer’s specifications

http://www.agilent.com/cs/library/usermanuals/Public/G2938-90322_HighSensitivityDNA_QSG.pdf8.2 Qubit Quantification following manufacturer’s specifications for the dsDNA High Sensitivity kit

https://tools.thermofisher.com/content/sfs/manuals/Qubit_dsDNA_HS_Assay_UG.pdf

- Pool and concentrate samples

9.1 Materials

- Agencourt AMPure XP (#A63882)

- DynaMag™-96 Side Magnet (#12331D)

- 200 Proof Pure Ethanol (KOPTEC #V1016)

- DNase-RNase Free Water Non-DEPC Treated (Boston Bioproducts #R-100DR)

9.2 Pooling and SPRI clean up

- Calculate the molar amount of each product based on the concentration from qubit and the size from the bioanalyzer.

- Pool equimolar amounts of each product into a 1 mL Eppendorf tube.

- Clean and concentrate pool using Agencourt Ampure XP SPRI beads (0.9x)

  - Add 0.9 times the reaction volume of Ampure XP beads

  - Mix beads with reaction by pipetting up and down 15 times

  - Let stand in room temperature for 5 minutes

  - Place the plate on the magnet and let stand for 5 minutes or until the solution is clear

  - Remove and discard supernatant

  - Wash two times with 200 µL of 70% ethanol (made fresh before use). Let the ethanol sit in the beads for 30 seconds then remove ethanol

  - Let the beads air dry for 3-5 minutes (until the pellet is not glossy)

  - Elute with 25 µL of 1x low TE

  - Set 2 µL aside for QC. Proceed with size selection with remaining

Size selection using Sage BluePippin following manufacturer’s specifications

10.1 Reagents

    - BluePippin Gel cassettes: 1.5% agarose, 250bp – 1.5kb (Sage Science)

10.2 Creating a Size selection protocol for the BluePippin 1.5% internal standard R2 (400-850bp)

http://www.sagescience.com/wp-content/uploads/2011/10/Quick-Guide-BDF1510-marker-R22.pdf

- Create size selection protocol for 400-850bp on all lanes

- Load only 1.5 µg of product per lane (run multiples lanes if necessary)

11. Clean up size selected library using Ampure XP SPRI beads (0.9x reaction volume)

11.1 Materials:

- Agencourt AMPure XP (#A63882)

- DynaMag™-96 Side Magnet (#12331D)

- 200 Proof Pure Ethanol (KOPTEC #V1016)

- DNase-RNase Free Water Non-DEPC Treated (Boston Bioproducts #R-100DR)

11.2 Ampure XP setup

- Add 0.9 times reaction volume of size selected product from BluePippin

- Mix beads with reaction by pipetting up and down 15 times

- Let stand in room temperature for 5 minutes

- Place the plate on the magnet and let stand for 5 minutes or until the solution is clear

- Remove and discard supernatant

- Wash two times with 200 µL of 70% ethanol (made fresh before use). Let the ethanol sit in the beads for 30 seconds then remove ethanol

- Let the beads air dry for 3-5 minutes (until the pellet is not glossy)

- Elute with 25 ul of 1x low TE

- Quantification of final size selected library

  - KAPA library quant qPCR kit

    - KAPA library quant kit (Illumina) Universal qPCR mix; cat# KK4824

    - Vortex to mix all reagents well

    - Dilute sample according to the table below:

| Library dilution | dilution factor | Sample (µL) | Qiagne EB buffer (µL) | Total (µL) |
| --- | --- | --- | --- | --- |
| 1 | 1:10 | 10 | 90 | 100 |
| 2 | 1:100 | 10 | 90 | 100 |
| 3 | 1:1,000 | 10 | 90 | 100 |
| 4 | 1:10,000 | 10 | 90 | 100 |
| 5 | 1:100,000 | 10 | 90 | 100 |
| 6 | 1:1,000,000 | 10 | 90 | 100 |
| 7 | 1:2,000,000 | 50 | 50 | 100 |

- Set up KAPA library quant qPCR according to the manual

- Add 6 ul of KAPA master mix into each well

- Use 4 ul of each dilution or standard in triplicate

- Place on the qPCR instrument (2 hours)

- Use the KAPA analysis excel sheet provided online to copy the Cq values and the size of the library from the bioanalyzer to calculate the nM concentration of the sample

  - QC final pooled library before and after size selection on Agilent Bioanalyzer

- Dilute all samples 1:10 before running on the Bioanalzyer high sensitivity chip

- Obtain the median size of the library

Example:

13. MiSeq Sequencing

13.1 Follow manufacturer’s specifications for loading library on MiSeq using a Illumina MiSeq Reagent Kit V3

- Load 10 pM denatured library with 10-15% PhiX

- Generate MiSeq run sheet:

  - Read 1: 300 bases

  - Index read 1: 8 bases

  - Read 2: 300 bases

  - Index read 2: 16 bases