Table S1. Conditions of GC-MS-SIM and HPLC for analyzing the plant hormones.

| GC-MS condition |  |  |
| --- | --- | --- |
| Equipment | Hewlett-Packard 6890, 5973N Mass Selective Detector |  |
| Column | HP-1 capillary column (30 m × 0.25 mm i.d. 0.25 µm film thickness) |  |
| Carrier gas | He (40 ℃/min) |  |
| Source temperature | 250℃ |  |
| Oven conditions | GA: 60℃(1 min)→15℃/min→200℃(1 min)→5℃/min→285℃(5 min) JA:60℃(2min)→10℃/min→140℃(3min)→3℃/min→170℃→15℃/min→285℃(8 min) |  |
| Injector temperature | 200℃ |  |
| Ionizing voltage | 70 ev |  |
| HPLC condition |  |
| Equipment | Waters model 510 |
| Column | μ Bondapak C18 (3.9 mm × 300 mm) |
| Solvent A | 28% MeOH + 72% double distilled water (include 1% acetic acid) |
| Solvent B | 100% MeOH |
| Gradient | 100% Solvent A (0 - 5 min)→100% Solvent B (5 - 36 min)→100% Solvent B (36 - 40 min) |
| Flow rate | 1.5 ml/min |