Appendix 1 – Intervention protocol

## Monitoring

Continuous monitoring included ECG, SpO₂, NIBP, EtCO₂, processed EEG (NeuroSENSE®, NeuroWave Systems Inc., Beachwood, OH, USA), and quantitative neuromuscular transmission.

## Premedication

None

## Preoxygenation

3 min with 100% O₂, aiming for end-tidal O₂ ≥ 90%.

## Induction of anesthesia

- Sufentanil 0.1 µg/kg IV at start of preoxygenation

- Propofol TIVA (Schnider model), initial Ce 4 µg/mL, titrated to clinical effect (loss of ciliary reflex)

- Lidocaine 1 mg/kg IV and Rocuronium 0.6 mg/kg IV at loss of consciousness

## Maintenance of anesthesia

Propofol TIVA titrated to maintain slow and alpha EEG activity, absence of burst-suppression (suppression ratio = 0), and BIS 40–60.

## Hemodynamic Management

Arterial blood pressure maintained >65 mmHg. Ephedrine (3–9 mg IV boluses) was used first-line; if inadequate, phenylephrine or norepinephrine infusion was initiated at the anesthesiologist’s discretion.

## Airway Management

Direct laryngoscopy and tracheal intubation (ID 8.0 mm for men, 7.0 mm for women).

## Ventilation (VCV mode, initial settings)

Tidal volume 7 mL/kg IBW; PEEP 5 cmH₂O; FiO₂ 0.5; respiratory rate adjusted to maintain EtCO₂ 35–40 mmHg.

## Positioning

Supine, head in neutral alignment.

# Interventional Arms

Procedures common to both groups

- General anesthesia.

- Femoral vascular access by interventional cardiologist.

- Transseptal puncture guided by TEE in collaboration with anesthesiologist.

- Left atrial ABG obtained after successful access.

- Ventilation strategy adjusted according to randomized allocation.

- If SpO₂ decreased below 90%, FiO₂ was increased at the discretion of the attending anesthesiologist.

## Arm 1: High-Frequency Jet Ventilation (HFJV)

Device: Monsoon III (Acutronic Medical Systems AG, Hirzel, Switzerland).

Cannula/connection: 40 cm LaserJet® cannula (Acutronic) via Sheridan® Sher-I-Swiv® connector (Teleflex).

Circuit: Breathing circuit disconnected between HME filter and swivel connector.

Initial settings:

- Working pressure: 0.02 bar/kg IBW (≈ 0.2 kPa/kg IBW, ≈ 0.29 PSI/kg IBW)

- Frequency: 150 cycles/min

- FiO₂: 0.5

- I:E ratio: 0.3

- PPmax alarm: 10 mbar

- PIP max alarm: 15 mbar

- Humidification setting: 3

EtCO₂ monitoring and adjustment (every 30 min, starting 5 min after HFJV initiation):

- Pause HFJV.

- Reconnect standard anesthesia ventilator.

- Deliver 5 VCV breaths (7 mL/kg IBW).

- Measure EtCO₂

- If >40 mmHg: increase working pressure by 0.1 bar (1.45 PSI).

- If <30 mmHg: decrease working pressure by 0.1 bar (1.45 PSI).

- Resume HFJV with adjusted settings if applicable.

## Arm 2: Volume-Controlled Ventilation (VCV)

Device: Zeus® ventilator (Dräger Medical Belgium, Wemmel, Belgium).

Initial settings:

- Tidal volume: 7 mL/kg IBW

- Respiratory rate: 10–20 breaths/min, titrated to maintain EtCO₂ 30–40 mmHg

- PEEP: 5 cmH₂O

- FiO₂: 0.5

# End of Intervention

Upon completion of the ablation procedure:

- Discontinue HFJV (intervention group only)

- Obtain final left atrial ABG sample before catheter removal

- Withdraw jet cannula and transition to conventional ventilation (intervention group only)

- Reverse neuromuscular block with sugammadex (2–4 mg/kg IV), under quantitative monitoring

- Perform emergence and extubation according to standard criteria (adequate spontaneous breathing, responsiveness, protective airway reflexes)

- Admit patient to Post-Anesthesia Care Unit (PACU) for routine postoperative monitoring.

| A | B |
| --- | --- |
| Figure 1. High-Frequency Jet Ventilation setup via endotracheal tube. (A) Components before assembly. (B) Fully assembled configuration: swivel connector with HME filter, ETT (7.0–8.0 mm ID), and 40 cm LaserJet® cannula. |  |