SUPPLEMENTAL TABLES AND FIGURES

Supplemental Table 1. Participant characteristics

| Characteristic | Mean ± SD1 | Normal range |
| --- | --- | --- |
| Age (years) | 39.1 ± 12.8 | n/a |
| Heart rate (beats/min)2 | 60.7 ± 8.6 | 45 – 954 |
| Systolic blood pressure (right arm) (mmHg)2 | 100.1 ± 12.5 | 97-1205 |
| Diastolic blood pressure (right arm) (mmHg)2 | 72.8 ± 7.7 | 57-875 |

1 n=39 participants (24 females, 15 males)

2Data averaged from 3 measurements for each participant

4,5 Reference ranges from [45, 46], respectively.

n/a= not applicable

Supplemental Table 2. Vascular parameter values and distribution

| Parameter | Mean ± SE1 | Range | CV2 |
| --- | --- | --- | --- |
| VP-ba-PWV (m/s) right | 12.5 ± 1.3 | 8.7 - 16.5 | 0.14 |
| VP-ba-PWV (m/s) left | 12.9 ± 1.3 | 9.7 - 16.2 | 0.14 |
| SC-ra-PWV (m/s) | 8.3 ± 1.1 | 5.7 - 11.4 | 0.15 |
| SC-AIx (%) | 16.5 ± 3.7 | -11.3 - 43.3 | 0.83 |
| SC-AIx @ 75 bpm (%) | 9.6 ± 1.1 | -12.3 - 35.7 | 1.28 |
| EP-AIx (%) | 10.9 ± 4.6 | -22 - 69 | 1.92 |
| EP-AIx @ 75 bpm (%) | 2.1 ± 4.4 | -27 - 60 | 9.33 |
| EP-RHI | 2.1 ± 0.7 | 1.2 - 3.3 | 0.21 |

1n=39

2coefficient of variation = standard deviation/mean×100

Instrument abbreviations: VP: VP-1000; SC: SphygmoCor; EP: EndoPat

Measurement abbreviations: PWV: Pulse wave velocity; AIx: Augmentation index; @75: Corrected to 75 bpm; RHI: Reactive hyperemia index, ba: brachial-ankle, ra: radial-ankle

Supplemental Figure Legends

Supplemental Figure 1: Agreement between outputs of devices measuring the same parameters. The differences between the absolute values obtained from the different methods are plotted against their means. Bias (solid horizontal line) is defined as the mean of the differences and the limits of agreement are set at ± 2SD of this mean (dashed horizontal line). Panel A: Agreement of PWV, as measured by the VP-1000 and the SphygmoCor. Panel B: Agreement of AIx@75, as measured by the SphygmoCor and the EndoPAT.

A.

B.