Appendix E1

Methods and Materials

Study Population

The exclusion criteria were: (1) Significant coronary artery disease, defined as luminal stenosis >50% on coronary angiography or coronary computed tomography, or evidence of an ischemic late gadolinium enhancement (LGE) pattern (e.g., perfusion defects or segmental endocardial LGE); (2) primary valvular disease; (3) evidence of cardiac amyloidosis, sarcoidosis, myocarditis, hypertensive heart disease, end-stage hypertrophic cardiomyopathy, tachycardia-induced cardiomyopathy, or arrhythmogenic right ventricular cardiomyopathy; (4) congenital heart disease or inflammatory/inﬁltrative diseases; (5) inadequate image quality, presence of arrhythmia, loss to follow-up, or unwillingness to participate; and (6) contraindications for CMR examination or an estimated glomerular filtration rate < 30 ml/min/1.73 m2.

MRI Acquisition

All CMR imaging was acquired using a 3.0-T scanner (MAGNETOM Trio or Skyra, Siemens Healthcare, Erlangen, Germany) with a 32-channel cardiac coil or a 30-channel body coil. Cine images were acquired during multiple breath holds using balanced steady-state free precession imaging with electrocardiographic gating. Short-axis images covered the entire LV, while long-axis views were obtained as follows: the four-chamber view was acquired from the vertical long-axis scout, passing through the interior and anterior myocardial walls, and the two-chamber view was acquired through the center of the tricuspid and mitral valves and the apex.

The acquisition parameters were as follows: field of view, 280 × 340 mm2; repetition time, 2.6 ms; echo time, 1.3 ms; flip angle, 50°; matrix size, 256 × 144; slice thickness, 8 mm with no gap; temporal resolution, 42 ms; in-plane spatial resolution reconstruction, 1.4 mm × 1.3 mm. Fifteen minutes after gadolinium (Magnevist; Bayer Schering Pharma) injection with a 0.15 mmol/kg bolus, Late gadolinium enhancement (LGE) images were acquired in the long-axis and consecutive short-axis planes using phase-sensitive inversion recovery (PSIR) gradient-echo sequence, covering the entire LV. The typical scan parameters of LGE images were as follows: repetition time, 700 ms; echo time, 2.0 ms; delay time after inversion pulse, 300–380 ms; flip angle, 20°; field of view, 260 × 340 mm2; matrix size, 116 × 192; and slice thickness, 8 mm.

Results

LACI Survival Analysis in CMR

The Kaplan-Meier survival curves for HF readmission, HF death, and SCD stratified by LACI are presented in Figure S1, Figure S2, Figure S3, demonstrating significantly lower event-free survival in patients with LACI > 30% (P = 0.004, P < 0.001, P = 0.092, respectively). These results show significant outcomes for HF readmission, HF death but not for SCD.