Supplementary material-2

Table S1. Communication Staff Setup and Equipment

Table S2. Dispatcher-Assisted CPR Protocol and Communication Workflow for V-CPR group

Table S3. Examples of Real-Time Dispatcher Feedback During Video-Assisted CPR

Table S4. Dispatcher-Assisted CPR Protocol and Communication Workflow for T-CPR group

Table S5. Steps for Setting Up LINE and Initiating a Video Call

Table S1. Dispatcher Setup and Equipment

| Component | Description |
| --- | --- |
| Location | Communication staff were located in a private room without external noise interference. |
| Personnel | A single Dispatcher |
| Mobile phone 1 | Used to receive the initial emergency telephone call from the participant. |
| Mobile phone 2 | Used to conduct the LINE video call with the participant. |
| Tablet device (iPad) | Installed with the CPR Tempo application to provide metronome guidance (110 compressions/min). |
| Preparation | Communication staff prepared the instruction script, metronome, and communication devices prior to each session. |

Table S2. Dispatcher-Assisted CPR Protocol and Communication Workflow for V-CPR group

| Phase | Dispatcher Action | Participant Action | Research Staff Action |
| --- | --- | --- | --- |
| Pre-simulation | Prepare instruction script, metronome, and devices | Receive standardized instructions | Prepare timing and recording equipment |
| Initial call | Assess patient condition | Describe simulated patient condition | Observe |
| Dispatcher: “Hello, Emergency Medical Dispatch Center. What is the patient’s condition?” Participant A: ……………………….. |  |  |  |
| Location confirmation | Confirm location, phone number, and caller identity | Provide location and phone number | Observe |
| Dispatcher: “Where is the patient located?” Participant A: “At the Lecture Building, 3rd floor, Faculty of Medicine.” Dispatcher: “What phone number are you calling from?” Participant A: xxx-xxxxxxx |  |  |  |
| Patient assessment | Ask about consciousness and breathing | Confirm unconsciousness and absence of breathing | Observe |
| Dispatcher : “Please activate the speakerphone and follow my instructions. Is the patient conscious? Are they responding?” Participant A: “No, the patient is unconscious and not responding.” Dispatcher: “Ask your friend to kneel beside the patient and expose the patient’s chest. Is the patient breathing? Is the chest rising and falling?” Participant A: “No breathing. No chest movement.” |  |  |  |
| CPR initiation | Instruct participant to begin chest compressions with proper hand placement, depth (~5 cm), and rate (110/min) | Begin CPR | Start 10-minute timer |
| Dispatcher: “The patient is in cardiac arrest. Please begin chest compressions while waiting for emergency medical services. Are you ready to follow my instructions?” Participant A: “Yes.” |  |  |  |
| 3-4 minutes | Provide continuous CPR guidance using metronome | Perform CPR | Record time and events |
| Dispatcher: - Place the heel of your dominant hand in the center of the patient’s chest, between the nipples. - Place your other hand on top of the first hand. - Keep your arms straight. Use your shoulders to apply downward force. Compress the chest approximately 5 cm deep. - Compress in rhythm with the metronome sound (“beep–beep–beep”) at approximately 110 compressions per minute. - Continue compressions continuously. - Continue compressions for approximately 2 minutes until instructed to stop, then switch rescuers. - If you become fatigued, you may switch rescuers earlier. Participant A: Perform CPR |  |  |  |
| LINE activation | Send SMS with LINE link and instruct participant to initiate video call | Open link and initiate LINE video call | Observe |
| Dispatcher: “We have sent an SMS to your phone. Please open the link to add the dispatch center LINE account.” Participant B: “I see the link.” Participant B: “I have clicked the link.” Dispatcher: “Please initiate a LINE video call to the dispatch center.” Participant B: (Initiates video call) |  |  |  |
| Video-assisted CPR | Provide real-time visual feedback on compression rate, depth, hand position, and technique | Continue CPR under video guidance | Record performance and timing |
| Dispatcher: “Please end the telephone call and position the smartphone camera to allow visual guidance during chest compressions.” Dispatcher provided real-time CPR feedback, including: - If compression rate was too slow → “Please compress faster.” - If compression rate was too fast → “Please compress slower.” - If compression depth was appropriate → “Compression depth is appropriate. Continue.” - If compression depth was insufficient → “Compress harder while maintaining the same rhythm.” - If compression depth was excessive → “Compress less forcefully while maintaining the same rhythm.” - If arm position was incorrect → “Please keep your arms straight.” - If hand position was incorrect → “Please adjust your hand position toward the patient’s head.” |  |  |  |
| 5–6 minutes | Instruct rescuer switch | Switch compressors to participant B | Observe and record |
| 7–8 minutes | Instruct rescuer switch | Switch compressors to participant A | Observe and record |
| 9–10 minutes | Instruct rescuer switch and continue CPR | Switch compressors to participant B | Stop timer at 10 minutes |
| Post-simulation | End communication session | Stop CPR | Document completion |

Table S3. Examples of Real-Time Dispatcher Feedback During Video-Assisted CPR

| CPR Performance Issue | Dispatcher Instruction |
| --- | --- |
| Compression rate too slow | “Please compress faster.” |
| Compression rate too fast | “Please compress slower.” |
| Compression depth appropriate | “Good compression depth. Continue.” |
| Compression too shallow | “Compress harder while maintaining the same rhythm.” |
| Compression too deep | “Compress slightly less forcefully.” |
| Incorrect arm position | “Please keep your arms straight.” |
| Incorrect hand position | “Please adjust your hand position toward the center of the chest.” |

Table S4. Dispatcher-Assisted CPR Protocol and Communication Workflow for T-CPR group

| Phase | Dispatcher Action | Participant Action | Research Staff Action |
| --- | --- | --- | --- |
| Pre-simulation | Prepare instruction script, metronome, and devices | Receive standardized instructions | Prepare timing and recording equipment |
| Initial call | Assess patient condition | Describe simulated patient condition | Observe |
| Dispatcher: “Hello, Emergency Medical Dispatch Center. What is the patient’s condition?” Participant: ……………………….. |  |  |  |
| Location confirmation | Confirm location, phone number, and caller identity | Provide location and phone number | Observe |
| Dispatcher: “Where is the patient located?” Participant: “At the Lecture Building, 3rd floor, Faculty of Medicine.” Dispatcher: “What phone number are you calling from?” Participant: xxx-xxxxxxx |  |  |  |
| Patient assessment | Ask about consciousness and breathing | Confirm unconsciousness and absence of breathing | Observe |
| Dispatcher: “Please activate the speakerphone and follow my instructions. Is the patient conscious? Are they responding?” Participant: “No, the patient is unconscious and not responding.” Dispatcher: “Ask your friend to kneel beside the patient and expose the patient’s chest. Is the patient breathing? Is the chest rising and falling?” Participant: “No breathing. No chest movement.” |  |  |  |
| CPR initiation | Instruct participant to begin chest compressions with proper hand placement, depth (~5 cm), and rate (110/min) | Begin CPR | Start 10-minute timer |
| Dispatcher: “The patient is in cardiac arrest. Please begin chest compressions while waiting for emergency medical services. Are you ready to follow my instructions?” Participant: “Yes.” |  |  |  |
| 3-4 minutes | Provide continuous CPR guidance using metronome | Perform CPR | Record time and events |
| Dispatcher: - Place the heel of your dominant hand in the center of the patient’s chest, between the nipples. - Place your other hand on top of the first hand. - Keep your arms straight. Use your shoulders to apply downward force. Compress the chest approximately 5 cm deep. - Compress in rhythm with the metronome sound (“beep–beep–beep”) at approximately 110 compressions per minute. - Continue compressions continuously. - Continue compressions for approximately 2 minutes until instructed to stop, then switch rescuers. - If you become fatigued, you may switch rescuers earlier. |  |  |  |
| 5-6 minutes | Instruct rescuer switch | Switch compressors to participant B | Observe and record |
| Continue CPR 2 min - Place your other hand on top of the first hand. - Keep your arms straight. Use your shoulders to apply downward force. Compress the chest approximately 5 cm deep. - Compress in rhythm with the metronome sound (“beep–beep–beep”) at approximately 110 compressions per minute. - Continue compressions continuously. - Continue compressions for approximately 2 minutes until instructed to stop, then switch rescuers. |  |  |  |
| 7-8 minutes | Instruct rescuer switch | Switch compressors to participant A | Observe and record |
| Continue CPR 2 min - Place your other hand on top of the first hand. - Keep your arms straight. Use your shoulders to apply downward force. Compress the chest approximately 5 cm deep. - Compress in rhythm with the metronome sound (“beep–beep–beep”) at approximately 110 compressions per minute. - Continue compressions continuously. - Continue compressions for approximately 2 minutes until instructed to stop, then switch rescuers. |  |  |  |
| 9-10 minutes | Instruct rescuer switch | Switch compressors to participant B | Observe and record |
| Continue CPR 2 min - Place your other hand on top of the first hand. - Keep your arms straight. Use your shoulders to apply downward force. Compress the chest approximately 5 cm deep. - Compress in rhythm with the metronome sound (“beep–beep–beep”) at approximately 110 compressions per minute. - Continue compressions continuously. - Continue compressions for approximately 2 minutes until instructed to stop, then switch rescuers. |  |  |  |
| Post-simulation | End communication session | Stop CPR | Document completion |

Table S5. Steps for Setting Up LINE and Initiating a Video Call

- Participant B: Opens the LINE app on their smartphone, then navigates to “Add Friends.”

- Enters the dispatcher’s phone number, taps Search, and selects “Add as Friend.”

- Waits momentarily for the dispatcher to accept their friend request.

- Once accepted, the participant initiates a LINE video call to the dispatcher by opening the dispatcher’s chat and tapping the video call icon.