Additional file 1

A description of the 10 tests (42 digital markers)

Balance (postural sway)

Static balance – 4 tests

One leg stance (left and right) for a duration of 10 seconds, and tandem stance (left foot forward, and right foot forward) for a duration of 20 seconds.

Dynamic balance – 2 tests

10 steps tandem walk forward and backward.

Five relevant digital markers (DMs) will assess the sway in each of the above 6 tests (n=30): average linear acceleration assessed in meter per second squared (m/sec2), which is generated from the accelerometer (Acc) in ML (medio-lateral) and AP (anterior-posterior) directions, and angular (radial) velocity assessed in radians per second (rad/sec), which is generated from the gyroscope (Gyro) in the ML, AP, and SI (superior-inferior) directions. Lower scores indicate less sway, thus better performance.

Legs strength: Sit-to-Stand –10 repetitions. DMs (n=2): total time (seconds) and average duration for each repetition. Shorter duration indicates better performance.

Upper body flexibility (each test will be performed twice and the score will be the average of the two)

Torso rotation: In a seated position, holding a ball between thighs and a stick on chest for stabilization:  torso rotation as far as possible to the right and to the left. DMs (n=2): angle (peak pitch in the SI direction). Increased angle indicates a greater range of motion, thus better performance.

Upper extremities flexibility:

Arm flexion: In a seated position on an armless chair, back against the wall – lifting straight arm forward and up trying to reach the ear (right and left). DMs (n=2): angle between arm and the horizon (peak yaw in the AP direction). Increased angle indicates a greater range of motion, thus better performance.

Arm extension: In a standing position, face toward the wall, all front body against the wall – lifting straight arm backward as far as possible (right and left). DMs (n=2): angle between arm and the horizon (peak yaw in the AP direction). Increased angle indicates a greater range of motion, thus better performance.

Upper extremities strength

In a seated position on an armless chair, a weight of 0.5 kg for women and 1 kg for men attached to wrist:

Lifting straight arm forward to shoulder height as fast as possible 20 times (right and left arm), and to the side 20 times (right and left arm). DMs (n=4): average duration for the 20 repetitions measured in seconds. A shorter duration indicates better performance.