Increased Investment in Gametocytes in Asymptomatic Plasmodium falciparum Infections in the Wet Season

Additional File 1

Droplet digital PCR varATS protocol

| Component | Final concentration | Volume/Reaction |
| --- | --- | --- |
| Bio-Rad supermix for probe (No dUTP) | 1X | 11 µL |
| Probe | 10 µM | 0.55 µL |
| Primer set (Fwd+Rev) | 10 µM | 1.98 µL |
| Nuclease-free water |  | 6.47 µL |
| Template DNA |  | 2 µL |

Thermocycling conditions

950C (10 min)

940C (30 sec)

550C (1 min)            45 cycles

980C (10 min)

Primer and probe sequences

varATS fwd	CCCATACACAACCAAYTGGA

varATS rev	TTCGCACATATCTCTATGTCTATCT

varATS probe	6-FAM-TRTTCCATAAATGGT-NFQ-MGB

pfs25 RT-qPCR protocol

| Component | Final concentration | Volume/Reaction |
| --- | --- | --- |
| Radiant TM 1-step Lo-Rox 2X Mix (Alkali Scientific) | 1X | 6 µL |
| 20x RTase | 1X | 0.6 µL |
| Primer set (Fw+Rv) | 10 µM | 1 µL |
| Nuclease-free water |  | 2.4 µL |
| Template RNA |  | 2 µL |

Thermocycling conditions

450C (10 min)

950C (2 min)

950C (10 sec)

580C (30 sec) 	     45 cycles

Primers

pfs25 fwd	CGT TTC ATA CGC TTG TAA ATG

pfs25_rev	TTA ACA GGA TTG CTT GTA TCT AA

Droplet digital PCR pfs25 protocol

Bio-Rad probe (2X supermix for probe (No dUTP)

| Component | Final concentration | Volume/Reaction |
| --- | --- | --- |
| Bio-Rad supermix for probe (No dUTP) | 1X | 11 µL |
| Probe | 10 µM | 0.22 µL |
| Primer set (Fwd+Rev) | 10 µM | 0.66 µL |
| Nuclease-free water |  | 8.12 µL |
| Template DNA |  | 2 µL |

Thermocycling conditions

950C (10 min)

940C (30 sec)

540C (1 min)           45 cycles

980C (10 min)

Primers and probe

pfs25_Probe	HEX-TG TAA GAA TG TAA CTT GTG GTA ACG GT-ZEN

pfs25_fwd	CGT TTC ATA CGC TTG TAA ATG

pfs25_rev	TTA ACA GGA TTG CTT GTA TCT AA