Supplementary Material 1. Determination of plasma concentrations of INH

The plasma concentration of INH was determined via high-performance chromatography (HPLC) via a Phenomenex C18 column, Gemini 5 m (150–4.6 mm). A total of 600 L of acidified acetonitrile (0.1% formic acid) was added to 200 L of human plasma in a 15 mL Falcon tube. The mixture was vortexed on a tube shaker for 60 seconds, and the tube was subsequently centrifuged at 3800 rpm for 10 minutes at 5°C. A total of 20 L of the supernatant was diluted in 180 L of acidified water (0.1% formic acid), and the resulting 200 L was filtered through a syringe filter (hydrophilic PTFE 22 m) and injected into the UHPLC–MS/MS instrument. The mobile phases were water + 0.1% formic acid (phase A) and acetonitrile + 0.1% formic acid (phase B). A flow rate of 0.6 mL/min was used. Multiple reaction monitoring (MRM) was performed in positive mode. The transition used for isoniazid quantification was 138–121.2. The calibration curve included points between 1 and 50 ng/mL (1, 2, 5, 10, 20 and 50 ng/mL), with correlation coefficients greater than 0.99.