Supplemental Table 1 MR parameters

| 1.5T GE Signa HDxt |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | FIESTA | T2WI | DWI b=800 | TIWI FS | TIWI DUAL | T1WI+C Mph | T1WI+C | T1WI+C Delay |
| Image mode | 2D | 2D | 2D | 2D | 2D | 3D | 3D | 3D |
| Special parameters | GRE | FRFSE | EPI | FSPGR | FSPGR | LAVA | LAVA | LAVA |
| Breath | Hold breath | Respiratory triggering | Hold breath | Hold breath | Hold breath | Hold breath | Hold breath | Hold breath |
| Scan plane | Cor | Ax | Ax | Ax | Ax | Ax | Cor | Ax |
| Fat suppression | No | Yes | No | Yes | In-Phase Out-Phase | Yes | Yes | Yes |
| TR (ms) | 3.4 | 8000 | 6667 | 220 | 220 | 3.3 | 3.5 | 3.3 |
| TE (ms) | 1.5 | 86 | 69 | 2.2 | 2.2/4.8 | 1.6 | 1.7 | 1.6 |
| FOV (cm) | 38×38 | 38×29 | 38×29 | 38×34 | 38×34 | 38×34 | 40×40 | 38×34 |
| Slice thickness (mm) | 5 | 6 | 6 | 6 | 6 | 4 | 3 | 4 |
| Spacing | 0.5 | 1.2 | 1.2 | 1.2 | 1.2 | -2 | -1.5 | -2 |
| Slices | 20 | 24 | 24 | 24 | 24 | 92 | 88 | 92 |
| Matrix | 224× 224 | 288× 224 | 128× 128 | 288× 170 | 288× 170 | 288× 160 | 300× 224 | 288× 160 |
| NEX | 1 | 2 | 4 | 1 | 1 | 0.72 | 0.71 | 0.72 |

Supplemental Table 1-continued MR parameters

| 3.0T GE Discovery MR750w |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | T2WI | T2WI | DWI b=800 | LAVA FLEX | IDEAL -IQ | T1W+C Mph | T1W+C | T1W+C delay |
| Image mode | 2D | 2D | 2D | 3D | 3D | 3D | 3D | 3D |
| Special parameters | SSFSE | FSE-XL | EPI | LAVA | IDEAL | LAVA | LAVA | LAVA |
| Breath | Hold breath | Respiratory triggering | Respiratory triggering | Hold breath | Hold breath | Hold breath | Hold breath | Hold breath |
| Scan plane | Cor | Ax | Ax | Ax | Ax | Ax | Cor | Ax |
| Fat suppression | No | Yes | Yes | No/Yes | - | Special | Special | Special |
| TR (ms) | Min | 8000 | 8000 | 6.6 | 7.3 | 4.1 | 4.2 | 4.1 |
| TE (ms) | Min | 85 | Min | Min Full | Min | 1.9 | 2 | 1.9 |
| FOV (cm) | 42×42 | 38×38 | 38×38 | 38×34 | 40×40 | 38×34 | 40×40 | 38×34 |
| Slice thickness (mm) | 5 | 6 | 6 | 4 | 8 | 4 | 3 | 4 |
| Spacing (mm) | 0.5 | 1.2 | 1.2 | -2 | 0 | -2 | -1.5 | -2 |
| Slices | 24 | 24 | 24 | 96 | 32 | 96 | 136 | 96 |
| Matrix | 288× 256 | 288× 224 | 128× 128 | 288× 180 | 160× 160 | 288× 170 | 300× 224 | 288× 170 |
| NEX | 0.53 | 2 | 2 | 1 | 0.5 | 0.72 | 0.71 | 0.72 |

Supplemental Table 1-continued MR parameters

| 3.0T Siemens MAGNETOM Prisma |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | T2WI | T2WI | Diff | q-dixon | Vibe | T1WI+C Vibe | T1WI+C Vibe | T1WI+C Delay |
| Image mode | 2D | 2D | 2D | 3D | 3D | 3D | 3D | 2D |
| Special parameters | Haste | TSE | EPI | - | Dixon | Dixon | Dixon | Dixon |
| Breath mode | Hold breath | Respiratory triggering | Free breath | Hold breath | Hold breath | Hold breath | Hold breath | Hold breath |
| Scan plane | Cor | Tra | Tra | Tra | Tra | Tra | Cor | Tra |
| Fat suppression | No | SPAIR | SPAIR | - | - | Yes | Yes | Yes |
| TR (ms) | 800 | 2000- 5000 | 2400- 3000 | 9 | 3.89 | 3.89 | 4.21 | 3.89 |
| TE (ms) | 91 | 83 | 58 | 1.05 | 1.23 | 1.23 | 1.34 | 1.23 |
| FOV (mm) × % | 360× 100% | 400× 87.5% | 420× 65.7﹪ | 450× 87.5% | 400× 81.3% | 400× 81.3% | 400× 81.3% | 400× 81.3% |
| Slice thickness (mm) | 5 | 6 | 6 | 3.5 | 3 | 3 | 1.5 | 3 |
| Spacing | 20% | 30% | 30% | 20% | 20% | 20% | 20% | 20% |
| Slices | 20 | 30 | 30 | 64 | 72 | 72×5 phases | 144 | 72 |
| Matrix size | 320× 100% | 384× 80﹪ | 134× 100% | 160× 79% | 288× 75% | 288× 75% | 320× 90% | 288× 75% |
| NEX | 1 | 3 | 1 | 1 | 1 | 1 | 1 | 1 |

Supplemental Table 1-continued MR parameters

| 3.0T Philips Ingenia |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | T2WI | T2WI | Diff | Mdxion Quant | mDIXON | mDIXON | mDIXON cor | mDIXON |
| Image mode | 2D | 2D | 2D | 3D | 3D | 3D | 3D | 2D |
| Special parameters | SSTSE | TSE | EPI | - | Dixon | Dixon | Dixon | Dixon |
| Breath mode | Hold breath | Respiratory triggering | Free breath | Hold breath | Hold breath | Hold breath | Hold breath | Hold breath |
| Scan plane | Cor | Tra | Tra | Tra | Tra | Tra | Cor | Tra |
| Fat suppression | No | SPIR | SPAIR | - | Yes | Yes | Yes | Yes |
| TR (ms) | 924 | 2000- 5000 | 2000 | 5.7 | 3.7 | 3.6 | 3.5 | 3.6 |
| TE (ms) | 80 | 78 | 69 | 0.97 | 1.32 | 1.31 | 1.14 | 1.31 |
| FOV (mm) | 450× 398 | 380× 380 | 400× 272 | 400× 300 | 400× 362 | 400× 352 | 380x 418 | 400× 352 |
| Slice thickness (mm) | 5 | 6 | 6 | 6 | 4 | 4 | 3 | 4 |
| Spacing (mm) | 0.6 | 1.2 | 1.2 | -3 | -2 | -2 | -1.5 | -2 |
| Slices | 24 | 24 | 24 | 80 | 100 | 100×2 phases | 80 | 100 |
| Matrix | 376× 290 | 280× 280 | 124× 75 | 168× 119 | 252× 228 | 252× 222 | 200× 220 | 252× 222 |
| NEX | 1 | 1 | 2 | 1 | 1 | 1 | 1 | 1 |

Note: FIESTA, fast imaging employing steatly-state acquisition; T2WI, T2 weighted imaging; DWI, diffusion weighted imaging; T1WI, T1 weighted imaging; LAVA, Liver Acquisition with Volume Acceleration; GRE, gradient echo; FRFSE, fast relaxation fast spin echo; EPI, echo planar imaging; FSPGR, fast spoiled gradient recalled echo; TR, repetition time; TE, echo time; FOV, field of view; NEX, number of average; IDEAL-IQ, Iterative Decomposition of water and fat with Echo Asymmetry and Least squares estimation; SSFSE, single-shot fast spin echo; TSE, turbo spin echo; HASTE, half-Fourier acquisition single shot turbo spin echo; SPIR, spectral presaturation inversion recovery; SPAIR, spectrally selective attenuated inversion recovery; VIBE, volume interpolated body examination.