Magnetic resonance scanning parameter:

From the First Affiliated Hospital of Zhejiang Chinese Medical University:

1.5T scanners were as follows:

(1)T1WI: repetition time(TR)/echo time(TE)=550ms/8.4ms, slice thickness/slice spacing=5.0mm/1.5mm, FOV=240mm*240mm; (2)T2WI: repetition time(TR)/echo time(TE)=4000ms/99ms, slice thickness/slice spacing=5.0mm/1.5mm, FOV=240mm*240mm; (3)SPEI-DWI：repetition time(TR)/echo time(TE)=3600ms/102ms, slice thickness/slice spacing=5.0mm/1.0mm, FOV=240mm*240mm, b = 0 and 1000s/mm2.

3.0T scanners were as follows:

(1)T1WI: repetition time(TR)/echo time(TE)=1800ms/9.4ms, slice thickness/slice spacing=5.0mm/0.7mm, FOV=256mm*256mm; (2)T2WI: repetition time(TR)/echo time(TE)=6000ms/95ms, slice thickness/slice spacing=5.0mm/0.6mm, FOV=256mm*256mm; (3)SEPI-DWI：repetition time(TR)/echo time(TE)=6300ms/94ms, slice thickness/slice spacing=5.0mm/1.0mm, FOV=256mm*256mm, b = 0 and 1000s/mm2.

From the First People's Hospital of Hangzhou:

1.5T scanners were as follows:

(1)T1WI: repetition time(TR)/echo time(TE)=487ms/14ms, slice thickness/slice spacing=5.0mm/1.0mm, FOV=230mm*230mm; (2)T2WI: repetition time(TR)/echo time(TE)=3690ms/94ms, slice thickness/slice spacing=5.0mm/1.0mm, FOV=230mm*230mm; (3)SEPI-DWI：repetition time(TR)/echo time(TE)=2900ms/89ms, slice thickness/slice spacing=5.0mm/1.0mm, FOV=230mm*230mm, b = 0 and 1000s/mm2.

3.0T scanners were as follows:

(1)T1WI: repetition time(TR)/echo time(TE)=1500ms/8.5ms, slice thickness/slice spacing=5.0mm/1.0mm, FOV=230mm*230mm; (2)T2WI: repetition time(TR)/echo time(TE)=3700ms/96ms, slice thickness/slice spacing=5.0mm/1.0mm, FOV=230mm*230mm; (3)SEPI-DWI：repetition time(TR)/echo time(TE)=5100ms/100ms, slice thickness/slice spacing=5.0mm/1.0mm, FOV=230mm*230mm, b = 0 and 1000s/mm2.