# ===================== SUPPLEMENTARY CODE (COLAB + GOOGLE DRIVE) =====================

# Purpose:

#   Fully automated inference pipeline for disc-level labeling and AP thecal sac diameter measurement

#   from mid-sagittal lumbar spine MRI DICOM images.

#

# Key steps:

#   1) Load DICOM -> robust intensity normalization -> 8-bit

#   2) YOLOv8-OBB detection (letterbox to TRAIN_SIZE, then map OBB corners back)

#   3) Oriented (perspective) warp crop using OBB -> Attention U-Net segmentation (thecal sac)

#   4) Warp segmentation back to original image -> build full thecal sac mask

#   5) Disc-level labeling bottom-to-top (L5–S1 to T12–L1)

#   6) AP diameter estimation using skeletonized intersection along an oriented line

#   7) Save annotated JPEG outputs to Drive

#

# Notes:

#   - Pixel spacing is read from DICOM (PixelSpacing), and mm conversion is applied.

#   - The “AP line” is intended to run from anterior CSF margin to posterior CSF margin (front-white to back-white),

#     represented by an oriented line intersected with the predicted thecal sac mask.

from google.colab import auth, drive

auth.authenticate_user()

drive.mount('/content/drive', force_remount=True)

!pip -q install ultralytics pydicom scikit-image opencv-python-headless tensorflow

import os

import cv2

import numpy as np

import pydicom

from skimage.morphology import skeletonize

from ultralytics import YOLO

from keras.models import load_model

from keras import backend as K

# --------------------------- USER CONFIG ---------------------------

BASE = "/content/drive/MyDrive/spine_pipeline"

INPUT_FOLDER  = os.path.join(BASE, "inputforpipeline")

OUTPUT_FOLDER = os.path.join(BASE, "pipeline_output")

OBB_MODEL_PATH = os.path.join(BASE, "yolo obb.pt")

UNET_MODEL_PATH = os.path.join(BASE, "attention unet.keras")

os.makedirs(OUTPUT_FOLDER, exist_ok=True)

# U-Net input size

UNET_IMG_SIZE = (256, 256)

# YOLO inference size MUST match your training size

TRAIN_SIZE = 384  # change if you trained with 640, etc.

# Disc labels assigned bottom-to-top

DISC_LEVELS_BOTTOM_TO_TOP = ["L5-S1", "L4-L5", "L3-L4", "L2-L3", "L1-L2", "T12-L1"]

# YOLO inference thresholds

YOLO_CONF = 0.30

YOLO_IOU  = 0.30

# Text settings

FONT = cv2.FONT_HERSHEY_SIMPLEX

FONT_SCALE = 0.42

FONT_THICK = 1

TEXT_COLOR = (0, 0, 255)   # red

SKELETON_COLOR = (255, 0, 255)  # magenta (BGR)

# ------------------------------------------------------------------

# --------------------------- METRICS (MODEL LOAD) ---------------------------

def dice_coefficient(y_true, y_pred, smooth=1):

y_true_f = K.flatten(y_true)

y_pred_f = K.flatten(y_pred)

inter = K.sum(y_true_f * y_pred_f)

return (2. * inter + smooth) / (K.sum(y_true_f) + K.sum(y_pred_f) + smooth)

def iou_score(y_true, y_pred, smooth=1):

y_true_f = K.flatten(y_true)

y_pred_f = K.flatten(y_pred)

inter = K.sum(y_true_f * y_pred_f)

union = K.sum(y_true_f) + K.sum(y_pred_f) - inter

return (inter + smooth) / (union + smooth)

# ---------------------------------------------------------------------------

# --------------------------- HELPERS ---------------------------

def order_points(pts):

"""Order 4 points as: top-left, top-right, bottom-right, bottom-left."""

rect = np.zeros((4, 2), dtype=np.float32)

s = pts.sum(axis=1)

rect[0] = pts[np.argmin(s)]

rect[2] = pts[np.argmax(s)]

d = np.diff(pts, axis=1)

rect[1] = pts[np.argmin(d)]

rect[3] = pts[np.argmax(d)]

return rect

def dicom_to_uint8(ds):

"""Robust DICOM -> uint8 with rescale + percentile windowing."""

arr = ds.pixel_array.astype(np.float32)

slope = float(getattr(ds, "RescaleSlope", 1.0))

intercept = float(getattr(ds, "RescaleIntercept", 0.0))

arr = arr * slope + intercept

p1, p99 = np.percentile(arr, (1, 99))

arr = np.clip(arr, p1, p99)

arr = (arr - p1) / (p99 - p1 + 1e-6) * 255.0

return arr.astype(np.uint8)

def get_pixel_spacing_mm(ds):

"""Return mean pixel spacing in mm (safe fallback = 1.0)."""

if hasattr(ds, "PixelSpacing"):

ps = ds.PixelSpacing

return 0.5 * (float(ps[0]) + float(ps[1]))

return 1.0

def letterbox_to_square(img_bgr, new_size):

"""

Letterbox image to square while keeping aspect ratio.

Returns: square_image, scale, pad_left, pad_top

"""

h, w = img_bgr.shape[:2]

scale = min(new_size / w, new_size / h)

nw, nh = int(w * scale), int(h * scale)

img_rs = cv2.resize(img_bgr, (nw, nh), interpolation=cv2.INTER_AREA)

canvas = np.zeros((new_size, new_size, 3), dtype=np.uint8)

pad_left = (new_size - nw) // 2

pad_top  = (new_size - nh) // 2

canvas[pad_top:pad_top+nh, pad_left:pad_left+nw] = img_rs

return canvas, scale, pad_left, pad_top

def unletterbox_corners(corners_sq, scale, pad_left, pad_top):

"""Map corners from letterboxed-square coords back to original image coords."""

c = corners_sq.copy().astype(np.float32)

c[:, 0] = (c[:, 0] - pad_left) / (scale + 1e-9)

c[:, 1] = (c[:, 1] - pad_top)  / (scale + 1e-9)

return c

# ------------------------------------------------------------------

# --------------------------- LOAD MODELS ---------------------------

for p in [INPUT_FOLDER, OBB_MODEL_PATH, UNET_MODEL_PATH]:

if not os.path.exists(p):

raise FileNotFoundError(f"Missing required path: {p}")

print("✅ DICOM count:", len([f for f in os.listdir(INPUT_FOLDER) if f.lower().endswith(".dcm")]))

print("\n🔵 Loading YOLO OBB model...")

obb_model = YOLO(OBB_MODEL_PATH)

print("🟢 Loading Attention U-Net model...")

unet_model = load_model(

UNET_MODEL_PATH,

custom_objects={"dice_coefficient": dice_coefficient, "iou_score": iou_score}

)

# ------------------------------------------------------------------

# --------------------------- MAIN LOOP ---------------------------

processed, skipped = 0, 0

print("\n🚀 Running pipeline...\n")

for file_name in os.listdir(INPUT_FOLDER):

if not file_name.lower().endswith(".dcm"):

continue

dicom_path = os.path.join(INPUT_FOLDER, file_name)

# --- Read DICOM ---

try:

ds = pydicom.dcmread(dicom_path)

pixel_spacing = get_pixel_spacing_mm(ds)

img_8bit = dicom_to_uint8(ds)

except Exception as e:

print(f"❌ Read error: {file_name} | {e}")

skipped += 1

continue

img = cv2.cvtColor(img_8bit, cv2.COLOR_GRAY2BGR)

original = img.copy()

# Full thecal sac mask accumulated across detected levels

full_mask = np.zeros(img.shape[:2], dtype=np.uint8)

# --- YOLO on letterboxed square ---

img_sq, scale, pad_left, pad_top = letterbox_to_square(img, TRAIN_SIZE)

results = obb_model.predict(

source=img_sq, imgsz=TRAIN_SIZE, conf=YOLO_CONF, iou=YOLO_IOU, verbose=False

)

boxes = results[0].obb

if boxes is None or len(boxes) == 0:

print(f"⚠️ No OBB detected: {file_name}")

skipped += 1

continue

# --- Sort detections bottom-to-top using mapped original coords ---

sorted_items = []

for i, box in enumerate(boxes):

corners_sq = box.xyxyxyxy.cpu().numpy().reshape(-1, 2).astype(np.float32)

corners_f = unletterbox_corners(corners_sq, scale, pad_left, pad_top)

# Clamp to image bounds

corners_f[:, 0] = np.clip(corners_f[:, 0], 0, img.shape[1] - 1)

corners_f[:, 1] = np.clip(corners_f[:, 1], 0, img.shape[0] - 1)

y_center = corners_f[:, 1].mean()

sorted_items.append((y_center, i, box, corners_f))

sorted_items.sort(key=lambda x: x[0], reverse=True)  # bottom-to-top

# Rank -> label

rank_to_label = {}

for rank, (_yc, _i, _box, _c) in enumerate(sorted_items):

rank_to_label[rank] = (

DISC_LEVELS_BOTTOM_TO_TOP[rank]

if rank < len(DISC_LEVELS_BOTTOM_TO_TOP)

else f"Level_{rank+1}"

)

# --- OBB warp crop -> U-Net -> warp back ---

for rank, (yc, i, box, corners_f) in enumerate(sorted_items):

try:

rect = order_points(corners_f)

(tl, tr, br, bl) = rect

maxW = int(max(np.linalg.norm(br - bl), np.linalg.norm(tr - tl)))

maxH = int(max(np.linalg.norm(tr - br), np.linalg.norm(tl - bl)))

if maxW < 2 or maxH < 2:

continue

dst = np.array([[0, 0],

[maxW - 1, 0],

[maxW - 1, maxH - 1],

[0, maxH - 1]], dtype=np.float32)

M = cv2.getPerspectiveTransform(rect, dst)

roi = cv2.warpPerspective(img, M, (maxW, maxH))

# U-Net inference

resized_roi = cv2.resize(roi, UNET_IMG_SIZE)

input_unet = np.expand_dims(resized_roi.astype(np.float32) / 255.0, axis=0)

pred = unet_model.predict(input_unet, verbose=0)[0]

mask = (pred > 0.5).astype(np.uint8)  # binary mask

# Resize mask back to ROI size

mask_roi = cv2.resize(mask.squeeze(), (maxW, maxH), interpolation=cv2.INTER_NEAREST)

# Simple cleanup: opening + largest CC

kernel = np.ones((3, 3), np.uint8)

mask_clean = cv2.morphologyEx(mask_roi, cv2.MORPH_OPEN, kernel)

nlab, lab, stats, _ = cv2.connectedComponentsWithStats(mask_clean, connectivity=8)

if nlab > 1:

largest = 1 + np.argmax(stats[1:, cv2.CC_STAT_AREA])

mask_clean = (lab == largest).astype(np.uint8)

# Warp mask back to original image space

Minv = np.linalg.inv(M)

mask_back = cv2.warpPerspective(

(mask_clean * 255).astype(np.uint8),

Minv,

(img.shape[1], img.shape[0])

)

full_mask = np.maximum(full_mask, (mask_back > 0).astype(np.uint8))

except Exception as e:

print(f"❌ Warp/seg error: {file_name} | box {i} | {e}")

# --- Measurement + annotation (single line per level: "L1-L2=4.00mm") ---

for rank, (yc, i, box, corners_f) in enumerate(sorted_items):

disc_label = rank_to_label[rank]

try:

corners_int = corners_f.astype(np.int32)

pt1, pt2, pt3, pt4 = corners_int

edge1 = np.linalg.norm(pt1 - pt2)

edge2 = np.linalg.norm(pt2 - pt3)

# Choose an oriented midline based on box geometry

if edge1 >= edge2:

mid_left  = ((pt1[0] + pt4[0]) // 2, (pt1[1] + pt4[1]) // 2)

mid_right = ((pt2[0] + pt3[0]) // 2, (pt2[1] + pt3[1]) // 2)

else:

mid_left  = ((pt1[0] + pt2[0]) // 2, (pt1[1] + pt2[1]) // 2)

mid_right = ((pt3[0] + pt4[0]) // 2, (pt3[1] + pt4[1]) // 2)

# Intersect the oriented line with the full thecal sac mask

line_mask = np.zeros_like(full_mask, dtype=np.uint8)

cv2.line(line_mask, mid_left, mid_right, 255, thickness=2)

valid_line = cv2.bitwise_and(line_mask, (full_mask * 255).astype(np.uint8))

# Skeletonize to get a 1-pixel-wide