SUPPLEMENTARY MATERIALS

S1. MRI scanning parameters and contrast agent

The three 3.0-Tesla MRI scanners and their scanning parameters of different phases were illustrated in Table S1.

Table S1 MRI Scanning parameters

|  | Umr790, United Imaging Healthcare | Discovery MR 750（GE） | Prisa（Siemens） |
| --- | --- | --- | --- |
| T2-weighted fast spin echo (FSE) imaging |  |  |  |
| repetition time (TR): 2000 ms | 2950 | 4000 | 1600 |
| echo time (TE): 107 ms | 88.9 | 93.4 | 95 |
| flip angle (FA): 100° | 135 | 110 | 160 |
| field of view (FOV): 380×400 mm2 | 379.2x300.0 | 380x380 | 380x295.7 |
| matrix: 256 × 256 |  |  |  |
| section thickness: 6 mm | 6 | 8 | 6 |
| axial T1-weighted gradient echo (GRE) imaging for in/out phase liver imaging |  |  |  |
| TR: 4200 ms | in phase 4.1 out phase 4.1 | in phase 4.0 out phase 4.0 | in phase 4.0 out phase 4.0 |
| TE: 1.24 and 2.52 ms | in phase 2.6 out phase 1.4 | in phase 2.6 out phase 1.4 | in phase 2.5 out phase 1.3 |
| FA: 10° | in phase 12.0 out phase 12.0 | in phase 11.0 out phase 11.0 | in phase 9.0 out phase 9.0 |
| FOV: 300 × 400 mm2 | in phase 399.4x279.7 out phase 340.0x340.0 | in phase 388.4x308.7 out phase 388.4x308.7 | in phase 378.8.x308.8 out phase 378.8.x308.8 |
| matrix: 232 × 432 |  |  |  |
| T1-weighted GRE sequence for contrast-enhanced MRI |  |  |  |
| TR: 3200 ms | 3.4 |  | 4 |
| TE: 1.45 ms | 1.4 |  | 1.3 |
| FA: 10° | 10 |  | 9 |
| FOV: 300 × 400 mm2 | 340.0x340 |  | 378.8x308.8 |
| matrix: 307 × 480 |  |  |  |
| Diffusion Weighted Image（DWI） |  |  |  |
| TR: 3800 ms | 3008 | 3333.3 | 4000 |
| TE: 75 ms | 59.2 | 47.5 | 47 |
| FA: 90° | 90 | 90 | 90 |
| FOV: 300 × 380 mm2 | 379.2x299.3 | 400x400 | 378.6x283.6 |
| section thickness: 6 mm | 6 | 7 | 5 |
| matrix: 300 × 380 |  |  |  |
| b values: 0, 800 s/mm2 | b50、800 | b500、800 | b50、800 |

In this study, multiple agents including Gadopentetate dimeglumine (Gd-DTPA) and Gadobutrol (Gd-BT-DO3A) were employed over the 2015-2024 period. The contrast agents we used were all extracellular agents. And these different contrast agents were used with the same scanning method: the agents were injected at 2.0 mL/s with a dose of 0.1 mmol/kg, followed by a 20 mL saline flush. The arterial phase was imaged 20-25 seconds post-contrast injection, the portal venous phase at 55-65 seconds, and the delayed phase at 120-160 seconds.