# Table S1. Protocols of functional magnetic resonance sequences

| Sequence | IVIM | DTI | DKI |
| --- | --- | --- | --- |
| Scan plane | Axial | Axial | Axial |
| TR/TE | 2400/58 | 2500/58 | 2500/58 |
| FOV (cm) | 38–42 | 38–42 | 38–42 |
| Thickness (mm) | 5 | 5 | 5 |
| Spacing (mm) | 2 | 2 | 2 |
| Slices | 20–24 | 20–24 | 20–24 |
| Matrix | 256 × 256 | 256 × 256 | 256 × 256 |
| NEX | 1–6 | 2 | 2 |
| b values | 25, 50, 100, 150, 200, 400, 800, 1000 | 0, 1000 | 0, 500, 1000 |
| Number of directions | 3 | 30 | 30 |

Abbreviation: DKI, diffusion kurtosis imaging; DTI, diffusion tensor imaging; FOV, field of view; IVIM, intravoxel incoherent motion; NEX; number of excitations; TE, echo time; TR, repetition time.