Multiplex PCR

Multiplex PCR was performed using the Qiagen Multiplex PCR kit. The reaction mixture contained 8.5 uL of 2x Master Mix (provides final concentration of 3mM MgCl2), 1.7ul of primer mix (for final concentration of 0.2uM), 100ng of template DNA and RNase-free water to bring volume to 17ul per reaction. Depending on the type of QC performed (primary vs. secondary) two different sets of 8-primer pairs were used, as described in the main manuscript. Thin-walled tubes were used for individual PCR reactions. PCR reaction was performed using the C1000 Thermal Cycler (Bio-Rad). The PCR condition was 95 oC for 15min, denaturation at 94oC for 30 seconds, annealing at 64oC for 30 seconds, and extension at 72oC for 30 seconds for a total of 10 cycles, with a final step of 72 oC for 10 min.

The quality and efficacy of multiplex PCR pre-amplification reaction was confirmed for each WGA sample undergoing the QC process. This was achieved by performing concurrent multiplex PCR reactions using a positive (batch DNA) and a negative (sterile-water) control for each individual WGA sample being tested, and subsequently evaluating their reaction outputs using a qPCR assay. For a multiplex PCR reaction to be deemed effective and reliable, all amplified genes (8-genes covered by the primer sets) should be detectable for the positive control (batch DNA) using qPCR. On the other hand, none of the 8-genes should be detected by the qPCR assay when using a negative control (sterile-water). Only those multiplex PCR reactions passing this requirement could be deemed useful for evaluating a WGA sample quality.