Supplementary Methods S1. Manual quality control (QC) for keyframe selection

Scope. This supplement details the manual, outcome-agnostic QC applied to candidate keyframes used for analysis. The procedure is identical for BMUS and SMI unless otherwise stated.

QC principle. A candidate keyframe is accepted only if all checklist items are satisfied. If any item fails, the frame is replaced by the nearest acceptable frame within the same sub-interval (i.e., within [s, f₀) or (f₀, e] and within its 1/3 or 2/3 segment).

Uniform visual QC (BMUS & SMI).

- Completeness of lesion display – The entire nodule is visible within the imaging window; no edge cut-off of the ROI.

- Margin clarity – Lesion margins are clearly delineable over the majority of the circumference (≈≥75%) without dominant speckle obscuration.

- Motion blur – Subjective 0–3 scale (0 none, 1 mild, 2 moderate, 3 severe); accept ≤1.

- Acoustic artifacts – No dominant acoustic shadow or reverberation obscuring >25% of the lesion interior.

- Exposure/gain – No obvious over- or under-gain; background parenchymal speckle remains visible; no visible clipping/saturation bands.

- On-screen overlays – Frames with calipers, text or graphics crossing the lesion are rejected.

SMI-specific QC.

- Flow stability – Microvascular signals show a consistent pattern across adjacent frames without global “flash” artifacts.

- Blooming/overspill – Color blooming does not obscure lesion boundary; spill outside the lesion is minimal and non-dominant.

- Parameter transitions – Frames during visible gain ramping or probe stabilization are excluded.