Table S1. 1.5 T and 3.0 T MRI acquisition parameters

| Weighting | GE Signa Excite 1.5T |  |  |  | GE Signa HDx 3.0T |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | T2 | Diffusion | Dyn 3D T1 fs |  | T2 | Diffusion | Dyn 3D T1 fs |
| Sequence name | TSE | DWI | Vibrant |  | TSE | DWI | Vibrant |
| n of slices | 28 | 28 | 88 |  | 32 | 32 | 212 |
| Thickness(mm) | 4 | 4 | 1.8 |  | 4 | 4 | 1.0 |
| Gap(mm) | 1 | 1 | 0 |  | 1 | 1 | 0 |
| FOV(mm×mm) | 300×300 | 350×350 | 380×380 |  | 300×300 | 320×320 | 320×320 |
| Matrix (read×phase) | 320×256 | 128×128 | 380×240 |  | 288×224 | 128×96 | 320×320 |
| TR(ms) | 4000 | 4500 | 4.9 |  | 5428 | 3875 | 4.8 |
| TE(ms) | 42 | 73.6 | 2.2 |  | 85 | 72.2 | 2.2 |
| Flip angle(degrees) | 90 | 90 | 15 |  | 90 | 90 | 15 |
| Fat suppression | STIR | STIR | STIR |  | STIR | STIR | STIR |
| b-value (s/mm2) | - | 800 | - |  | - | 800 | - |
| Contrast injection | N/A | N/A | Intravenous injection of 0.1 mmol/kg body weight gadoteridol |  | N/A | N/A | Intravenous injection of 0.1 mmol/kg body weight gadoteridol |
| Acquisition duration (min:s) | 4:51 | 02:29 | 1:10 mins per phase, 10:30 min total (1 pre-contrast phase, 8 post-contrast phases with K0 values acquired every 1 :10 seconds after contrast injection) |  | 04:55 | 02:48 | 1:12 mins per phase, 10:48 min total (1 pre-contrast phase, 8 post-contrast phases with K0 values acquired every 1 :12 seconds after contrast injection) |

n, number; TR, repetition time; TE, echo time; T2, T2-weighted; Dyn 3D T1 fs, dynamic three-dimensional

T1-weighted with fat suppression; TSE, turbo spin echo; STIR, short tau inversion recovery;

DWI, diffusion-weighted imaging.

Table S2. The numbers of false-negative and false-positive results obtained after applying the ADCmin and ADCACRIN cutoff.

|  | Derivation cohort |  |  | Validation cohort |  |
| --- | --- | --- | --- | --- | --- |
|  | ADCmin | ADCmean |  | ADCmin | ADCmean |
| False nagative(total） | 8/167 (4.79%) | 8/167 (4.79%) |  | 4/107 (3.73%) | 6/107 (5.61%) |
| mass | 6 | 6 |  | 1 | 1 |
| NME | 2 | 2 |  | 3 | 5 |
| ＞1cm | 5 | 5 |  | 3 | 5 |
| ≤1cm | 1 | 1 |  | 1 | 1 |
| 1.5T | 4 | 4 |  | 3 | 5 |
| 3.0T | 4 | 4 |  | 1 | 1 |
| BI-RADS 4 | 8 | 8 |  | 4 | 6 |
| BI-RADS 5 | 0 | 0 |  | 0 | 0 |
| False-positive（total） | 76/144 (52.8%) | 79/144 (54.9%) |  | 32/76 (42.1%) | 34/76 (44.7%) |
| mass | 55 | 57 |  | 27 | 28 |
| NME | 21 | 22 |  | 5 | 6 |
| ＞1cm | 44 | 46 |  | 24 | 25 |
| ≤1cm | 32 | 33 |  | 8 | 9 |
| 1.5T | 43 | 47 |  | 25 | 25 |
| 3.0T | 33 | 32 |  | 7 | 9 |
| BI-RADS 4 | 76 | 78 |  | 32 | 34 |
| BI-RADS 5 | 0 | 1 |  | 0 | 0 |