S1 Table. Characteristics of the Precizon toric IOL and Tecnis toric IOL.

| Characteristics | Precizon toric IOL | Tecnis toric IOL |
| --- | --- | --- |
| Optic characteristics |  |  |
| Powers (D) | +10 to +30 D | +5.00 to +34.00 |
| Cylinder powers (IOL plane) (D) | 1 to 10 D (0.5 D increments) | 1.50/2.25/3.00/4.00 |
| Cylinder powers (corneal plane) (D) | 0.69 to 6.85 | 1.03/1.54/3.06/2.74 |
| Diameter (mm) | 6.0 | 6.0 |
| Shape | Biconvex, transitional conic toric aspheric surface | Biconvex, anterior toric aspheric surface |
| Material | Acrylic - hydrophilic | Acrylic - UV filtering hydrophobic |
| Refractive Index | 1.46 | 1.47 |
| Edge design | Square edge | Square edge |
| Spherical aberration (㎛) | 0 | -0.27 |
| Biometry |  |  |
| A-constant (mm) | 118.0 (US); 118.5 (optical) | 118.8 (US); 119.3 (optical) |
| Theoretical ACD (mm) | 5.26 | 5.4 |
| Haptic characteristics |  |  |
| Overall length (mm) | 12.5 | 13.0 |
| Configuration | close-loop design | modified C-loop design, integral with optic |
| Material | Same as optic | Same as optic |

IOL: intralocular lens; ACD: anterior chamber depth; US: ultrasound.