The method of tissue-based assay

- Kit preparation: Take out the required brain pieces according to the number of samples and place them in a dry environment at room temperature (18~25℃) for 10 min.

- Sample preparation: When testing serum specimens, it is necessary to dilute at least 10 times. Cerebrospinal fluid (CSF) can be used directly for testing without dilution.

- Sample incubation: Add the prepared sample (brain slice 100μL) to the reaction zone and incubate at 37℃ for 1h.

- Washing: Remove the sample solution, immediately add 0.01M Phosphate Buffered Saline (PBS) buffer (brain slice 200μL) to the reaction area, and let it stand at room temperature for 5 min. Remove lotion and repeat 2 times.

- Secondary antibody incubation: Prepare 1× fluorescent secondary antibody working solution. The washing solution of the slide was removed, the slide was immediately placed in a wet box and the second antibody working solution (brain slice 100μL) was added to the reaction area, and incubated in a constant temperature incubator at 37℃ for 30min.

- Washing: same as step 4.

- Observation results: After washing, the film is not sealed, and after wiping with a paper towel, the inverted fluorescence microscope is used immediately to observe the results and collect images. Fluorescence wavelength parameters: Green fluorescence: Ex 495nm, Em 519nm.

- Interpretation of results.

Positive: there were obvious specific fluorescent staining patterns;

Negative: no obvious fluorescent staining.

The method of cell-based assay

- Add sample: Place the sample on the foam board, add 30µL in order to dilute the sample to each reaction zone of the sample to avoid bubbles. Start warming after adding all samples to be tested.

- Incubation: Place the slide, biosheeting side down, in the template groove, and the reaction begins immediately. Ensure that each sample is in contact with the biological sheet and that the samples are not in contact with each other. Incubate at room temperature (18~25°C) for 30 minutes.

- Rinse: Rinse the slide in a beaker with PBS Twain buffer under running water, then immediately immerse it in a wash cup with PBS Twain buffer for at least 5 minutes.

- Adding sample: 25µL FITC-labeled anti-human globulin (fluorescent secondary antibody) was added to clean the reaction zone of the sample, and all fluorescent secondary antibodies were added to proceed to the next incubation.

- Warm up: Remove a slide from the wash cup, wipe the moisture on the back and edge with absorbent paper within 5 seconds, and immediately cover the groove with the template. Incubate at room temperature (18~25°C) for 30 minutes, avoiding direct sunlight.

- Rinse: Rinse the slide in a beaker with PBS Twain buffer under running water, then immediately dip it into a wash cup with PBS Twain buffer for at least 5 minutes.

- Seal: Place the cover glass directly in the groove of the foam board. Add glycerin /PBS drops to cover slide: up to 10 µL per reaction zone. Remove a slide from the PBS Twain buffer and dry the back and edges with absorbent paper, taking care not to wipe the reaction zone gap. Place the biofilm side down on the prepared cover slide, immediately view and gently adjust the cover slide to fit into the groove of the slide.

- Results: Fluorescence model was observed under microscope.

The method of oligoclonal bands electrophoresis detection

1.  Preparation: The CSF samples were used directly, and the serum samples were diluted with 0.85% saline, the ratio of which was 1:300.

2.  Experimental Procedure

3.  Adding sequence: Add samples in the order of control, cerebrospinal fluid and serum from the red dot end (from bottom to top).

4.  Gel treatment

Place 2 buffer strips in 50ml plastic EP tube, then wet the two strips with 500ul/2ml anodic solution and 500ul/2ml cathodic solution respectively as a salt bridge. Before applying to the film, Wick Blotters hold the Electrode strips to Electrode Wicks respectively and press them from one end to the other with their fingers to absorb any excess water on the strips.

5.  Electrophoresis

Take 0.5ml Rep prep solution into the electrophoresis tank and wipe off excess liquid at the edges of the gel. Place the gel sheet in the center of the electrophoresis tank with the sample hole facing the anode on the right; Place the anode soaked electrode core between two Wick bloters to remove excess liquid. The electrode core is almost dry after moisture absorption. The anode core is placed on the anode tail of the gel surface, the electrode core is placed in the cathode solution, and the cathode core is placed on the cathode tail of the gel surface; Place the electrodes on top of the two electrode cores, ensuring that the magnetic extension is in contact with the inside of the magnet. In order to mount the electrode directly on the electrode core, the position of the gel should be adjusted appropriately. The absorbent paper is placed on the Yin and Yang extremes of the gel to absorb more liquid and ensure full contact between the absorbent paper and the gel.

Add 5ul of samples to each addition hole, select the CSF IFE display screen, press the Start button to start the electrophoresis operation.

6.  Western Blot

Following the above steps, remove the gel from the electrophoresis tank, place the gel on Blotter D (filter paper), sample 1 on the left, discard the electrode core.

In advance, soak a transfer film in deionized water, gently remove excess water on the film with a paper towel, and gently place the film on the adhesive surface, with the red dot below. To ensure that the film placed on the surface of the gel has a certain humidity and should prevent bubbles between the film and the glue. Leave for 10 seconds and then discard the film.

Wet the second transfer film with 10% methanol. Place the film vertically for 20 seconds to remove excess liquid, carefully place the film on the surface of the gel, the red dot should be in the lower left corner (this marked point is the position of sample 1), gently slide your finger on the surface of the film to remove the bubbles between the gel and the film. Press 1 sheet of Blotter C (filter paper), 3 sheets of Blotter D (filter paper) and a weight on the gel for 30 minutes. Remove Blotters and weights, place the transfer membrane with the red dot on top into A bottle containing 20ml sealer A and gently shake it for 30 minutes.

Wash the film with water 3 times.

Pour 15ml of sealer B into a plate and mix with 15ul of enzyme-labeled antibody.

Place the film in this solution for 30 minutes and gently shake.

Wash the membrane with deionized water 3 times, and wash with salt water for 5 minutes, and finally deionized water.

7.  Color development

Pour 25ml of diluted acetic acid buffer into a plate, add 5ml of dye solution and 25ul 30% hydrogen peroxide, and mix immediately.

Put the film into the mixture and gently shake the plate. The dyeing time is about 20 minutes. During this process, the dye will precipitate, which is a normal phenomenon.

Wash the film 3 times with water, then dry the film.

The transfer membrane was quantitatively detected.

8.  Result saving