# Supplemental information

Supplemental Table 1. MRSinMRS checklist.

| - Hardware |  |
| --- | --- |
| - Field strength (T) | 3 |
| - Manufacturer | Siemens |
| - Model | Skyra, software version XA30 |
| - RF coils | 1H, 64 channels, head coil |
| - Additional hardware | N/A |
| - Acquisition |  |
| - Pulse sequence | HERCULES |
| - VOI location(s) | mPFC |
| - Nominal VOI size (cm3) | 3×3×3 |
| - TR, TE (ms) | 2000, 80 |
| - Total number of acquisitions per spectrum | 320 total averages (80 averages per sub-spectra) |
| - Additional sequence parameters | 2000Hz width, 2048 points |
| - Water suppression method | WET |
| - Shimming method | Automated B0-field mapping followed by manual shimming of water to <22 Hz |
| - Triggering or motion correction method | No triggering or motion correction |
| - Data analysis methods and outputs |  |
| - Analysis software | Osprey |
| - Processing steps deviating from software | N/A |
| - Output measure | Tissue-corrected, water-scaled |
| - Quantification references and assumptions, fitting model assumptions | Default Osprey basis set for Siemens HERCULES sequences |
| - Data quality |  |
| - Reported variables | SNR and linewidth of NAA |
| - Data exclusion criteria | CRLB < 20% |
| - Quality measures of postprocessing model fitting | CRLB |

VOI: volume of interest; TR: repetition time; TE: echo time; HERCULES:  Hadamard Editing Resolves Chemicals Using Linear- combination Estimation of Spectra; mPFC: medial prefrontal cortex; WET: water suppression enhanced through T1 effects; SNR: signal-to-noise ratio; NAA: N-Acetylaspartic acid; CRLB: Cramér-Rao lower bounds

Supplemental Table 2. CRED-nf best practices checklist.

| Domain | Item | Checklist item | Section |
| --- | --- | --- | --- |
| Pre-experiment |  |  |  |
|  | 1a | Pre-register experimental protocol and planned analyses | Trial registration |
|  | 1b | Justify sample size | Aim 1a: Target engagement |
| Control-groups |  |  |  |
|  | 2a | Employ control group(s) or control condition(s) | Randomization |
|  | 2b | When leveraging experimental designs where a double-blind is possible, use a double-blind | Randomization |
|  | 2c | Blind those who rate the outcomes, and when possible, the statisticians involved | Randomization |
|  | 2d | Examine to what extent participants and experimenters remain blinded | Randomization |
|  | 2e | In clinical efficacy studies, employ a standard-of-care intervention group as a benchmark for improvement | N/A |
| Control measures |  |  |  |
|  | 3a | Collect data on psychosocial factors | Assessments |
|  | 3b | Report whether participants were provided with a strategy | Mindfulness training |
|  | 3c | Report the strategies participants used | Neurofeedback paradigm |
|  | 3d | Report methods used for online-data processing and artifact correction | Neurofeedback implementation |
|  | 3e | Report condition and group effects for artifacts | N/A |
| Feedback specifications |  |  |  |
|  | 4a | Report how the online-feature extraction was defined | Neurofeedback implementation |
|  | 4b | Report and justify the reinforcement schedule | Neurofeedback display and interaction |
|  | 4c | Report the feedback modality and content | Neurofeedback display and interaction |
|  | 4d | Collect and report all brain activity variable(s) and/or contrasts used for feedback, as displayed to experimental participants | Neurofeedback implementation |
|  | 4e | Report the hardware and software used | Neurofeedback implementation |
| Outcome measures |  |  |  |
| Brain | 5a | Report neurofeedback regulation success based on the feedback signal | N/A |
|  | 5b | Plot within-session and between-session regulation blocks of feedback variable(s), as well as pre-to-post resting baselines or contrasts | N/A |
|  | 5c | Statistically compare the experimental condition/group to the control condition(s)/group(s) (not only each group to baseline measures) | N/A |
| Behavior | 6a | Include measures of clinical or behavioural significance, defined a priori, and describe whether they were reached | N/A |
|  | 6b | Run correlational analyses between regulation success and behavioural outcomes | N/A |
| Data storage |  |  |  |
|  | 7a | Upload all materials, analysis scripts, code, and raw data used for analyses, as well as final values, to an open access data repository, when feasible | GITHUB |