Supplemental Figure 1. Proposed FCT protocol.

For Coronary calcium score scan

Position:	          Supine

Scanning mode:	Axial with prospective ECG gating

Scan range:             From below the aortic arch to base of heart

Scan parameters:   120 kV fixed, mAs BMI dependent.

BMI < 30 = 120 kVp @ 40 mAs

BMI 31-34 = 120 kVp @ 50 mAs

BMI > 120Kvp @ 60 mAs

Slice thickness        2.5 mm with interval of 1 mm

Reconstructions/ post processing:

Axial soft tissue widow – Slice thickness – 2.5 mm

Axial lung window- Slice thickness – 2.5 mm

Calcium scoring performed using Philips Intellispace Portal   software, Version 10.1,as per Agatson method

Approximate study time: 10 – 15 minutes

For FCT

Position:	          Supine

Scanning mode:	Axial with prospective ECG gating

Scan range:            From base of neck through entire lung fields to base of heart

Scan parameters:   120 kV fixed, mAs BMI dependent.

BMI < 30 = 120 kVp @ 40 mAs

BMI 31-34 = 120 kVp @ 50 mAs

BMI > 120Kvp @ 60 mAs

Slice thickness        2.5 mm with interval of 1 mm

Reconstructions/ post processing:

Axial soft tissue widow – Slice thickness – 2.5 mm

Axial lung window- Slice thickness – 2.5 mm

Calcium scoring performed using Philips Intellispace Portal   software, Version 10.1,as per Agatson method

Sagittal and coronal reconstruction

Approximate study time: 10 – 15 minutes