Intensive Care Unit SBT Assessment Form

| Bed No./MRN： | Name:： | Sex: | Age: |
| --- | --- | --- | --- |
| Diagnosis: |  | Ventilation Start Date: |  |
| Pre-trial Assessment (SBT Initiation Criteria) |  |  |  |
| - □ Improvement in the cause of respiratory failure or intubation □ Sedatives discontinued/not used |  |  |  |
| 2. Circulation □ No acute myocardial ischemia or severe arrhythmia □ Heart rate <140 bpm □ Dopamine ≤5μg/kg/min □ Dobutamine ≤5μg/kg/min □ Norepinephrine ≤0.05μg/kg/min |  |  |  |
| - Ventilation □ SpO2 >90% and FIO2≤0.5 and PEEP≤8 cmH2O □ Tidal volume >5 ml/IBW □ Minute ventilation <15 L/min □ Rapid shallow breathing index < 105/min/L □ No respiratory acidosis (pH≥7.25) □ Normal respiratory pattern (rate <30/min, no thoracoabdominal paradox, no significant suprasternal or intercostal retractions during inspiration) |  |  |  |
| 4. General Condition □ Core temperature ≤38 °C □ No severe electrolyte disturbances (Na 135-145 mmol/L, K 3.5-5.0 mmol/L, Ca 2.1-2.6 mmol/L, Mg 0.7-1.1 mmol/L) □ No severe anemia (Hemoglobin > 7 g/dL, or > 8-10 g/dL depending on specific conditions) □ No severe fluid overload (24-hour fluid balance not exceeding +1000-1500 mL, no significant peripheral edema) |  |  |  |

GCS Score: ____   Cough Strength Score: ____

SBT Method: Pressure Support Ventilation (PSV)

* Department standard does not include T-piece, Y-piece trials for weaning are not recommended

Pressure Support Level: ____ cmH2O (5-8 cmH2O) | PEEP: ____ cmH2O (< 5 cmH2O) | FiO2: ____ % (same as pre-SBT)

SBT Implementation Assessment

| Parameter | Baseline | 30 min | 60 min | 90 min | 120 min |
| --- | --- | --- | --- | --- | --- |
| HR |  |  |  |  |  |
| BP |  |  |  |  |  |
| SpO₂ |  |  |  |  |  |
| RR |  |  |  |  |  |
| VT |  |  |  |  |  |
| pH |  |  |  |  |  |
| PaO₂ |  |  |  |  |  |
| PaCO₂ |  |  |  |  |  |

Subjective Assessment

| Parameter | Baseline | 30 min | 60 min | 90 min | 120 min |
| --- | --- | --- | --- | --- | --- |
| Mental status deterioration |  |  |  |  |  |
| Diaphoresis |  |  |  |  |  |
| Subjective discomfort |  |  |  |  |  |
| Use of accessory respiratory muscles |  |  |  |  |  |
| Thoracoabdominal paradox |  |  |  |  |  |

SBT Failure Criteria:

Respiratory rate > 35/min or significant increase in accessory respiratory muscle activity

Persistent SpO2 < 92% (< 88% in chronic lung disease patients), with FiO2 ≥ 0.4

Hemodynamic instability (HR > 140/min or SBP > 180 mmHg, with signs of hypoperfusion)

Significant decrease in mental status or severe agitation

SBT Successful? □ Yes □ No   Reason for failure (if applicable): ________________________

Cuff leak test: □ Pass □ Fail   Corticosteroids used: □ Yes □ No   Extubation performed? □ Yes □ No

Post-extubation therapy:

□ Conventional Oxygen Therapy (COT) □ Non-Invasive Ventilation (NIV) □ High-Flow Nasal Cannula (HFNC)

Reintubation within 48h post-extubation? □ Yes □ No

Signature: _____________ Date: _____________