# Supplements

Supplement Figure 1. Break-even analysis of costs per flexible cystoscopy including extended capital investment.

The figure displays the calculated per-procedure costs for reusable cystoscopes (solid curved line) across varying annual case volumes, compared with the constant per-procedure costs of single-use devices (dashed line). Incorporating the extended capital investment scenario described in Table 2, the break-even point—at which reusable cystoscopes become more cost-effective than single-use alternatives—is reached at approximately 265 cystoscopies per year.

Supplement Figure 2. Radar plot of mean physician ratings for reusable cystoscopes (blue) and single-use cystoscopes (orange).

Displayed are the mean scores for six evaluation criteria: image quality, manoeuvrability, ergonomics, diagnostic confidence, logistics, and overall impression.

Supplement Figure 3. Heatmap of physician ratings for reusable cystoscopes (left) and singleuse cystoscopes (right).

Shown are the relative proportions of Likert scale ratings (1 = very poor to 5 = very good) for image quality, manoeuvrability, ergonomics, diagnostic confidence, logistics, and overall impression. Darker shades indicate higher proportions.

Supplement Figure 4. Radar plot of mean nursing staff ratings for reusable cystoscopes (blue) and single-use cystoscopes (orange).

Displayed are the evaluation criteria overall impression, handling, reliability, logistics, and reprocessing effort.

Supplement Figure 5. Heatmap of nursing staff ratings for reusable cystoscopes (left) and single-use cystoscopes (right).

Shown are the relative proportions of Likert scale ratings (1 = very poor to 5 = very good) for overall impression, handling, reliability, logistics, and reprocessing effort. Darker shades indicate higher response proportions.

Supplement Figure 6. Percentage distribution of physician ratings for reusable cystoscopes (left) and single-use cystoscopes (right).

The figure illustrates the percentage distribution of Likert scale scores—1 = very poor (red), 2 = yellow, 3 = turquoise, 4 = blue, and 5 = violet—for image quality, manoeuvrability, ergonomics, diagnostic confidence, logistics, and overall impression. Each horizontal bar represents the relative proportion of ratings within the respective device group.

Supplement Figure 7. Likert plot of nursing staff ratings for reusable cystoscopes (left) and single-use cystoscopes (right).

The figure presents the percentage distribution of Likert scale scores—1 = very poor (red), 2 = yellow, 3 = turquoise, 4 = blue, and 5 = violet—for overall impression, handling, reliability, logistics, and reprocessing effort. Each horizontal bar depicts the relative proportion of ratings within the respective device group.

Supplement Figure 8. Tornado plot of the deterministic one-way sensitivity analysis. The x-axis indicates the annual cystoscopy volume at which reusable and single-use cystoscopy incur equal costs, and the y-axis lists the individually varied model parameters. The analysis used a break-even model separating fixed reusable-system costs from variable per-procedure costs and comparing these with the constant unit cost of single-use cystoscopy. Reusable equipment costs, annual repair costs, reprocessing material costs, reprocessing time, and gross hourly wages were varied from 50% to 200% of their base-case values, while the single-use device price was varied by ±25%. All remaining inputs were held at their base-case values. Horizontal bars represent the resulting range in break-even volume, and the dashed vertical line denotes the base-case threshold of approximately 67 procedures per year.

# Supplement Tables

| Capital investment Unit cost (€) | Amortization Annual cost (€) (years) |  |  |  | Cost per cystoscopy (€) |
| --- | --- | --- | --- | --- | --- |
| Olympus CYF-5 cystoscope 3.665,00 € | 5 |  |  | 733,00 € | 2,18 € |
| Olympus AR-TF08E 500,00 € | 5 |  |  | 100,00 € | 0,30 € |
| Olympus OTV-SC digital signal processor 500,00 € | 5 |  |  | 100,00 € | 0,30 € |
| Karl Storz light cable |  | 150,00 € | 5 | 30,00 € | 0,09 € |
| Karl Storz halogen light source 250 twin |  | 600,00 € | 5 | 120,00 € | 0,36 € |
| LG monitor 15LC1R |  | 100,00 € | 5 | 20,00 € | 0,06 € |
| Total capital costs |  |  |  |  | 3,27 € |
| Maintenance Repair |  | 1.804,64 € |  | 1.804,64 € | 5,36 € |
| Leak tester (Karl Storz) |  | 150,00 € | 5 | 30,00 € | 0,09 € |
| Reprocessing SICOLAB mini Endo, Dürr Technik |  | 3.200,00 € | 5 | 640,00 € | 1,90 € |
| Sekusept aktiv, 6 kg powder |  | 202,75 € |  | 2.162,00 € | 6,42 € |
| Reprocessing basins (3 units) |  | 240,00 € | 5 | 48,00 € | 0,14 € |
| Distilled water, 21 L |  | 5,25 € |  | 1.050,00 € | 3,12 € |
| Syringe, 10 ml (BRAUN) |  | 0,06 € |  |  | 0,06 € |
| Cleaning brush |  | 0,89 € |  |  | 0,89 € |
| Gauze compress, 10 × 10 cm |  | 0,05 € |  |  | 0,05 € |
| Gloves, 100 units |  | 0,06 € |  |  | 0,06 € |
| Cap |  | 0,09 € |  |  | 0,09 € |
| Spray (Aesculap Sterilit Oil, 300 ml) |  | 15,68 € |  | 31,36 | 0,14 € |
| Total reprocessing costs |  |  |  |  | 12,86 € |
| Staff costs (20min) |  | 5,67 € |  |  | 5,67 € |
| Total cost per cystoscopy |  |  |  |  | 27,25 € |

Supplement Table 1. Overview of cost components per reusable flexible cystoscopy.
Capital costs were amortised linearly over five years based on an annual case volume of 337 procedures. Reprocessing accounted for the largest share of total costs (€12.86 per procedure), followed by staff costs (€5.67), maintenance (€5.45), and capital investment (€3.27). The overall mean cost per reusable flexible cystoscopy was €27.25.

| Capital investment | Unit cost (€) | Amortization Annual cost (€) (years) |  | Cost per cystoscopy (€) |
| --- | --- | --- | --- | --- |
| Olympus CYF-5 cystoscope | 7.600,00 € | 5 | 1.520,00 € | 4,51 € |
| HD pendulum camera head | 14.500,00 € | 5 | 2.900,00 € | 8,61 € |
| Video system incl. light source OTV-S500 | 17.500,00 € | 5 | 3.500,00 € | 10,39 € |
| Olympus light cable | 450,00 € | 5 | 90,00 € | 0,27 € |
| Olympus HD monitor 21" | 3.000,00 € | 5 | 600,00 € | 1,78 € |
| Total capital costs | 43.050,00 € |  |  | 25,55 € |
| Maintenance Repair | 1.804,64 € |  | 1.804,64 € | 5,36 € |
| Leak tester (Karl Storz) | 150,00 € | 5 | 30,00 € | 0,09 € |
| Reprocessing SICOLAB mini Endo, Dürr Technik | 3.200,00 € | 5 | 640,00 € | 1,90 € |
| Sekusept aktiv, 6 kg powder | 202,75 € |  | 2.162,00 € | 6,42 € |
| Reprocessing basins (3 units) | 240,00 € | 5 | 48,00 € | 0,14 € |
| Distilled water, 21 L | 5,25 € |  | 1.050,00 € | 3,12 € |
| Syringe, 10 ml (BRAUN) | 0,06 € |  |  | 0,06 € |
| Cleaning brush | 0,89 € |  |  | 0,89 € |
| Gauze compress, 10 × 10 cm | 0,05 € |  |  | 0,05 € |
| Gloves, 100 units | 0,06 € |  |  | 0,06 € |
| Cap | 0,09 € |  |  | 0,09 € |
| Spray (Aesculap Sterilit Oil, 300 ml) | 15,68 € |  | 31,36 | 0,14 € |
| Total reprocessing costs |  |  |  | 12,86 € |
| Staff costs (20min) | 5,67 € |  |  | 5,67 € |
| Total cost per cystoscopy |  |  |  | 49,53 € |

Supplement Table 2. Overview of cost components per reusable flexible cystoscopy under extended capital investment.	
Capital costs were amortised linearly over five years based on an annual case volume of 337 procedures. In this extended investment scenario, capital expenditure accounted for €25.55 per procedure, substantially higher than in the baseline model, while reprocessing (€12.86), staff (€5.67), and maintenance (€5.45) remained similar. The overall mean cost per reusable cystoscopy was €49.53.

| Physicians Item | Reusable | Single-Use |
| --- | --- | --- |
| Image quality | 4.0 (4.0-5.0) [2-5] 0/1/0/8/4 | 5.0 (4.0-5.0) [1-5] 2/0/1/3/7 |
| Deflection | 5.0 (4.0-5.0) [1-5] 1/0/0/3/9 | 4.0 (3.0-4.0) [1-5] 2/0/4/5/2 |
| Ergonomics | 4.0 (4.0-5.0) [2-5] 0/1/0/6/6 | 3.0 (3.0-4.0) [1-5] 1/1/5/4/2 |
| Diagnostic safety | 4.0 (4.0-5.0) [2-5] 0/1/0/6/6 | 4.0 (3.0-5.0) [1-5] 1/1/2/5/4 |
| Logistics | 3.0 (3.0-3.0) [3-3] 0/0/13/0/0 | 5.0 (3.0-5.0) {3-5] 0/0/2/4/7 |
| Overall impression | 4.0 (4.0-4.0) [4-4] 0/0/0/13/0 | 4.5 (3.5-4.0) {3-5] 0/0/1/5/7 |

Supplement Table 3. Descriptive results of the physician survey (n = 13 per arm) comparing reusable and single-use cystoscopes

Data are presented as median (Q1–Q3), range [min–max], and absolute frequencies of Likert scale ratings (n1/n2/n3/n4/n5).

| Nursing staff Item | Reusable | Single-Use |
| --- | --- | --- |
| Handling | 4.0 (3.0-4.0) [3-4] 0/0/9/16/0 | 5.0 (5.0-5.0) [4-5] 0/0/0/2/23 |
| Reliability | 4.0 (3.0-4.0) [3-4] 0/0/6/19/0 | 5.0 (5.0-5.0) [5-5] 0/0/0/0/25 |
| Logistics | 3.0 (3.0-4.0) [3-4] 0/0/13/12/0 | 5.0 (5.0-5.0) [5-5] 0/0/0/0/25 |
| Reprocessing effort | 3.0 (3.0-4.0) [1-5] 1/5/11/7/1 | 5.0 (5.0-5.0) [4-5] 0/0/0/1/24 |
| Overall impression | 4.0 (3.0-4.0) [3-5] 0/0/10/13/2 | 5.0 (5.0-5.0) [3-5] 0/0/1/2/22 |

Supplement Table 4. Descriptive results of the nursing staff survey (n = 25 per arm) comparing reusable and single-use cystoscopes

Data are presented as median (Q1–Q3), range min–max, and absolute frequencies of Likert scale ratings (n1/n2/n3/n4/n5).

| Author | Country | Study design | RU System | SU system | Reprocess-ing | Cost RU | Cost SU | Advantage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Franzoso 2025 (11) | Italy | Retrospective (model-based) | Reusable modelled | manufacturer not specified | Modelled | €181-199€ | €189 | Context-dependent |
| Foo 2025 (12) | Singapore | Retrospective | Olympus CYF-5 | manufacturer not specified | HLD (AER) | ≤ SGD 135 | typically > SGD 200 | Reusable |
| Kim 2022 (9) | Canada | Retrospective | Olympus CYF-VH | Ambu aScope | LTS | CAD 149 | CAD 246 | Reusable |
| Ait Taleb 2025 (14) | France | Retrospective | KARL STORZ ref. 11272 | Ambu a 4 C | LTS | €136 | €192 | Reusable |
| Bertolo 2024 (15) | Italy | Retrospective | KARL STORZ, Olympus (model not specified) | Ambu aScope | HLD (AER) | €332 | €220 | Single-use |
| Kenigsberg 2021 (10) | USA | Retrospective | manufacturer not specified | theoretical cost model | HLD | USD 133-161 | USD 222 | Reusable |
| Assmus 2022 (16) | USA | Prospective | manufacturer not specified | Ambu aScope 4 C | HLD | USD 272 | USD 185 | Single-use |
| Young 2021 (6) | USA | Retrospective | Olympus CYF-VHR, CYF-V2 | Ambu aScope 4 C | HLD (AER) | USD 66–233 | USD 227–460 | Reusable |
| Boucheron 2022 (13) | France | Retrospective | manufacturer not specified | Ambu aS4C | Manual HLD | €196 | €192 | Single-use |

Supplement Table 5. Economic comparison of reusable (RU) and single-use (SU) flexible cystoscopes

Abbreviations: RU = reusable; SU = single-use; HLD = high-level disinfection; AER = automated endoscope reprocessor; LTS = low-temperature sterilisation.